For Cisco Packet Tracer version 8.1, configuring a unicast routing protocol

### Updated Steps for Cisco Packet Tracer 8.1

### Step 1: Create the Topology

- Drag and drop devices: Add the necessary devices (e.g., routers, switches, and end devices like PCs) from the device list.

  - Select routers (e.g., 3 routers).

  - Add PCs connected to each router's network.

- Connect the devices:

  - Use Straight-through cables to connect PCs to routers' FastEthernet interfaces.

  - Use Cross-over cables or Serial cables to connect routers together using their respective Serial or FastEthernet interfaces.

### Step 2: IP Address Configuration

- Configure the routers with appropriate IP addresses on the interfaces connecting to other routers and to the PCs.

#### Code for Router 1

Router1> enable

Router1# configure terminal

Router1(config)# interface fastethernet 0/0

Router1(config-if)# ip address 192.168.1.1 255.255.255.0

Router1(config-if)# no shutdown

Router1(config-if)# exit

Router1(config)# interface serial 0/0/0

Router1(config-if)# ip address 10.0.0.1 255.255.255.252

Router1(config-if)# clock rate 64000 (for DCE connection if applicable)

Router1(config-if)# no shutdown

Router1(config-if)# exit

Router1(config)# exit

#### Code for Router 2

Router2> enable

Router2# configure terminal

Router2(config)# interface fastethernet 0/0

Router2(config-if)# ip address 192.168.2.1 255.255.255.0

Router2(config-if)# no shutdown

Router2(config-if)# exit

Router2(config)# interface serial 0/0/0

Router2(config-if)# ip address 10.0.0.2 255.255.255.252

Router2(config-if)# no shutdown

Router2(config-if)# exit

Router2(config)# exit

#### Code for Router 3 (if applicable)

Router3> enable

Router3# configure terminal

Router3(config)# interface fastethernet 0/0

Router3(config-if)# ip address 192.168.3.1 255.255.255.0

Router3(config-if)# no shutdown

Router3(config-if)# exit

Router3(config)# interface serial 0/0/0

Router3(config-if)# ip address 10.0.0.6 255.255.255.252

Router3(config-if)# no shutdown

Router3(config-if)# exit

Router3(config)# exit

### Step 3: Configure RIP on All Routers

- Enable RIP routing protocol on all routers. Below are the RIP configurations for each router. RIP Version 2 is used here.

#### Code for Router 1

Router1> enable

Router1# configure terminal

Router1(config)# router rip

Router1(config-router)# version 2

Router1(config-router)# network 192.168.1.0

Router1(config-router)# network 10.0.0.0

Router1(config-router)# no auto-summary

Router1(config-router)# exit

Router1(config)# exit

#### Code for Router 2

Router2> enable

Router2# configure terminal

Router2(config)# router rip

Router2(config-router)# version 2

Router2(config-router)# network 192.168.2.0

Router2(config-router)# network 10.0.0.0

Router2(config-router)# no auto-summary

Router2(config-router)# exit

Router2(config)# exit

#### Code for Router 3 (if applicable)

Router3> enable

Router3# configure terminal

Router3(config)# router rip

Router3(config-router)# version 2

Router3(config-router)# network 192.168.3.0

Router3(config-router)# network 10.0.0.0

Router3(config-router)# no auto-summary

Router3(config-router)# exit

Router3(config)# exit

### Step 4: Verify RIP Routing Protocol

To verify that RIP is functioning properly and that the routes have been propagated to other routers, use the following command:

#### Command to Verify Routes on Each Router

Router1# show ip route

Expected Output:

Gateway of last resort is not set

192.168.2.0 [120/1] via 10.0.0.2, 00:00:10, Serial0/0/0

192.168.3.0 [120/1] via 10.0.0.2, 00:00:10, Serial0/0/0

C    192.168.1.0 is directly connected, FastEthernet0/0

C    10.0.0.0/30 is directly connected, Serial0/0/0

### Step 5: Test Connectivity

You can now ping between devices to check if the routing protocol is working as expected.

#### Command to Ping from Router 1 to PC in Router 2's network:

Router1# ping 192.168.2.2

Expected Output:

Pinging 192.168.2.2 with 32 bytes of data:

Reply from 192.168.2.2: bytes=32 time=20ms TTL=255

Reply from 192.168.2.2: bytes=32 time=15ms TTL=255

Reply from 192.168.2.2: bytes=32 time=12ms TTL=255

### Step 6: Save Configuration

Save your router configuration so that it persists after reboot:

#### Command to Save Configuration:

Router1# copy running-config startup-config

Router2# copy running-config startup-config

Router3# copy running-config startup-config

Expected Output:

Destination filename [startup-config]?

Building configuration...

[OK]

### Step 7: Enable Simulation Mode (Optional)

Switch to Simulation Mode in Packet Tracer to visualize the routing process and the packet exchanges. You will see the RIP update packets as well as ICMP (ping) requests.

### Conclusion

By following these updated steps and codes, you can successfully simulate the unicast routing protocol (RIP) in Cisco Packet Tracer version 8.1. If you wish to configure different routing protocols such as OSPF or EIGRP, the process is similar, but the commands will differ slightly.