### Steps to Simulate RIP in Cisco Packet Tracer 8.1

#### Step 1: Set Up Network Topology

- Open Cisco Packet Tracer version 8.1.

- Create a simple network topology with three routers connected in a line or triangle configuration. Each router will have one PC connected to it to simulate different subnets.

  - Drag and drop 3 routers and 3 PCs from the device list onto the workspace.

- Connect the routers using serial cables or cross-over cables if using FastEthernet interfaces. Connect the routers to the PCs using straight-through cables.

#### Step 2: Assign IP Addresses to Interfaces

- Assign IP addresses to the router interfaces and PCs to create distinct subnets for each router and the connected PCs.

Here’s an example network layout:

- Router 1 (R1): Connects to Router 2 (R2) via Serial interface.

- Router 2 (R2): Connects to both Router 1 (R1) and Router 3 (R3).

- Router 3 (R3): Connects to Router 2 (R2).

IP Addresses:

- Router 1: FastEthernet 192.168.1.1/24 and Serial 10.0.0.1/30.

- Router 2: Serial 10.0.0.2/30 and 10.0.0.5/30, FastEthernet 192.168.2.1/24.

- Router 3: Serial 10.0.0.6/30 and FastEthernet 192.168.3.1/24.

PC IPs:

- PC 1: 192.168.1.2/24 (Gateway: 192.168.1.1).

- PC 2: 192.168.2.2/24 (Gateway: 192.168.2.1).

- PC 3: 192.168.3.2/24 (Gateway: 192.168.3.1).

#### Code to Configure IP Addresses on Routers

- Router 1 configuration:

Router1> enable

Router1# configure terminal

Router1(config)# interface fastethernet 0/0

Router1(config-if)# ip address 192.168.1.1 255.255.255.0

Router1(config-if)# no shutdown

Router1(config-if)# exit

Router1(config)# interface serial 0/0/0

Router1(config-if)# ip address 10.0.0.1 255.255.255.252

Router1(config-if)# clock rate 64000

Router1(config-if)# no shutdown

Router1(config-if)# exit

- Router 2 configuration:

Router2> enable

Router2# configure terminal

Router2(config)# interface fastethernet 0/0

Router2(config-if)# ip address 192.168.2.1 255.255.255.0

Router2(config-if)# no shutdown

Router2(config-if)# exit

Router2(config)# interface serial 0/0/0

Router2(config-if)# ip address 10.0.0.2 255.255.255.252

Router2(config-if)# no shutdown

Router2(config-if)# exit

Router2(config)# interface serial 0/0/1

Router2(config-if)# ip address 10.0.0.5 255.255.255.252

Router2(config-if)# no shutdown

Router2(config-if)# exit

- Router 3 configuration:

Router3> enable

Router3# configure terminal

Router3(config)# interface fastethernet 0/0

Router3(config-if)# ip address 192.168.3.1 255.255.255.0

Router3(config-if)# no shutdown

Router3(config-if)# exit

Router3(config)# interface serial 0/0/0

Router3(config-if)# ip address 10.0.0.6 255.255.255.252

Router3(config-if)# no shutdown

Router3(config-if)# exit

#### Step 3: Configure RIP on All Routers

To enable RIP v2 on all routers, follow these steps:

- Router 1 configuration:

Router1> enable

Router1# configure terminal

Router1(config)# router rip

Router1(config-router)# version 2

Router1(config-router)# network 192.168.1.0

Router1(config-router)# network 10.0.0.0

Router1(config-router)# no auto-summary

Router1(config-router)# exit

Router1(config)# exit

- Router 2 configuration:

Router2> enable

Router2# configure terminal

Router2(config)# router rip

Router2(config-router)# version 2

Router2(config-router)# network 192.168.2.0

Router2(config-router)# network 10.0.0.0

Router2(config-router)# no auto-summary

Router2(config-router)# exit

Router2(config)# exit

- Router 3 configuration:

Router3> enable

Router3# configure terminal

Router3(config)# router rip

Router3(config-router)# version 2

Router3(config-router)# network 192.168.3.0

Router3(config-router)# network 10.0.0.0

Router3(config-router)# no auto-summary

Router3(config-router)# exit

Router3(config)# exit

### Step 4: Verify RIP Configuration

- Use the following command on each router to verify the routing table and ensure RIP has learned the routes from neighboring routers.

Router1# show ip route

#### Expected Output on Router 1:

Gateway of last resort is not set

R    192.168.2.0/24 [120/1] via 10.0.0.2, 00:00:15, Serial0/0/0

R    192.168.3.0/24 [120/1] via 10.0.0.2, 00:00:15, Serial0/0/0

C    192.168.1.0/24 is directly connected, FastEthernet0/0

C    10.0.0.0/30 is directly connected, Serial0/0/0

#### Expected Output on Router 2:

Gateway of last resort is not set

R    192.168.3.0/24 [120/1] via 10.0.0.6, 00:00:15, Serial0/0/1

C    192.168.2.0/24 is directly connected, FastEthernet0/0

C    10.0.0.4/30 is directly connected, Serial0/0/1

C    10.0.0.0/30 is directly connected, Serial0/0/0

R    192.168.1.0/24 [120/1] via 10.0.0.1, 00:00:15, Serial0/0/0

#### Expected Output on Router 3:

Gateway of last resort is not set

R    192.168.1.0/24 [120/1] via 10.0.0.5, 00:00:15, Serial0/0/0

R    192.168.2.0/24 [120/1] via 10.0.0.5, 00:00:15, Serial0/0/0

C    192.168.3.0/24 is directly connected, FastEthernet0/0

C    10.0.0.4/30 is directly connected, Serial0/0/0

### Step 5: Test Network Connectivity

Once the RIP configuration is complete, you can ping between devices in different subnets to check if routing is working correctly.

- From PC1 (connected to Router 1) to PC3 (connected to Router 3):

PC1> ping 192.168.3.2

#### Expected Ping Output:

Pinging 192.168.3.2 with 32 bytes of data:

Reply from 192.168.3.2: bytes=32 time=12ms TTL=255

Reply from 192.168.3.2: bytes=32 time=10ms TTL=255

Reply from 192.168.3.2: bytes=32 time=11ms TTL=255

If the ping is successful, it confirms that RIP has correctly propagated the routes across the network.

### Step 6: Save the Configuration

Make sure to save the configuration on each router to avoid losing it after a reboot:

Router1# copy running-config startup-config

Router2# copy running-config startup-config

Router3# copy running-config startup-config

### Conclusion

With these steps and configurations, you can successfully simulate RIP v2 on Cisco Packet Tracer version 8.1. You’ll be able to observe how routing updates are propagated across routers and verify connectivity between devices in different subnets. If you'd like to extend this to OSPF or EIGRP, the configurations would be similar, with protocol-specific commands.