Contents lists available at ScienceDirect

Ad Hoc Networks

journal homepage: www.elsevier.com/locate/adhoc

Ad Hoc Networks 178 (2025) 103983

Survey paper

Utilization of machine learning in future wireless networks for resource optimization: A survey

Mudassar Liaq a, Sana Sharif a, Sherali Zeadally b,c, Waleed Ejaz a,∗

a Department of Electrical Engineering, Lakehead University, ON, Canada

b College of Communication and Information, University of Kentucky, Lexington, KY, USA

c Imam Abdulrahman bin Faisal University (IAU), Dammam, Saudi Arabia

A R T I C L E  I N F O	A B S T R A C T

Keywords:

Deep learning Internet of things Machine learning Network

Optimization Resource management

Wireless communication

Future wireless networks will play an essential role as the need for performance and feature availability grows. Most of the traffic in future wireless networks is due to increased Internet of things (IoT) devices, making resource optimization critical. Traditional optimization algorithms have limitations due to their high computational complexity, which restricts their use in modern applications. To address this, machine learning algorithms are now the preferred alternative to traditional optimization algorithms due to their improved runtime complexity. We present a comprehensive survey on the use of machine learning for resource optimization in future wireless networks. The use of machine learning is divided into three categories: (i) comprehensive solutions, where machine learning is the primary component of the solution approach; (ii) partial solutions, where machine learning is used alongside a traditional approach for optimization; and

(iii) environment-only solutions, where optimization is performed in a machine-learning environment. We have further classified objective functions (e.g., energy, latency, data rate, etc.) within each category based on the pure objective function, variations on the objective function, and objective function tradeoffs with respect to other objective functions. We present objective functions and constraints used in the literature for optimization problem formulation. We provide an overview of frequently used machine learning algorithms for resource optimization, followed by a detailed survey of machine learning works in the literature in the three aforementioned categories. Finally, we discuss future research directions for utilizing machine learning to optimize resource management in future wireless networks.

# Introduction

Wireless networks have become increasingly important due to the growing number of connected devices and data generated. Future wire- less networks involve diverse users, devices, and applications in sixth- generation (6G) and beyond. These networks enable ultra-reliable, high-speed, low-latency communication across terrestrial, aerial (drones/satellites), and underwater systems. Emerging in the post-5G era, these networks address the increasing demands of artificial in- telligence (AI), IoT, augmented reality, and smart cities. Wireless networks have become increasingly essential due to the rising number of connected devices and the vast amounts of data they generate. As the number of connected devices continues to grow, existing networks are reaching their limits. The rapid expansion of applications for the IoT and emerging technologies such as autonomous vehicles and aug- mented reality requires robust, high-capacity connectivity to facilitate seamless communication.

Consequently, developing future wireless networks is vital to ad- dress the exponential growth in data demand, ultra-low latency require- ments, massive connectivity needs, enhanced reliability, security, and energy efficiency [1]. Resource optimization is critical to addressing the growing demands of an increasingly connected world in future wireless networks. The exponential increase in the number of devices, such as smartphones, IoT devices, and autonomous systems, coupled with the exponential growth in data traffic, places immense pressure on network resources. These resources, which include spectrum, band- width, and energy, must be utilized efficiently to ensure the seamless operation of networks under such conditions. The radio spectrum is a finite resource, and its efficient use is crucial to meet next-generation networks’ high throughput and low-latency demands. Advanced tech- niques like dynamic spectrum access, cognitive radio, and spectrum sharing allow networks to adaptively allocate frequencies based on real- time requirements. These strategies reduce interference and maximize the available spectrum’s capacity, enabling networks to simultaneously

∗ Corresponding author.

E-mail addresses: mliaq@lakeheadu.ca (M. Liaq), ssharif@lakeheadu.ca (S. Sharif), szeadally@uky.edu (S. Zeadally), wejaz@lakeheadu.ca (W. Ejaz).

https://doi.org/10.1016/j.adhoc.2025.103983

Received 5 May 2025; Received in revised form 24 June 2025; Accepted 18 July 2025

Available online 28 July 2025

1570-8705/© 2025 The Authors. Published by Elsevier B.V. This is an open access article under the CC BY license (http://creativecommons.org/licenses/by/4.0/).

handle more devices and data streams. Bandwidth, a key determinant of data transfer rate, becomes increasingly scarce when data-intensive applications like streaming, cloud computing, and augmented reality grow in popularity. Innovative bandwidth management techniques, such as multiplexing, beamforming, and network slicing, are essential for optimizing bandwidth usage. By partitioning and prioritizing band- width allocation based on user needs, these techniques ensure support for applications requiring higher bandwidth, thereby enhancing overall network performance.

Furthermore, energy consumption in wireless networks is another critical concern, particularly with the shift towards green and sustain- able communication systems. As the number of connected devices in- creases, energy efficiency becomes paramount for reducing operational costs and minimizing the environmental impact. Techniques such as energy harvesting, energy-aware routing, and machine learning models for predictive resource management are being explored to optimize energy utilization. These approaches enable networks to balance per- formance with energy savings, ensuring sustainable growth. Effective resource optimization contributes significantly to wireless networks’ reliability, scalability, performance, and user experience. By reduc- ing bottlenecks and enhancing the dynamic allocation of resources, networks can maintain high-quality service levels even under peak loads, ensuring networks’ scalability to support future demands and improved reliability for mission-critical applications such as healthcare and autonomous navigation systems. Furthermore, optimized resources enhance user experience by minimizing latency, improving throughput, and ensuring consistent connectivity. Hence, resource optimization is indispensable for the evolution of future wireless networks. With the integration of advanced technologies such as AI, edge computing, and network virtualization, the effective utilization of spectrum, bandwidth, and energy will be pivotal in addressing the challenges posed by the growing demands of a connected world.

Traditional optimization approaches often lack the flexibility and adaptability required to handle the dynamic nature of future wireless environments. Conventional approaches rely on fixed models and pre- defined rules, making them less effective in responding to real-time changes and varying network conditions. To address the above limita- tions, machine learning algorithms can play a pivotal role in managing and optimizing resources in future wireless networks. In contrast to traditional optimization algorithms, machine learning algorithms can learn from data, adapt to new scenarios, and continuously improve their performance. This capability makes them highly suitable for managing the complex and dynamic challenges associated with future wireless networks [2,3]. By leveraging large amounts of network data, these algorithms can make data-driven decisions for resource alloca- tion, ensuring that networks operate more efficiently and intelligently. The ability of machine learning algorithms to be parallelized and distributed allows them to scale effectively with the increasing size and complexity of modern wireless networks [4,5]. This scalability is essential as the number of connected devices and the demand for high-speed, low-latency communication continue to grow.

Machine learning has numerous potential applications in resource optimization in wireless networks. One key application is dynamic adaptability, where algorithms adjust resource allocation in real-time based on current network demands and conditions. Predictive analysis is another crucial area, allowing networks to forecast future resource requirements and proactively manage resources to prevent conges- tion and inefficiencies. Additionally, machine learning enables context- aware decision-making when allocating resources by considering user behavior, device capabilities, and network topology. This approach ensures that resources are used in the most effective manner possible. Machine learning also contributes to inference mitigation by min- imizing errors in resource allocation decisions, thereby improving network reliability and performance. Real-time adaptation to net- work changes is another significant advantage because machine learn- ing algorithms can quickly respond to variations in traffic patterns,

device mobility, and environmental conditions [6,7]. These capabilities are critical for achieving efficient, adaptive, and intelligent network management in the face of rapidly evolving communication needs. Overall, integrating machine learning into resource optimization strate- gies represents a transformative step for future wireless networks. By addressing the shortcomings of traditional approaches and enabling advanced functionalities such as dynamic adaptability and predictive analysis, machine learning ensures that networks can meet the growing demands for high performance, scalability, and user satisfaction. The continued development and implementation of machine learning-based algorithms will be instrumental in shaping the next generation of wireless communication systems.

Our survey is unique and comprehensive because it provides a detailed review of machine learning-based resource management so- lutions in future wireless networks, covering the most recent six years, i.e., 2020 onwards, of the literature. Unlike existing surveys [8–22], we categorize the literature based on the level of integration of machine learning techniques, distinguishing between comprehensive use, par- tial use, and optimization within machine learning-only environments. We offer a refined perspective on resource management by analyz- ing whether recent works employ traditional optimization techniques, machine learning-based approaches, or both. Additionally, our survey identifies critical research gaps and highlights lessons learned within each category, offering valuable insights. Finally, we present open research challenges, discuss future directions and potential solutions, ensuring a well-rounded contribution to advancing future wireless networks.

  - Existing surveys

Recently, several efforts have surveyed the literature on resource optimization in wireless networks. For example, the authors of [8] provided a comprehensive survey on machine learning for end-to-end network optimization. They considered the use of machine learning to address issues such as network access, routing optimization, net- work congestion control, and adaptive streaming control. They covered the literature in a broad context where problems are formulated in various ways (e.g., Markov decision process (MDP), game theory, liq- uid state machine, and game auction theory). Their work discussed constraints such as quality of service (QoS), queue-based, and data rate with a generic focus, which was limited due to generalization. In [9], the authors presented a survey of traditional approaches for resource optimization and then covered modern machine learning tech- niques deployed in cellular and IoT networks for resource optimization and management. The authors of [10] examined various technologies employed in 5G networks, such as software-defined network (SDN), milli meter wave (mmWave), network function visualization (NFV), massive multiple inputs multiple outputs (MIMO), and cloud radio access network (CRAN) to enhance resource optimization with a focus on energy efficiency. Their study also explored the role of machine learning in enabling green initiatives within in 5G systems and pro- vided an overview of standardized green metrics designed to improve energy efficiency in telecommunication networks. In [12], the authors presented a survey on machine learning for ultra reliable low latency communication (URLLC) in 6G networks. They focused on the use of machine learning approaches (i.e., supervised, unsupervised, and semi- supervised (e.g., deep reinforcement learning (DRL)) in the context of URLLC. They highlighted research results that optimize reliability and delay. In [13], the authors presented a survey on resource optimization in unmanned aerial vehicle (UAV)-assisted wireless networks. They considered multiple factors such as charging, spectrum, data offloading, and joint resource metrics. The authors analyzed traditional algorithms and game-theory-based solutions for resource optimization. In [14], the authors discussed the potential implementation of AI in various areas of 6G networks. They focused on using AI in advanced radio interference, intelligent traffic control, network optimization, and network security.

Regarding QoS, quality of experience (QoE) management, and energy efficiency within network optimization, the authors primarily focused on machine learning-based solutions.

In [18], the authors investigated the use of machine learning and explainable artificial intelligence (XAI) for resource allocation in 6G networks, with a particular focus on network slicing within vehicular networks. They categorized several XAI algorithms such as shapley additive explanations (SHAP), local interpretable model-agnostic expla- nations, counterfactual explanations, layer-wise relevance propagation, permutation importance, and Bayesian reinforcement learning and ex- amined their applicability across various layers of the communication stack. In [19], the authors categorized the existing literature on the use of AI-based techniques, including machine learning, deep learn- ing, and DRL, as well as heuristic algorithms in fog networks. Their study focused specifically on areas such as resource management, task scheduling, load balancing, and anomaly detection. They analyzed the impact of these algorithms on a limited range of objective function categories, namely QoS, energy efficiency, and latency. In [20], the authors concentrated on the application of XAI in 6G networks. They explored the technical architecture and various use cases of XAI within 6G environments. Additionally, they provided a layered breakdown of XAI-powered 6G architectures, which included the intelligent sensing layer, data mining layer, intelligent controller layer, and smart appli- cation layer. In [21], the authors investigated resource management in IoT networks within 6G environments. They categorized resource management into four main areas: (i) network-aware resource man- agement, which allocated resources based on network conditions and architecture; (ii) dynamic resource allocation, which focused on real- time management of resources in response to changing demands; (iii) predictive resource distribution, which utilized topology and traffic forecasts to proactively manage resources; and (iv) energy-centric allo- cation, which aimed to minimize power consumption based on location and mobility patterns.

In [22], the authors explored resource management in 6G net- works using AI-based techniques, with a strong focus on enabling green communications. They categorized resource management into three key domains: (i) communication network resource management, encompassing base station (BS) operations, user association strategies, and machine-type communications; (ii) radio resource management, involving spectrum allocation, channel assignment, user access control, and power management; and (iii) computing and caching resource management, which addressed edge computing infrastructures and content distribution. The study emphasized the potential of machine learning approaches to advance green communication objectives and introduced a comprehensive taxonomy of AI-enabled resource manage- ment strategies. In [15], the authors surveyed machine learning-based techniques for optimization problems in wireless networks. They pre- sented machine learning algorithms such as deep neural networks (DNNs), DRL, and federated learning (FL). They considered various applications of machine learning in areas such as BS placement, of- floading, queue parameters, and vehicular communication. The authors categorized these applications based on different objective function types such as offloading, algorithmic parameters, and data rate. In [16], the authors focused on radio resource management in the UAV-assisted next-generation computing paradigms. They sub-classified these works based on different parameters such as strategies, problem formula- tions and optimization objectives and provided solutions to address the problems in UAV-assisted computing environments. In [17], the authors organized their works based on cache, compute, communicate, and control (4Cs). They categorized the problem formulated and the optimization problem based on how the number of Cs involved (1C, 2C, 3C, or 4Cs). They solved these problems using machine learning algorithms but limited themselves to only DRL and FL problems.

  - Research motivation:

Table 1 presents an overview of recent surveys on resource opti- mization with respect to machine learning in future wireless networks.

These surveys are categorized based on their primary area of re- search, whether they are using traditional optimization techniques or machine learning approaches. Some surveys focused on resource opti- mization from various perspectives, such as resource management tech- niques [9], generic wireless communication [11], scalability and trust- worthiness in AI [4], machine learning at the infrastructure level [2], bio-inspired machine learning algorithms for resource optimization [6], and machine learning for IoT devices [3]. Other surveys focus on specific subsets of objectives in resource optimization. This survey [12] exclusively addresses latency in 6G networks, while [7] focuses on offloading strategies only. These works predominantly explore op- timization using various traditional algorithms or machine learning applications in future wireless networks, except [13,16,23], and [17]. While these latter works share a similar contextual framework, they either omit machine learning-based methodologies [13,16,23], or se- lectively cover specific categories [17]. Specifically, in [13], the focus is more aligned with our area of interest; however, their analysis primarily emphasizes solutions based on traditional approaches. From the review of the literature above, we conclude found that limited work analysis of the literature has been conducted in the area of wire- less communications to analyze the literature where machine learning serves as a core component.

Our review of the existing literature investigates the role of machine learning in wireless networks. Most prior works either present machine learning as a minor component within broader solution approaches or focus on highly specific use cases, lacking a comprehensive analysis. Motivated by these shortcomings, our work aims to highlight this gap by systematically organizing the literature according to the extent of machine learning integration and further sub-classifying it based on the nature of the objective functions considered. By structuring and analyzing the existing body of work in this way, we aim are able to contribute to a deeper understanding of how machine learning is currently applied in various wireless network scenarios and how it can be more effectively utilized to improve overall network performance.

  - Research contributions

Our survey is a unique contribution to the field of wireless com- munications due to its comprehensive analysis of machine-learning techniques applied to resource optimization in future wireless net- works. We focus on the challenges traditional optimization approaches face and how machine learning algorithms are utilized for context- aware and adaptive resource optimizations. We categorize the use of machine learning-based solutions for resource optimization in the existing literature into three different categories as Fig. 1 shows:

- Comprehensive solutions, in which authors used machine learn- ing primarily to solve the resource optimization problem. The level of machine learning involvement can range from using machine learning as the only solution to using it as a significant component in solving the problem.

- Partial solutions, in which authors used both machine learning and traditional optimization algorithms to solve the part of the problem. The extent to which traditional algorithms are used, determines whether a solution is comprehensive or partial. We consider work in this category if traditional algorithms are used equally or more than machine learning.

- Machine learning-based environment solutions, in which au- thors formulate and solve resource optimization problems in machine learning-based wireless communication environments. These solutions utilize traditional and machine learning approaches to solve the issues formulated.

The main contributions of this survey paper are:

- Comprehensive literature review (2020 and onwards): We comprehensively review publications that use machine learning- based resource management solutions in future wireless networks.

Table 1

Existing surveys in machine learning and optimization in 5G/6G/future wireless networks.

4Cs: cache, compute, communicate, and control; 5G: fifth-generation; 6G: sixth-generation; AI: artificial intelligence; BS: base station; FL: federated learning; IoT: Internet of things; QoE: quality of experience; QoS: quality of service; UAV: unmanned aerial vehicle; URLLC: ultra reliable low latency communication; XAI: explainable artificial intelligence.

Ref.	Year	Focus of survey	Remarks

- 2021	Focus on machine learning for end-to-end network optimization.

They cover machine learning for network access (channel allocation within the medium access control layer), routing optimization,

network congestion control, and adaptive streaming control.

- 2020	Focus on traditional resource management, machine learning-based resource management, and resource optimization in emerging cellular IoT networks.

- 2020	Focus on improving resource optimization in various technologies (e.g., software-defined networking) within 5G networks in the context of energy efficiency.

- 2021	Focus on machine learning approaches to optimize reliability and delay for works in URLLC spectrum.

- 2021	Focus on resource management and optimization for UAV-networks considering factors like spectrum, data offloading and joint resource metrics in traditional optimization algorithms and game-theory

based approaches.

- 2020	Focus on the use of AI in 6G networks leveraging machine learning based solutions for advanced radio interference, intelligent traffic

control, network optimization, and network security, QoS, QoE management, and energy efficiency.

- 2021	They focus on the application of machine learning techniques for BS placement, offloading, queue parameters, and vehicular

communication with classification over different system parameters.

- 2022	Focus on radio resource management in UAV-assisted next-generation computing paradigms.

- 2022 Focus on resource optimization works based on the 4Css and their solutions are based on traditional optimization approaches as well as reinforcement learning and FL among machine learning

approaches.

- 2024  Focus on use of AI-based techniques—including machine learning and heuristic algorithms in fog networks.

We categorize and cover the problem formulation in a lot more detail (compared to their network access, routing, and adaptive streaming control).

Their focus is on management techniques (which drives the context of their paper), while we are interested in their formulation process and then the

ML-based proposed solution approaches.

Their work focuses on energy efficiency while we cover resource management, including energy efficiency as one of its components.

They consider latency and reliability improvement in their work. We focus on categorizing factors that control them (i.e., latency, energy efficiency) along with other mathematical formulations and ML-based solutions.

They consider solutions based on traditional optimization methods, while we consider works based on the extent of ML utilization (partial, comprehensive, or environment only).

With partial overlap in energy efficiency and QoS/ QoE management, they focus on ML solutions only, while we also consider formulations and solution details.

They cover different objective functions (e.g., throughput maximization, energy efficiency maximization, transmit power minimization) briefly based on machine learning techniques utilized by researchers to solve the problem. For each ML technique they then consider one model environment and focus on problem

formulation for that specific problem.

They consider solutions based on traditional optimization methods while we consider works based on the extent of ML utilization (partial, comprehensive, or environment only).

They cover only DRL and FL in ML solutions, while we cover (almost) all

common types of ML algorithms. They focus on the 4Cs classification while our focus is on the objective function’s goals (e.g., time, power). We also cover the extent of ML utilized in the solution approach.

They only addressed a few classes of objective functions and maintained a broader focus on both AI-based and heuristic solutions. In contrast, our study concentrates specifically on machine learning-based approaches, providing a more detailed analysis of objective functions and solution strategies.

- 2024	Focus on the application of XAI in 6G networks.	Their work was focused exclusively on XAI and did not address the specific

objectives to which it was applied. In contrast, our study considered a wider range of machine learning methods, placing particular emphasis on the associated objective functions.

- 2024	Focused on resource management in IoT networks within 6G environments.

- 2024	Focused on resource management in 6G networks through AI techniques, placing particular emphasis on promoting green

communications.

- 2025	Focus on use of machine learning and XAI for resource allocation in 6G networks, with a particular focus on network slicing within vehicular networks.

Their study concentrated solely on resource allocation and did not specifically explore the use of machine learning techniques. In contrast, our research

primarily focused on resource allocation through machine learning-based methods.

They concentrated on green resource management in 6G networks using AI, our study focused on resource management through machine learning, providing a detailed analysis of objective functions and various solution approaches.

They focus on XAI algorithms only, while we cover (almost) all common types of ML algorithms and focus on the objective function’s goals (e.g., time, power). We also cover the extent of ML utilized in the solution approach.

Our

survey

2025	Focuses on the utilization of machine learning for resource optimization in future wireless networks.

We cover works that utilize machine learning to different extents (complete, partial, and environment-only) in resource optimization.

We survey recent works on resource management and analyze whether they used traditional approaches of optimization or ma- chine learning-based optimization techniques.

- Categorization of machine learning techniques based on en- vironment: We categorize the recent literature based on machine learning application techniques based on the use of machine learning, partial use of machine learning, and optimization within a machine learning environment-only. We then conduct a compre- hensive review state-of-the-art works utilizing machine learning in future wireless networks.

- Sub-categorization of each objective function: We further clas- sify each of the objective functions discussed above into three

subcategories: (i) pure objective functions, where the optimiza- tion parameters are directly determined by the objective under given constraints; (ii) variations of objective functions; and (iii) objective function trade-offs.

- Research gaps: We extensively analyze the available literature

to find out the areas where more research focus is required and we summarized the results as lessons learnt at the end of each category.

- Open issues and future work: Finally, we identify open research

challenges and future directions in future wireless networks and propose potential solutions to address these challenges.

/ig. 1. Machine learning based classification of works for resource optimization in future wireless networks.

/ig. 2. Organization of paper.

Fig. 2 shows the organization of this paper. We introduce objective functions and constraints utilized in literature for resource optimiza- tion in 2. We then briefly survey machine learning algorithms used in literature for resource optimization in Section 3. After that, we do a comprehensive survey of machine learning works for resource optimization based on our classifications (i.e., comprehensive machine learning utilization in Section 4, partial machine learning utilization in Section 5, and machine learning environment only utilization in Section 6). Finally, we conclude with challenges and future work at the end in Section 7. Table 2 presents a list of acronyms used in this paper.

# Research methodology

We used the methodology based on [24,25] to identify the litera- ture on machine learning for resource optimization in future wireless networks.

  - Research questions

The goal of this work is to investigate the role of machine learning in the optimization of future wireless networks. To achieve this goal, we address the following research questions:

- RQ1: How is machine learning employed to optimize resource allocation in future wireless networks? (Section: 3.3)

- RQ2: Which machine learning algorithms are used for resource optimization in future wireless networks? (Section: 3.3)

- RQ3: What types of objective functions and constraints are solved using machine learning? (Sections: 3.1, 3.2)

- RQ4: To what extent can machine learning algorithms replace traditional algorithms in solving optimization problems in fu- ture wireless networks? or will it only have a supportive role? (Sections: 4, 5, 6)

Table 2

List of Acronyms.

Acronym	Description

4Cs	Cache, compute, communicate, and control

Table 3

Search queries.

Journal	Search String	Other constraints	Relevant papers

5G	Fifth-generation

6G	Sixth-generation

ADMM	Alternating direction method of multipliers

AI	Artificial intelligence

ANN	Artificial neural network

B&B	Branch and bound

BS	Base station

CRAN	Cloud radio access network

CSI	Channel state information

D2D	Device to device

DCA	Difference of convex algorithm

DDPG	Deep deterministic policy gradient

DDQN	Double deep Q network

DNN	Deep neural network

IEEE

Science Direct

(‘‘resource optimization’’ OR ‘‘resource allocation’’ OR ‘‘resource management’’) AND (‘‘machine learning’’ OR ‘‘ML’’ OR ‘‘AI’’ OR ‘‘artificial intelligence‘‘) AND (’’future wireless networks’’ OR ‘‘5G’’ OR ‘‘6G’’)

| DQL | Deep Q-learning |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DQN | Deep Q network | ACM | [[All: ‘‘resource optimization’’] | 2020–2025, | 3 |
| DRL | Deep reinforcement learning |  | OR [All: ‘‘resource allocation’’] | Journal articles |  |
| DTRL | Deep transfer reinforcement learning |  | OR [All: ‘‘resource | only |  |
| FDMA | Frequency division multiple access |  | management’’]] AND [[All: |  |  |
| FL | Federated learning |  | ‘‘machine learning’’] OR [All: |  |  |
| GAN | Generative adversarial network |  | ‘‘ml’’] OR [All: ‘‘ai’’] OR [All: |  |  |
| HAB IoT INLP IRS KKT | High altitude baloon Internet of things Integer non-linear programming problem Intelligent reflecting surface Karush–Kuhn–Tucker |  | ‘‘artificial intelligence’’]] AND [[All: ’’future wireless networks’’] OR [All: ‘‘5G’’] OR [All: ‘‘6G’’]] AND [Publication Date: (01/01/2019 TO 01/04/2025)] |  |  |
| KLD | Kullback–leibler divergence | Arxiv | order -announced-date-first size: | – | 12 |
| KNN | K-nearest neighbors |  | 50 date-range from 2020-01-01 |  |  |
| LEOS | Low earth orbit satellite |  | include-cross-list True terms (AND |  |  |
| LSTM | Long short term memory |  | all = Resource allocation OR |  |  |
| MDP | Markov decision process |  | all=resource optimization OR all |  |  |
| MEC | Mobile edge computing |  | = resource management) (AND |  |  |
| MILP | Mixed integer linear programming |  | all = machine learning OR all = |  |  |
| MIMO | Massive multiple inputs multiple outputs |  | AI OR all = ML) (AND all = |  |  |
| MINLP | Mixed-integer nonlinear programming |  | future wireless networks OR all = |  |  |
| MIP mmWave | Mixed integer programming Milli meter wave |  | 5G OR all =6G) |  |  |
| NFV | Network function visualization |  |  |  |  |
| NOMA | Non orthogonal multiple access |  |  |  |  |

(‘‘resource optimization’’ OR ‘‘resource allocation’’ OR ‘‘resource management‘‘) AND (‘‘machine learning’’ OR ‘‘ML’’ OR ‘‘AI’’ OR ‘‘artificial intelligence‘‘) AND (’’future wireless networks’’ OR 5G OR 6G)

2020–2025,

Journal and early access articles only

2020–2025,

Review and research articles only

104

18

NP	Nondeterministic polynomial time

ORAN	Open radio access network

PCCP	Penalty convex-concave procedure

PEN	Patient edge network

PPO	Proximal policy optimization

QoE	Quality of experience

QoS	Quality of service

RAN	Radio access network

RNN	Recurrent neural network

SDN	Software-defined network

SHAP	Shapley additive explanations

SINR	Signal-to-interference-plus-noise ratio

SIR	Signal-to-interference ratio

SVM	Support vector machine

UAV	Unmanned aerial vehicle

UE	User equipment

URLLC	Ultra reliable low latency communication

V2N	Vehicle-to-network

V2V	Vehicle-to-vehicle

XAI	Explainable artificial intelligence

  - Search process

We derive keywords and formulate search strings based on pop- ulation, intervention, comparison, and outcomes (PICO) [26]. These (keywords) include ‘‘resource optimization’’, ‘‘resource allocation’’, ‘‘resource management’’, ‘‘machine learning’’, ‘‘artificial intelligence’’, ‘‘AI’’, ‘‘ML’’, ‘‘future wireless networks’’, ‘‘5G’’, and ‘‘6G’’. Table 3 shows search queries for each database. We searched the IEEE-Explore, ACM, Arxiv, and ScienceDirect databases with these keywords, limiting our search to journal papers published in 2019 and onwards. We use Zotero to manage and eliminate duplicates from the reference works. We

include literature focusing on mathematical optimization along with machine learning.

# Resource optimization: Key components

In this section, we present the most common objective functions (e.g., energy, latency, capacity, etc.) covered in the literature for ra- dio resource management. Afterward, we discuss the constraints that bound these objective functions. We end the section by covering most common machine learning solutions utilized in the literature for radio resource management. Fig. 3 shows a general framework for resource optimization using machine learning algorithms.

  - Objective function

The objective function determines the parameters to optimize based on given constraints. We identify five major objective functions con- sidered in the literature for resource optimization in future wireless networks:

    - Energy

Energy is one major element in managing the performance of future wireless networks. Most works include optimization of power [27– 30] energy efficiency [31–33] or some combination of energy effi- ciency [34–36] as their objective.

    - Sum rate

These objective functions encourage maximum data flow within the given domain. These include sum rate, data rate, throughput, and average data rate [37–62].

/ig. 3. Generalized framework for resource allocation in future wireless networks.

    - Latency

These objective functions focus on time-based parameters. These include latency, delay, and sometimes variation (e.g., communication, computation) of delays [63–78].

    - Capacity

End nodes sometimes need more processing or communication ca- pacity in future wireless networks to perform the necessary tasks. This can affect the performance of the whole network (if this node becomes a bottleneck). These capacity issues can be a lack of processing power, storage capacity, communication bandwidth, or any combination of these. Capacity is considered in the literature as an objective to enhance the performance of the network [79–84].

    - Algorithmic parameters

Future wireless networks can be designed by considering optimiza- tion functions that optimize algorithmic parameters. These algorithmic parameters can vary from queue length to bin height in different algorithms [85–94].

    - Less represented and task-specific formulations

For formulations that do not fit the above categories due to their limited representation in literature, we categorize them as ‘‘less rep- resented and task-specific formulations’’. Revenue [95], profit per user [96], transmission probability [37], node overhead [84], and user satisfaction [45] are a few examples that fall within this category. Work done in [95–102] also fall under this category.

  - Constraints

We broadly categorize the constraints considered in the resource optimization literature into communication, computation, and power.

    - Communication constraints

Communication constraints include different factors such as data rate, subchannel, transmission power, connection, distance, link, delay,

QoS, and queue stability. The sum of the achievable rates is the sum rate. In contrast, throughput denotes the amount of data exchanged between entities, such as and BS, or BS and cloud services. In recent studies such as [89,90,92], the authors have addressed data rate and throughput constraints in their optimization frameworks.

Queue constraints are related to limitations associated with network queues typically found in edge devices [27,54]. Spectrum management involves subchannel or spectrum constraints, including subchannel al- location considerations [63,73,103]. Transmission power constraints aim to regulate power consumption specifically for communication pur- poses, distinct from overall system power consumption [96,104,105]. Link constraints govern the feasibility and limitations of establishing or maintaining communication links between entities—such as user equipments (UEs) and BSs [29,103,106] and the extent to which those links support resource allocation or sharing in radio access networks (RANs) [41,46,50]. Certain studies optimize the placement of relays or mobile BSs, often incorporating distance constraints to restrict the prob- lem’s scope [30,44,73]. Time-based constraints predominantly regulate latency or delay metrics for specific sub-problems [31,74,106].

QoS constraints are relevant when addressing connection quality concerns and content delivery standards, including criteria like inter- ference, error rates, and signal-to-interference-plus-noise ratio (SINR). Various studies have combined these constraints into their work [83, 86,99]. Rest of Communication constraints classified task-specific con- straints, such as satellite beam [107], intelligent reflecting surface (IRS) bounds, offloading decisions [34], traffic path [65], UAV speed [73], access class barrier [49], signal-to-interference ratio (SIR) [40,108], phase shift and amplitude [73], packet transmission ratios [49], analog beamforming [61], and FL training communication [109].

    - Computation constraints

All constraints involving available computational resources, whether on the edge, UEs, or cloud, are collectively called computation con- straints. These contain various types, including resource allocation constraints, which are related to the quantity of resources allocated to requesting entities like UEs by hosting entities such as BSs. Priority constraints such as bandwidth or latency, prioritize treatment across different classes based on classification or groupings. Task scheduling constraints dictate the execution or processing of specific tasks, like FL iterations, on particular nodes, i.e., UEs. The rest of the constraints fall under compute resource utilization constraints. Some examples would be software caching bounds [34], CPU frequency and speed [36,70, 110,111], task coordination at different devices [63,112], CPU fraction allocation [68], cache allocation [74,113,114], strategy profile selec- tion for processing [66], time allocation for processing [65], battery bounds [72], memory utilization [70], task accuracy [115,116], data size bounds [115,116], data storage bounds [117,118], and virtual machine assignment [95].

    - Power constraints

All constraints that deal with system power (except for transmission power) are considered power constraints. The authors of [44,71,72] included these constraints in their work.

  - Machine learning algorithms

Several machine learning algorithms are considered in the literature for resource optimization. We give a brief summary of each in this section.

    - Support vector machines (SVMs)

In its most basic form, support vector machines (SVMs) (linearly) separates the data into two groups using a hyperplane (specifically, it

divides 𝑛-dimensional vectors using a 𝑛 − 1 dimensional hyperplane). The best choice for vector division is the hyperplane that generates the maximum separation between data points. In many cases, data separation is not achievable in the original vector’s dimensional space. Kernel functions (e.g., polynomial or Gaussian) map the data points into higher dimensions resulting in the nonlinear classifier. The authors of [119] comprehensively explored SVMs.

    - K-nearest-neighbors (KNN)

K-nearest neighbors (KNN) is another way of classifying (or regress- ing) data points without any assumption on data distribution. KNN classify the test data points into a class based on majority votes from its nearest neighbors. In advanced approaches, the weighted method is sometimes adopted (e.g., the weight of each neighbor is directly proportional to its distance from the test data point) to improve pre- diction accuracy. 𝐾 is a tuning parameter set to designate the number of neighbors used in the voting mechanism. The authors of [120] have worked extensively on KNNs.

    - Deep neural networks (DNNs)

These networks are composed of neurons that take input, have asso- ciated weights, and generate activation functions for output. The input is transformed as it propagates in network layers, with no connection between layers (in the most basic form) except between two consecu- tive layers. Backpropagation [121] and gradient descent methods [122] are used for optimization within these networks.

    - Recurrent neural networks (RNNs)

Recurrent neural networks (RNNs) have almost the same structure as DNNs but contain connections within the same hidden layer. The output is dependent on both current inputs as well as historical inputs. The most common flavor of RNN is long short term memory (LSTM), which is utilized the most in the literature in this work. The original work on RNNs can be found in [123].

    - Deep reinforcement learning (DRL)

In reinforcement learning, the agent tries to optimize the long-term goal in its interaction with the environment (by trial and error). The most common type of reinforcement learning used is Q-learning-based, in which the agent takes action and tries to learn a Q-value from the environment. The Q-value is a discounted cumulative reward, which is a measure to know, how good a decision is. Once the Q-value is learned (after some time), the agent can make better decisions based on choosing an action with the maximum Q-value. More details can be found in [124]. If these Q-values are approximated using a DNN, it becomes DRL [125]. The generated state in the agent’s interaction with the environment is stored and sampled to train the deep Q network (DQN). More details on DRL can be found in [124,125].

    - Federated learning (FL)

Due to the privacy concerns of user data, the concept of FL was introduced [126]. FL allows users to keep their data personal while contributing to training a global model. They do so by having a local model and training it on its local resources (i.e., data). Once the model is trained, they transmit the computed parameters to the connecting server.

In the next three sections, we provide a comprehensive and detailed view of research conducted within each category.

# Machine learning as a comprehensive solution

The literature in this section covers comprehensive machine learning-based solutions for formulated problems  in future wireless

/ig. 4. Machine learning as a comprehensive solution.

networks. Specifically, we focus on the literature where optimization problems were mathematically formulated with an objective function and constraints, and then solved with the help of machine learning methods. Some authors have proposed and replaced traditional op- timization solutions with better machine learning solutions to entire problems. Others had not considered a traditional approach and had only given a machine learning-based solution. A few authors have transformed problems using traditional techniques (e.g., Lagrange mul- tipliers) and solved the transformed problem using machine learning. In this section, we reviewed the literature that fulfilled the aforemen- tioned criteria.

Based on the available literature we reviewed for this paper, we further categorized the works based on their objective functions. We categorized the works under energy, latency, data rate, capacity, algo- rithmic parameters, and ‘‘less represented and task-specific’’ categories based on each category’s frequency in the literature as Fig. 4 shows.

  - Energy minimization

In wireless network systems, power consumption refers to the ag- gregate energy usage of the entire network infrastructure under con- sideration. This includes power consumed by all network components, including BSs, access points, end devices, and backend processing units, while maintaining an operational routine. Recent studies have utilized machine learning for power minimization in future wireless networks [27,111].

    - Pure objective function

This section explores power-focused optimization problems in wire- less networks, highlighting various approaches aimed at minimizing system power consumption while addressing challenges such as re- source allocation, mode selection, and energy efficiency under diverse communication and computation constraints.

The authors of [27] formulated an mixed integer programming (MIP) problem to optimize node selection and resource allocation in sliced fog RANs. The authors optimized system power with several constraints related to compute resource utilization (CPU utilization), QoS, link, and transmit power. First, the authors solved the problem using a solution based on Lyapunov optimization and branch and bound (B&B). The authors then proposed centralized and distributed DRL-based solutions. The centralized approach iteratively found the optimal solution at a central node. In contrast, in the distributed approach, each BS was independent in selecting its communication modes, generating one-to-one mapping between UEs and BS nodes. Based on this mapping, they used a block coordinate descent based algorithm to find the optimal solution. The performance of the system can be further enhanced by integrating robustness against model drift and incorporating generalization algorithms. The authors showed that the performance decrease (in terms of increase in power consumption) of their proposed approach was about six percent, compared to the orthogonal approach, which has a twelve percent decrease when the number of devices grows.

In [111], the authors focused on a UAV-assisted marine IoT environ- ment, where a UAV is deployed with some unmanned surface vehicles. The unmanned surface vehicles offload the computation-intensive tasks to UAVs based on their availability. They formulated the optimization problem to minimize the system’s total energy consumption with seven constraints related to compute resource utilization (CPU frequency), transmission power, link, delay and task-specific (UAV velocity) con- straints. The formulated problem was nondeterministic polynomial time (NP)-hard. They proposed a closed-form solution using Lagrangian and gradient descent for resource allocation. They utilized the deep deterministic policy gradient (DDPG) algorithm for the UAV trajectory to find the optimal route. They then proposed a layered optimization algorithm that optimizes resource allocation and UAV trajectory based on the abovementioned approaches. They evaluated their algorithm with fixed offloading and trajectory, considering only one UAV and four unmanned surface vehicles in the simulation environment. The authors showed significant improvement in energy consumption as compared to fixed trajectory and fixed offloading strategies.

The authors of [29] addressed power allocation and computation offloading in a dense distributed IoT environment. They formulated the problem to minimize energy consumption and delay, considering communication and power constraints as an mixed-integer nonlinear programming (MINLP) problem. They developed a solution in two parts: DNNs were first used to generate optimal offloading decisions, and then a projected sub-gradient method was applied for optimal power allocation. Initially, the DNN had no training data, resulting in random guesses for optimal decisions. Over time, with training, the system improved its predictions. In this work, initial random guesses could be enhanced using sample datasets generated from traditional optimization algorithms. They showed that the deployment of their proposed solution can significantly improve the system capacity and spectral efficiency.

The authors of [30] optimized power allocation and relay location in a network with a direct link from the host BS to the device and a secondary link through a nomadic relay node. They formulated the problem to optimize power consumption and relay node location with distance and power constraints. The problem was formulated as strictly convex, and they proposed a convex optimization-based algorithm to solve it. They then converted this problem into a deep learning problem, using a regression approach to generate the dataset and train DNN to solve it. They considered one BS for dataset generation in their simulation. Expanding this work to more BSs and connected relay nodes can improve their work. They showed in their work that a machine learning based approach reduces run-time complexity by a factor of a hundred times, at the expense of one percent of accuracy.

The study in [28] focused on optimizing power allocation in co- located and distributed antenna systems through machine learning

techniques. They devised a problem to maximize transmission power, with the only constraint being that the total transmitted power must not exceed the available power budget. Initially, they proposed an algorithm using a sub-gradient approach with Lagrange multipliers and found the optimal solution through a bisection search. Later, they introduced a solution using multi-class SVM (one-vs-all classification). They divided users into training groups and stored their location dis- tribution and power allocation in a database. They created the training dataset using the sub-gradient and Lagrange methods and then tested it on user groups in a separate database. Considering the imperfect channel state information (CSI) in their assumptions could lead to further enhancements in their work. The authors showed that using SVM, they can achieve the same performance for transmitting power with the added advantage of low run-time complexity of SVMs.

    - Objective function variations

Improving energy efficiency and optimizing performance metrics such as spectral efficiency, transmission power, and latency have become critical in modern communication systems, particularly in IoT and wireless networks. Various strategies, including optimization frameworks, DRL, and machine learning-based approaches, have been proposed to address these challenges while meeting stringent QoS requirements.

The authors of [106] proposed a green resource allocation frame- work for industrial IoT in 5G to improve energy efficiency while maintaining QoS requirements. They formulated the problem as an en- ergy efficiency optimization, defined as the total data rate over energy consumption, with communication, transmission power, connection quality, and delay constraints. To solve this NP-hard problem, they used hierarchical agglomerative clustering to form femtocells industrial IoT equipment clusters and applied an actor-critic DRL approach for re- source allocation prediction. They proposed an algorithm incorporating these steps and compared their work against a genetic algorithm base- line, showing better performance with all three tested reward functions. They compared the simulation results using traditional optimization benchmarks and exhaustive search.

The authors of [32] addressed resource allocation in a distributed antenna system by formulating a problem to optimize energy efficiency, considering transmission rate and power transmission with a single power constraint. They aimed to maximize energy efficiency through optimal power transmission. The problem was initially solved using a sub-gradient iterative method. With sufficient dataset availability, they applied KNN to find the nearest neighbor for power distribution. The study assumed perfect CSI from both transmitter and receiver, which limited its scope. The authors showed that they could achieve the same spectral and energy efficiency by using KNN as a sub-gradient method with the added advantage of KNN’s low runtime complexity.

The authors of [31] proposed a hybrid DRL-based framework for dynamic resource allocation and trajectory planning in a UAV-assisted wireless network to support various services with different QoS re- quirements. They formulated the problem as a maximization of energy efficiency with constraints related to link and delay, resulting in a non-convex and NP-hard problem. Their proposed model jointly opti- mized energy efficiency and minimized service delay by determining optimal transmit power, sub-carrier assignment, UAV locations, service function chain paths, function placement, and scheduling. They used both single-agent and multi-agent DRL-based algorithms to solve the problem. The simulation results demonstrated that their migration- enabled scheme outperformed state-of-the-art schemes and met QoS requirements for high-mobility users, particularly in large networks. Additionally, they developed a distributed, low-overhead, and low- complexity multi-agent scheme for large-scale UAV-assisted networks. The proposed algorithm achieved significant improvements over the standard DDPG algorithm. This work can be enhanced by including comparisons with traditional optimization algorithm baselines.

The authors in [127], the authors addressed resource allocation in a UAV-aided device to device (D2D) wireless environment with the goal of improving system energy efficiency. They formulated an opti- mization problem to maximize energy efficiency, subject to constraints on bandwidth, device association, subchannel allocation, transmission power, and data rate. The authors modeled the problem was modeled as a MDP, and they proposed a DDPG-based algorithm was proposed to determine the optimal power allocation strategy. This They further enhanced the approach was further enhanced using a LSTM-based model. Their evaluation focuses focused on comparisons with other DRL methods; however, the inclusion of a traditional optimization algorithm (e.g., B&B) as a baseline would could further strengthen the benchmarking of their results.

    - Objective function trade-offs

Balancing power consumption and system performance (in terms of latency, bandwidth) is crucial in modern communication systems, particularly in wireless edge computing, mobile edge networks, and UAV-assisted FL environments. Several approaches have been pro- posed, including gradient-based methods, DRL, and hybrid algorithms, to address tradeoffs between energy efficiency and performance metrics as discussed below.

The authors of [33] proposed a multiple-algorithm service model for completing tasks, optimizing workload assignment weight and compu- tation capacities using a gradient descent-based algorithm in a wireless edge computing network. They formulated the problem to minimize energy and task offloading delay, with QoS, compute resource uti- lization (task scheduling on CPU), and task-specific (workload as- signment weights) constraints. To solve the formulated problem, they first used a gradient descent-based algorithm to find optimal work- load assignment weights. Then, they proposed a secondary algorithm to generate a complete solution using the initial algorithm’s results. The authors compared their work with different configurations of it- self only. This work can be enhanced by including comparisons with traditional optimization algorithm baselines.

The authors of [34] addressed caching and computation offloading by users to mobile edge servers, aiming to minimize energy consump- tion and latency. They formulated the problem of minimizing the weighted sum of energy consumed and execution time with constraints related to association, power and latency, resulting in a long-term integer multi-timescale linear programming problem, which is NP-hard. To address this, they proposed a DRL-based cache-aided offloading approach. Their results showed that this scheme outperformed no-cache and random-cache incorporated offloading schemes. The authors com- pared their work with different configurations of a random, no-cache and DRL based approaches. Their work can be enhanced by including comparisons with traditional optimization algorithm baselines.

The authors of [35] focused on resource optimization in ultra-dense networks, formulating a problem to maximize the difference between energy efficiency and delay probability with constraints related to QoS, link and transmit power. They proposed a K-means clustering- based algorithm to cluster femto-base stations based on traffic load and geographic location. Following this, they introduced a DRL-based algo- rithm to assign resource blocks among femto-base stations, optimizing energy efficiency while maintaining QoS guarantees. Their approach outperformed greedy, standard DRL, and Stackelberg game-based meth- ods in performance comparisons. Rigorous baseline comparisons were utilized to evaluate the work’s effectiveness.

The authors of [110] studied an edge FL environment featuring a UAV swarm with limited computational capacities and ground-based edge servers responsible for model aggregation. They formulated an objective function to minimize the tradeoff between energy consump- tion and system delay, with constraints related to link and computation resource allocation (i.e., CPU frequency). They proposed a DRL-based algorithm to learn UAV association and resource allocation strategies

while simultaneously minimizing energy consumption and system de- lay. Their simulations involved thirty UAVs and three edge servers. The authors showed that their proposed machine learning based scheme performs significantly better than a traditional optimization algorithm (e.g., B&B) baseline.

Lessons learned: This section presents research results related to energy minimization. We observed some common issues during our literature review. In [28,32], they considered power optimization but assumed perfect CSI. Adding a good benchmark algorithm could improve the understandability and interpretability of their results. Sim- ilarly, the quality of works such as [34,36], and [31,33,110] could be enhanced by including comparisons with traditional optimization algo- rithm baselines. In [27], the authors mentioned B&B but did not utilize it as a baseline benchmark algorithm. On the other hand, rigorous baseline comparisons were utilized in [35,106] to evaluate the work’s effectiveness. In [111], the algorithm was evaluated with fixed of- floading and trajectory, considering only one UAV and four unmanned surface vehicles in the simulation environment. In [29], DNN were used with initialization with random values, which could lead to poor decision-making at the start of the process. In [30], the prediction error concerning epochs could be improved. In this section, we reviewed works that formulated their energy optimization objective and solved it using primarily machine learning. Table 4 summarizes all the works discussed in the above section. In the next section, we discuss papers with time-based objective functions (see Table 5).

  - Latency minimization

Latency and delay are essential time metrics to evaluate the per- formance of mobile edge computing (MEC) systems. Delay is the time between the task requested (by an IoT device) and the response re- ceived. It can be communication as well as computation delay. We reviewed works that used time-based factors as the objective function in this section. In some works, delay (as a whole) was preferred as an objective function [63,64,68,103]. Others [65,66,71,74] considered delay in conjunction with other variables to be their objective function. Delay was also utilized in slight variations by some authors, such as latency [65,66] and response time [69,70] among others [72,73].

    - Pure objective function

Optimizing specific performance objectives such as task delay, re- source allocation, and end-to-end latency remains critical for enhancing overall system efficiency. Pure objective function approaches focus on minimizing a single key performance metric under various communi- cation and computation constraints. Recent studies [63,64,68] have leveraged DRL-based methods to achieve near-optimal solutions in real-time, addressing limitations such as incomplete state information and dynamic environments. These approaches highlight the potential of advanced learning algorithms in minimizing delays and improving resource utilization.

In [64], the authors focused on minimizing task delay for offloaded tasks from IoT devices to nearby fog nodes. They aimed to minimize total delay (transmission and computation delay) under delay, QoS, and a compute resource utilization (CPU frequency) constraints. While their problem ideally required complete state information, this was not feasible in their scenario, leading them to opt for an DRL-based system using partial state information. They proposed an DRL-based online resource allocation algorithm, operating at the fog node to generate near-optimal solutions in real time. Their proposed algorithm allocated radio and computation resources upon task arrival and up- dated resource information when the task departed. They aimed to enhance system accuracy and efficiency by utilizing the actor-critic architecture in DRL. The authors compared their work with variations of itself (i.e., computation only and transmission only). This work can be enhanced by including comparisons with traditional optimization algorithm (e.g., B&B) baselines.

Summary of energy-based papers for machine learning based comprehensive solutions

B&B: branch and bound; BS: base station; CSI: channel state information; DNN: deep neural network; DRL: deep reinforcement learning; FDMA: frequency division multiple access; MIP: mixed integer programming; MINLP: mixed-integer nonlinear programming; NP: nondeterministic polynomial time; QoS: quality of service; UAV: unmanned aerial vehicle.

| Reference | Objectives | Constraint type | Problem type | Classic approach | Remarks |
| --- | --- | --- | --- | --- | --- |
| [27] | Minimize system power | Queue stability, data rate, | MIP | Lyapunov optimization | Incorporating robustness in model drift |
|  |  | subchannel allocation, |  | and B&B | and generalization algorithms into their |
|  |  | transmission power, compute |  |  | system can improve system performance. |
|  |  | resource utilization |  |  |  |
| [111] | Minimize system energy | Compute resource utilization | NP-hard | – | They only utilize variations (fixed |
|  |  | (CPU frequency), link, |  |  | allocation and FDMA scheme) to |
|  |  | transmission power, delay, |  |  | compare with their work. An |
|  |  | task-specific (UAVs velocity) |  |  | optimization algorithm baseline can be |
|  |  |  |  |  | added. |
| [29] | Minimize power | Connection, transmission | MINLP | Projected sub-gradient | Start by using DNN to make offloading |
|  | allocation and | power |  |  | decisions with no training; therefore, the |
|  | computation offloading |  |  |  | system will make many erroneous |
|  |  |  |  |  | readings before enough it gets enough |
|  |  |  |  |  | data to train and improve. |
| [30] | Minimize power | Transmission power, distance | Convex | – | Simulation size can be improved. |
|  | allocation and relay |  |  |  |  |
|  | location |  |  |  |  |
| [28] | Maximize transmission | System power | – | Lagrange and bisection | Instantaneous perfect CSI (from both |
|  | power |  |  | search | transmitter and receiver) is considered |
|  |  |  |  |  | in their work (which is impractical in |
|  |  |  |  |  | real-world scenarios). |
| [106] | Maximize energy | Transmission power, QoS, | – | – | The results are compared with a genetic |
|  | efficiency | delay |  |  | algorithm and exhaustive search (along |
|  |  |  |  |  | with their own algorithm variations of |
|  |  |  |  |  | DRL). |
| [32] | Maximize energy | System power | – | – | Instantaneous perfect CSI (from both |
|  | efficiency |  |  |  | transmitter and receiver) is considered |
|  |  |  |  |  | in their work (which is impractical in |
|  |  |  |  |  | real-world scenarios). |
| [31] | Maximize energy | Delay, link | – | – | The authors compared results with DRL |
|  | efficiency |  |  |  | based variations. To enhance the study, |
|  |  |  |  |  | an optimization benchmark algorithm |
|  |  |  |  |  | (e.g., B&B) could be incorporated. This |
|  |  |  |  |  | would provide a standard reference |
|  |  |  |  |  | point for evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |
| [127] | Maximize energy | Bandwidth, device association, | non-convex | – | They only utilize DRL variations to |
|  | efficiency | subchannel allocation, |  |  | compare with their work. An |
|  |  | transmission power, and data |  |  | optimization algorithm baseline, e.g., |
|  |  | rate, connection |  |  | B&B, can be added. |
| [33] | Minimize energy and | QoS, compute resource | – | – | They do not provide any comparisons |
|  | task offloading delay | utilization (task scheduling on |  |  | with other standard algorithms with |
|  |  | CPU), task-specific (workload |  |  | their proposed approach. |
|  |  | assignment weights) |  |  |  |
| [34] | Minimize energy and | Task-specific (offloading | – | – | They do not provide any comparisons |
|  | latency | decision), compute resource |  |  | with other standard algorithms with |
|  |  | utilization (software caching |  |  | their proposed approach. |
|  |  | limits) |  |  |  |
| [35] | Minimize energy and | QoS, transmit power, link | – | – | They consider perfect CSI and a small |
|  | delay probability |  |  |  | simulation size (four femto-BSs only). |
| [110] | Minimize energy and | Compute resource allocation | NP-hard | – | They only utilize DRL variations to |
|  | system delay | (CPU frequency), link |  |  | compare with their work. An |
|  |  |  |  |  | optimization algorithm baseline can be |
|  |  |  |  |  | added. |

In [63], the authors addressed the task, spectrum, and transmit power allocation problem, formulating it as a MINLP problem. They aimed to minimize the maximum computation and transmission delay among all users by optimizing parameters such as uplink and downlink sub-carrier allocation, transmission power, and task-allocation indica- tor. They considered subchannel, transmission power, link, and com- pute resource utilization (task collaboration in BS and UEs) constraints in their optimization problem. To solve this problem, they proposed a DRL-based algorithm, where the available action space served as input and the optimal resource allocation policy as output. The authors can improve their work by comparing it with traditional optimization

algorithms (e.g., B&B). The authors compared their work with the stock Q learning algorithm only.

In [68], the authors analyzed the M/D/1 queueing model to com- pute end-to-end delay, aiming to minimize average packet delay while considering constraints related to compute resource utilization (CPU resource allocation) and communication delay. They proposed a delay- sensitive actor-critic algorithm to address this problem, which they further enhanced by integrating an echo state network to improve performance and accuracy. The authors compared their work with variations of DRL algorithms (i.e., DQN and actor-critic) and random policy. This work can be enhanced by including comparisons with traditional optimization algorithm baselines.

Table 5

Summary of time-based papers for machine learning based comprehensive solutions

B&B: branch and bound; BS: base station; DRL: deep reinforcement learning; INLP: integer non-linear programming problem; MEC: mobile edge computing; MINLP: mixed-integer nonlinear programming; PEN: patient edge network; QoS: quality of service; RAN: radio access network; UAV: unmanned aerial vehicle; UE: user equipment.

| Reference | Objectives | Constraint type | Problem type | Classic approach | Remarks |
| --- | --- | --- | --- | --- | --- |
| [64] | Minimize total delay | Delay, QoS, compute resource | INLP | – | The authors can improve their work by |
|  |  | utilization (CPU frequency) |  |  | incorporating a comparison with |
|  |  |  |  |  | traditional optimization algorithms. |
| [63] | Minimize max computation | Subchannel, transmission power, | MINLP | – | The authors can improve their work by |
|  | and computational delay | link, compute resource utilization |  |  | incorporating a comparison with |
|  |  | (task collaboration in BS an UEs) |  |  | traditional optimization algorithms. |
| [68] | Minimize average packet | Delay, compute resource | – | – | The authors can improve their work by |
|  | delay | utilization (CPU resource fraction |  |  | comparing it with traditional |
|  |  | allocation) |  |  | optimization algorithms. |
| [74] | Minimize average delay to | Link, delay, compute resource | – | – | Appropriate baseline comparisons are |
|  | throughput ratio | utilization (cache resource |  |  | done to show the effectiveness of |
|  |  | bounds, CPU resource allocation, |  |  | proposed approach. |
|  |  | cache miss rate bounds) |  |  |  |
| [70] | Minimize response time | Compute resource utilization | – | – | Simulation size can be increased from |
|  | and energy consumption | (CPU, memory utilization) |  |  | ten MEC servers, with twenty UE, to |
|  |  |  |  |  | improve the generalizability of this |
|  |  |  |  |  | work. |
| [69] | Minimize response time | QoS | – | – | A complete baseline based on traditional |
|  | and energy consumption |  |  |  | optimization algorithms (e.g., B&B) can |
|  |  |  |  |  | be added to enhance this work. |
| [72] | Minimize task duration | Compute resource utilization | – | – | A complete baseline based on traditional |
|  |  | (battery capacity), transmission |  |  | optimization algorithms can be added to |
|  |  | power |  |  | enhance this work. |
| [71] | Minimize time delay and | Delay, system power | – | – | Compared the results with their own |
|  | energy consumption |  |  |  | algorithm variations. An optimization |
|  |  |  |  |  | algorithm benchmark can be added. |
| [66] | Minimize latency and | Link, compute resource utilization | Non-convex | – | A complete baseline based on traditional |
|  | energy consumed | (strategy selection) |  |  | optimization algorithms can be added to |
|  |  |  |  |  | enhance this work. |
| [65] | Minimize latency, energy | Link, bandwidth, task-specific | Convex | – | Simulations were conducted using three |
|  | consumed and cost | (traffic path constraint), compute |  |  | PEN (i.e, UEs) and three RANs. |
|  |  | resource utilization (time |  |  | However, the relatively small simulation |
|  |  | allocated at RAN for task |  |  | size may limit the generalizability of |
|  |  | execution) |  |  | their findings. |
| [128] | Minimize maximum task | Compute resource utilization | MINLP | – | Small simulation size (they simulate |
|  | latency | (CPU utilization at edge and |  |  | with four BSs and sixteen users only). |
|  |  | cloud), link , system power, |  |  |  |
|  |  | latency |  |  |  |
| [129] | Minimize time delay | System power, distance, link, | – | – | The authors compared results with |
|  |  | latency, and compute resource |  |  | variations of their own algorithm. An |
|  |  | utilization (CPU utilization) |  |  | optimization algorithm benchmark can |
|  |  |  |  |  | be added. |
| [73] | Minimize average | Subchannel, transmission power, | Concave | – | They provided comparisons with |
|  | information age | distance, task-specific (phase shift, |  |  | variations of their own proposed |
|  |  | amplitude of reflecting elements, |  |  | solution and random approach only. |
|  |  | UAVsspeed and coverage area) |  |  |  |
| [115] | Minimize maximum | Compute resource utilization | MINLP | – | A thorough analysis is done to show the |
|  | latency | (CPU utilization, task accuracy, |  |  | proposed scheme’s superiority over |
|  |  | and data size), queue, link |  |  | baseline algorithms (its own variations |
|  |  |  |  |  | and exhaustive search method). |
| [113] | Minimize delay | Compute resource utilization | – | – | More comprehensive information on |
|  |  | (CPU utilization at small BS, large |  |  | simulation parameters can enhance the |
|  |  | BS and cloud caching), link, data |  |  | generalizability of their results. |
|  |  | rate, delay, and QoS. |  |  |  |
| [130] | Minimize delay while | Link, QoS | – | – | The authors compared their method only |
|  | maximizing bandwidth |  |  |  | with DRL-based variants, including |
|  | utilization |  |  |  | DRLUA [131] and their own intelligent |
|  |  |  |  |  | low-complexity QoS-aware approaches. |
|  |  |  |  |  | Including a traditional optimization |
|  |  |  |  |  | baseline (e.g., B&B) could have |
|  |  |  |  |  | improved the evaluation and enhanced |
|  |  |  |  |  | the applicability of their work. |

In [74], network slicing emerged as a pivotal technique for enhanc- ing flexibility in 5G networks. The paper introduced two deep transfer reinforcement learning (DTRL) based solutions, namely Q-value-based

deep transfer reinforcement learning and action selection-based DTRL, for inter-slice resource allocation. They formulated an objective func- tion to minimize system delay while optimizing throughput. They

considered the constraints of link, delay, and compute resource utiliza- tion (i.e., cache resource bounds, CPU resource allocation, cache miss rate bounds) on their formulated problem. A global resource manager jointly addressed both objectives and allocated resources accordingly. Simulation results compared the proposed algorithms with two base- line algorithms, demonstrating better network performance and faster convergence speeds with the proposed methods. Rigorous baseline comparisons were established to evaluate the work’s effectiveness.

In summary, pure objective function approaches targeting delay op- timization in MEC systems demonstrate the effectiveness of DRL-based methods for real-time resource allocation and performance enhance- ment. While these studies successfully address key time metrics under various constraints, their quality and impact can be further improved through comprehensive comparisons with traditional optimization al- gorithms (for [63,68]), enabling a more holistic evaluation of their efficiency and scalability.

    - Objective function variations

In MEC systems, performance metrics like delay and energy opti- mization are critical for ensuring system efficiency and reliability. Sev- eral works [69,70,72] have explored additional metrics such as average task response time, system energy consumption, and task duration. These studies aim to achieve balanced resource allocation and task- offloading strategies while addressing communication and computation constraints in dynamic environments. This subsection reviews contribu- tions where optimization goals are multifaceted, leveraging advanced techniques such as DRL, asynchronous advantage actor-critic networks, and drift-adaptive learning to address these complex objectives.

In [70], the authors addressed resource allocation and task offload- ing, accounting for reliability in an MEC environment. Considering computation resource utilization (i.e., CPU and memory) constraints, they aimed to minimize average task response time and system energy consumption. Modeling it as a MDP problem, they proposed a self- learning task offloading and resource allocation algorithm leveraging DRL. The algorithm selected the computing node using DRL, deciding whether to process tasks locally or globally based on the node selection. Increasing the simulation size (from ten UEs and ten BSs) can enhance generalizability of this work.

In [69], the authors focused on resource allocation and scheduling in a wireless IoT environment. They formulated their problem as min- imizing response time and energy consumption, with one constraint ensuring the response time remains within the service level agreement (QoS constraint). They utilized DRL to solve this problem of resource al- location and resource scheduling. As the demand for resources change, their service scenarios also change. They employed drift-adaptive learn- ing in their proposed algorithm to handle dynamic changes in user demands. The authors compared their work with machine learning algorithms only. This work can be enhanced by including comparisons with traditional optimization algorithm baselines and incorporating fractional resource allocation.

In [72], the authors focused on the task offloading problem in ultra- dense networks. Their objective was to minimize task duration while considering compute resource utilization (battery capacity) and trans- mission power constraints. They formulated the problem as minimizing task duration with power and connection type constraints. To solve this problem, the authors proposed an asynchronous advantage actor- critic based DRL algorithm. They also briefly covered the proposed algorithm’s convergence in their work. The authors compared their work with variations of itself (i.e., delay optimal and energy optimal only). This work can be enhanced by including comparisons with traditional optimization algorithm baselines.

In summary, the works reviewed in this section highlight the po- tential of DRL-based solutions for optimizing diverse metrics such as task response time, energy consumption, and task duration in MEC systems. While these approaches demonstrate promising results, the lack of comparisons with traditional optimization algorithm baselines

limits their evaluation depth. Enhancing these works through broader benchmarking and exploring additional strategies, such as fractional resource allocation and larger simulation environments, can further validate their effectiveness and scalability.

    - Objective function tradeoffs

Tradeoff analysis plays a crucial role in MEC systems, where op- timizing multiple conflicting objectives, such as latency, energy con- sumption, and cost, is vital for system efficiency. Researchers often solve these challenges by formulating multi-objective optimization problems that consider communication, computation, and power con- straints. Advanced reinforcement learning approaches, including DRL and hybrid algorithms, have been widely adopted to achieve near- optimal solutions in these complex scenarios. This section reviews recent works that focus on addressing these tradeoffs, showcasing novel algorithms designed to balance performance metrics while handling the inherent complexities of MEC environments.

In [71], the authors addressed task assignment and scheduling in an energy-constrained network, focusing on a software-defined fog-IoT environment optimized for low latency and energy efficiency. Their objective was to minimize time delay and enhance energy efficiency in task assignments formulated with constraints related to power and delay. They proposed an algorithm based on deep Q-learning (DQL) to solve the problem and variations based on traditional DQL. Deploying the algorithm at the SDN controller enabled runtime action selection in cases where the agent lacked complete knowledge of rewards and state transitions. They experimented with different policies, including ran- dom learning, deterministic learning, and async actor-critic algorithm and concluded that their algorithm outperformed other default policies. The authors compared their work with variations of itself (i.e., delay optimal and energy optimal only). The authors could improve gener- alizability of their work by comparing it with traditional optimization algorithms.

In [66], the authors investigated resource allocation and optimiza- tion strategies for MEC networks. They formulated their problem to minimize the expected sum of latency and energy consumption, con- sidering constraints related to link and compute resource utilization (strategic profile selection). The problem was non-convex. Their work presented three solutions: the first utilized pure DQL for strategic profiles and resource allocation, the second employed DQL solely for strategic profile assignment and slack variable-based resource allo- cation, and the third proposed Q-Learning (QL) for strategic profile assignment and slack variable-based algorithm for resource assignment, without DQL. Through simulation, they determined that their second approach (DQL with slack-based resource allocation) performed best for large networks. In contrast, the third approach (QL plus slack variable- based algorithm) was optimal for small networks. The authors showed that their hybrid algorithm outperforms both traditional optimization and machine learning algorithms, thus highlighting the case of hybrid algorithms.

In [65], the authors addressed resource allocation in a multiple- radio-access-technologies network, proposing a team-based DRL algo- rithm. They focused on transferring data from multiple edge nodes to the cloud while ensuring QoS, latency, monetary value, and computa- tion constraints. Formulating their problem as a minimization task, they aimed to minimize latency, energy consumption, cost, and distortion in their objective function, with constraints related to link, bandwidth, task-specific (traffic path constraint), and compute resource utilization (time allocated at RAN for task execution). The formulated problem was multi-objective and non-convex. Their proposed solution was a team- based multi-agent DDPG, deploying patient edge network (PEN) and RAN agents. PEN agents, situated at PENs, minimize energy consump- tion, cost, latency, and distortion independently, while RAN agents, located at the RAN controller, optimize users’ bandwidth and QoE. The algorithm extended the DDPG to a multi-agent framework, with the critic network accessing joint observations and the actor network

utilizing local information. The training was centralized, employing a distributed training framework. Both RAN and PEN networks con- sidered each other as part of the environment. They evaluated their architecture with various configurations through simulations, com- paring the results. The authors provide a thorough comparison with appropriate benchmark algorithms. Simulations were conducted us- ing three PENs (i.e, UEs) and three RANs only which does limit the generalizability of their findings.

In [128], the authors targeted a MEC environment characterized by user mobility and variable connectivity. They structured the en- vironment into users, edge, and cloud layers to minimize maximum task latency through optimization with constraints including latency, system power, and link compute resource utilization (CPU utilization at the edge and cloud). The problem was formulated as a MINLP. The authors converted this problem into an MDP and employed DRL and NoisyNet [132] to obtain near-optimal solutions. Additionally, they devised a ConvLSTM-based mobility prediction algorithm to forecast mobile device movements, enhancing their approach’s performance. Their work was evaluated against two variations and an optimization heuristic algorithm baseline to demonstrate its efficacy and performed better than given benchmarks.

In the paper [129], the authors focused on a UAV-assisted computa- tion offloading architecture operating in the THz band. They addressed resource allocation, UAV placement, and computation offloading simul- taneously, aiming to optimize time delay under constraints related to system power, distance, link, latency, and compute resource utiliza- tion (CPU utilization). To solve this problem, the authors proposed algorithms based on DRL and DDPG. They conducted comparisons with complete local and complete offloading approaches to evaluate the results. The authors compared results with their algorithm varia- tions. An optimization algorithm benchmark can be added to improve generalizability.

In [73], a status update framework for IoT users in an aerial IRS communication system aimed to enhance data freshness by optimizing various parameters. The objective was to minimize the average age of information, considering ten constraints such as maximum transmit power limitations, power domain glspl noma-related restrictions, and task-specific constraints (UAVs movement, coverage area, and phase shift and amplitude of reflecting entities). It featured discrete and continuous variables, divided into two sub-problems. DRL-based algo- rithms, including double deep Q network (DDQN) and proximal policy optimization (PPO), were employed to solve them. Numerical analysis compared their performance with baseline algorithms, demonstrating the effectiveness of the proposed DDQN and PPO algorithms, particu- larly when combined with a matching algorithm and random trajectory. They only provide comparisons with variations of their proposed solu- tion and random approach (without utilizing any traditional algorithm baseline).

In [115], the authors devised a resource management scheme for edge-assisted machine learning task inference to minimize total task processing delay. They formulated the problem with constraints related to compute resource utilization (CPU utilization, task accuracy, and data size), link, and task queues. The problem was addressed as a MINLP. Initially, they utilized Lyapunov optimization to decompose the problem into three sub-problems: task adjustment, computing re- source allocation, and channel allocation with task offloading. Unique algorithms were proposed for each sub-problem, employing Lagrange and dual problem approaches. An iterative algorithm was introduced to solve these problems iteratively, utilizing the separately proposed algorithms for each sub-problem. Their approach was compared with four variations: (i) fixed task data; (ii) fixed task data, equal computing resource allocation, and random channel allocation; (iii) fixed task data and no extra delay caused by error inference; and (iv) local task processing to assess its effectiveness. A thorough analysis is done to show the proposed scheme’s superiority over baseline algorithms (its own variations and exhaustive search method).

In [113], the authors tackled content distribution in fog RANs, aiming to optimize resource allocation to minimize delay in handling content requests from user equipment. They aimed to minimize total network delay, considering constraints related to compute resource utilization (CPU utilization at small BS, large BS and cloud caching), link, data rate, delay, and QoS. They introduced a DRL-based algo- rithm to enable cooperative caching and routing for content requests, utilizing content history and resource availability. They illustrated their proposed algorithm’s superior performance through comparisons with distributed and cooperative schemes. More comprehensive information on simulation parameters can enhance the generalizability of their results.

The authors in [130] worked within an open radio access net- work (ORAN) environment to enhance network resource distribution. Their objective was to maximize bandwidth utilization while mini- mizing transmission delay. They formulated the problem with QoS and link-related constraints to ensure feasibility. To solve it, they proposed a DRL-based algorithm aimed at making optimal decisions for sub-channel allocation and MEC association among UEs. The au- thors evaluated their approach by comparing it exclusively with other DRL-based variations, including DRLUA [131] and their own variants— namely, the intelligent low-complexity QoS-aware resource allocation and its simplified version. However, the lack of comparison with a traditional optimization baseline limits the assessment of the general applicability and performance advantages of their proposed method.

The reviewed studies underscore the importance of balancing com- peting objectives such as latency, energy consumption, cost, and QoS in MEC systems. The authors demonstrated significant improvements in system performance by employing advanced techniques like DRL, multi-agent systems, and mobility prediction. However, while some works benchmarked their approaches against traditional optimization algorithms, others relied primarily on variations of their models, leav- ing room for improvement in evaluation depth. Future research should explore more robust benchmarking practices and hybrid approaches to better address the intricate tradeoffs in MEC systems.

Lessons learned: The past solutions discussed in this section highlight several key lessons in the context of time-based objectives for resource optimization. Simulation size significantly impacts the utility of the proposed work, as in [70,103], which used small simulation settings (six and twenty users, respectively). Proper baselines with optimization algorithms are crucial for effective evaluation, as demon- strated by [66], whereas [63,68,71], and [64] could have benefited from adding such baselines. Thorough comparisons with appropriate benchmark algorithms, as done by [65,115], are essential for demon- strating the efficacy of proposed schemes. Several studies, such as [72, 73], would benefit from incorporating traditional optimization algo- rithm baselines to strengthen their evaluations. Additionally, more comprehensive information on simulation parameters [113] and frac- tional resource allocation [69] can enhance the generalizability of the results. This review of recent literature on time-based objectives for resource optimization underscores the importance of simulation size, proper baselines, and thorough comparisons with traditional optimiza- tion algorithms (e.g., B&B), as summarized in Table 5. In the next section, we explore papers with data rate-based objectives for resource optimization.

  - Sum rate maximization

The sum rate represents a network’s total achievable data rate, serving as a key metric for maximizing data transmission efficiency. It reflects combined data rates of all network entities, making it a critical parameter for optimizing network performance and ensuring effective resource allocation. Several authors [38,41,43,44,52] have preferred it as their focus in objective functions. Others [39,46,138] have preferred its variations like data rate or average receiving rate. Throughput has also been considered a good metric for the objective function by quite a few [47,50,53,86] as well as SINR [40,42,51]. In this subsection, we cover them in more detail.

Table 6

Summary of data rate-based publications for machine learning based comprehensive solutions

B&B: branch and bound; BS: base station; DDPG: deep deterministic policy gradient; DRL: deep reinforcement learning; MEC: mobile edge computing; MINLP: mixed-integer nonlinear programming; NP: nondeterministic polynomial time; QoS: quality of service; SINR: signal-to-interference-plus-noise ratio; SIR: signal-to-interference ratio; UAV: unmanned aerial vehicle; UE: user equipment.

| Reference | Objectives | Constraint type | Problem type | Classic approach | Remarks |
| --- | --- | --- | --- | --- | --- |
| [43] | Maximize sum rate | Subchannel, link | Integer programming | – | The study utilized a relatively small |
|  |  |  |  |  | simulation size, which may impact the |
|  |  |  |  |  | generalizability of the results. |
| [44] | Maximize sum rate | Distance, system power, QoS | Non-convex NP-hard | – | The authors have effectively provided |
|  |  |  |  |  | appropriate comparisons with relevant |
|  |  |  |  |  | benchmarks in their study, enhancing |
|  |  |  |  |  | the generalizability and contextual |
|  |  |  |  |  | relevance of their findings. |
| [52] | Maximize sum rate | System power | – | – | To enhance the study, an optimization |
|  |  |  |  |  | benchmark algorithm (e.g., B&B) could |
|  |  |  |  |  | be incorporated. This would provide a |
|  |  |  |  |  | standard reference point for evaluating |
|  |  |  |  |  | the performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |
| [41] | Maximize sum rate | Link, distance, power, subchannel | Non-convex | – | The results were derived using only a |
|  |  |  |  |  | single simulation configuration. |
| [38] | Maximize downlink | System power | Non-convex | – | To enhance the study, an optimization |
|  | sum rate |  |  |  | benchmark algorithm (e.g., B&B) could |
|  |  |  |  |  | be incorporated. This would provide a |
|  |  |  |  |  | standard reference point for evaluating |
|  |  |  |  |  | the performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |
| [51] | Maximize Sum rate | Transmission power, connection | – | – | The authors compared results with |
|  |  |  |  |  | variations of their own solution. To |
|  |  |  |  |  | enhance the study, an optimization |
|  |  |  |  |  | benchmark algorithm (e.g., B&B) could |
|  |  |  |  |  | be incorporated. This would provide a |
|  |  |  |  |  | standard reference point for evaluating |
|  |  |  |  |  | the performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |
| [133] | Maximize data rate and | Task-specific (error probability), | MINLP | – | They simulate their network with eight |
|  |  | transmission power, and |  |  | vehicle-to-network and eight |
|  |  | subchannel |  |  | vehicle-to-vehicle (V2V) pairs, which can |
|  |  |  |  |  | impact the generalizability of their |
|  |  |  |  |  | proposed work. |
| [134] | Maximize sum rate | Queue stability, subchannel, QoS, | – | – | The authors compared results with |
|  |  | and transmission power |  |  | variations of their own work. To |
|  |  |  |  |  | enhance the study, an optimization |
|  |  |  |  |  | benchmark algorithm (e.g., B&B) could |
|  |  |  |  |  | be incorporated. This would provide a |
|  |  |  |  |  | standard reference point for evaluating |
|  |  |  |  |  | the performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |
| [47] | Maximize throughput | Data rate, link | MINLP | – | To enhance the study, an optimization |
|  |  |  |  |  | benchmark algorithm (e.g., B&B) could |
|  |  |  |  |  | be incorporated. This would provide a |
|  |  |  |  |  | standard reference point for evaluating |
|  |  |  |  |  | the performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |
| [53] | Maximize throughput | Data rate, QoS | – | – | The study utilized a relatively small |
|  |  |  |  |  | simulation size (six UEs, four resource |
|  |  |  |  |  | blocks only), which may impact the |
|  |  |  |  |  | generalizability of the results. |
| [86] | Maximize throughput | Data rate, transmission power, | – | – | The authors compared the results with |
|  |  | link, QoS |  |  | variations of their own work. To |
|  |  |  |  |  | enhance the study, an optimization |
|  |  |  |  |  | benchmark algorithm (e.g., B&B) could |
|  |  |  |  |  | be incorporated. This would provide a |
|  |  |  |  |  | standard reference point for evaluating |
|  |  |  |  |  | the performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |
| [135] | Maximize throughput | Transmission power, link, distance | – | – | The study utilized a relatively small |
|  |  |  |  |  | simulation size (two UAVs and seven |
|  |  |  |  |  | users only), which may impact the |
|  |  |  |  |  | generalizability of the results. |

(continued on next page)

Table 6 (continued).

| Reference | Objectives | Constraint type | Problem type | Classic approach | Remarks |
| --- | --- | --- | --- | --- | --- |
| [50] | Maximize | Link, QoS, compute resource | – | – | The study utilized a relatively small |
|  | utility-throughput | utilization (CPU utilization) |  |  | simulation size, which may impact the |
|  |  |  |  |  | generalizability of the results. |
| [40] | Maximize SIR at all | Task-specific (SIR), other | – | – | The study utilized a relatively small |
|  | time slots |  |  |  | simulation size, which may impact the |
|  |  |  |  |  | generalizability of the results. |
| [136] | Maximize achievable | Link, QoS | NP-hard | – | The study utilized a relatively small |
|  | data rate for all users |  |  |  | simulation size (thirty UEs and multiple |
|  |  |  |  |  | MECs), which may impact the |
|  |  |  |  |  | generalizability of the results. |
| [39] | Maximize minimum | Data rate, subchannel, | MINLP | – | Data imbalance in the dataset can be |
|  | data rate | transmission power, QoS |  |  | worked upon to enhance the results. |
| [137] | Maximize packets | QoS, link | – | – | They compared their results with |
|  | delivered successfully |  |  |  | variants of their work and standard |
|  |  |  |  |  | DDPG-based DRL algorithm. They can |
|  |  |  |  |  | enhance the generalizability of their |
|  |  |  |  |  | work by incorporating a traditional |
|  |  |  |  |  | optimization baseline (e.g. B&B) in their |
|  |  |  |  |  | simulations. |
| [138] | Maximize data rate and | Data rate, transmission power, | – | – | The authors could provide more |
|  | fairness index | and distance |  |  | comprehensive information about the |
|  |  |  |  |  | simulation setup to enhance the |
|  |  |  |  |  | generalizability of the work. |
| [46] | Maximize average | Data rate, compute resource | – | – | The study utilized a relatively small |
|  | receiving rate on the | utilization (CPU bounds) |  |  | simulation size (ten UEs, ten edge |
|  | cloud |  |  |  | nodes, and ten clouds), which may |
|  |  |  |  |  | impact the generalizability of the results. |
| [42] | Maximize SINR | Subchannel | Non-convex | – | The authors compared results with |
|  |  |  |  |  | variations of their own work. To |
|  |  |  |  |  | enhance the study, an optimization |
|  |  |  |  |  | benchmark algorithm (e.g., B&B) could |
|  |  |  |  |  | be incorporated. This would provide a |
|  |  |  |  |  | standard reference point for evaluating |
|  |  |  |  |  | the performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |
| [139] | Maximize sum rate | Link, transmission power, | – | – | The study utilized a relatively small |
|  |  | subchannel, QoS constraint |  |  | simulation size (one BS and ten users |
|  |  |  |  |  | only), which may impact the |
|  |  |  |  |  | generalizability of the results. |

    - Pure objective function

Maximizing the sum rate is a critical objective in future wire- less communication networks because it directly impacts the system’s overall throughput and efficiency. Several researchers have employed a variety of approaches, from traditional optimization algorithms to advanced machine learning and reinforcement learning methods, to address the complexities of sum rate maximization under diverse con- straints such as power, connectivity, latency, and system capacity. This section reviews recent works on sum rate maximization in vari- ous network scenarios, including UAV-assisted communication systems, machine-to-machine (M2M) networks, and multi-cluster systems with IRS. The solutions leverage state-of-the-art techniques such as neural networks, reinforcement learning, and hybrid algorithms to achieve near-optimal or computationally efficient solutions while exploring tradeoffs between accuracy and runtime complexity.

In [43], the authors aimed to maximize the sum rate and formulated a problem for sum rate maximization, considering subchannel and link constraints in wireless networks. Initially, they proposed a convex optimization-based algorithm using the dual method. However, they explored alternative approaches like convolutional neural networks, gated recurrent unit, and random forest ensemble for enhanced com- putational efficiency, replacing their initial algorithm. Their conclusion emphasized that while the machine learning-based solution offered a sub-optimal solution, it provides a highly efficient alternative, closely approaching the optimal solution. The study utilized a relatively small simulation size, which may impact the generalizability of the results.

The authors showed that they can achieve same subchannel association by using machine learning methods as dual method with the added advantage of low runtime complexity.

In [44], the authors aimed to maximize the sum rate for all ground users by optimizing power allocation and UAV positioning in a UAV- assisted visible light communication environment. They formulated a problem for sum rate maximization, considering constraints related to power, QoS and distance. Due to its NP-hard and non-convex na- ture, they employed the harris hawk optimization algorithm [140], inspired by the hunting behavior of Harris hawks, which operates through exploration and exploitation phases. The authors proposed an algorithm derived from harris hawk optimization to address the optimization problem. Additionally, they utilized DNN to model the hunting behavior, with inputs representing ground user states and outputs determining transmission power and UAV positions, aiming to mitigate the risk of converging to local optimal solutions. The au- thors have effectively provided appropriate comparisons with relevant benchmarks in their study, enhancing their findings’ generalizability and contextual relevance.

In [52], the authors used scalable learning-based methods to address radio resource management challenges in dense wireless networks. They formulated the problem to maximize the sum rate while adhering to a power constraint. They introduced a wireless channel graph convo- lution network within the message passing graph neural networks class to address this radio resource management problem. The wireless chan- nel graph convolution network offered distinct advantages for solving

radio resource management issues. First, it effectively utilized both agent-related and channel-related features. Second, it demonstrated resilience to feature corruption, such as CSI imperfections, allowing for application with partial and imperfect CSI. Additionally, the authors established the equivalence between message passing graph neural networks and a family of distributed optimization algorithms, provid- ing interpretability and theoretical guarantees. Extensive simulations revealed that the proposed wireless channel graph convolution net- work matches or surpassed classic optimization-based algorithms, even without domain knowledge, and achieves significant improvements in sum rate achieved. The authors compared their work with variations of itself. To enhance their work, the authors could incorporate an optimization benchmark algorithm (e.g., B&B) to provide a baseline for evaluating the performance and effectiveness of the proposed methods. In [41], the authors aimed to maximize the sum rate for UAV-served

M2M communication systems. They formulated a problem targeting sum rate maximization while considering constraints such as link, distance, power, and subchannel. These factors correspond to commu- nication constraints related to distance and capacity. To address this problem, they treated it as a Markov game, resembling an DRL problem formulation. Leveraging an unlabeled dataset containing channel gain and some CSI information, they employed a generative adversarial network (GAN) to process the dataset, followed by a dataset extension using GAN. The authors used LSTM to predict the Q-value for the Markov game, instead of DRL. The results were derived using only a sin- gle simulation configuration for this work. The authors compared their work with machine learning algorithms (i.e., GAN, LSTM, DQN) only. This work can be enhanced by including comparisons with traditional optimization algorithm baselines.

In [38], the authors focused on maximizing the downlink sum rate in multi-user multi-cell cellular networks, formulating a non-convex and NP-hard problem. They introduced a customized DRL approach to address this challenge. They adopted trust-region methods [141] rather than the conventional method to compute the policy gradient. Their objective function maximizes the network sum rate, with the sole constraint of ensuring that power allocated to each user remains in the range of zero to the maximum allowed transmit power. For computing the policy network parameters, they employed DRL but developed their custom solution, integrating the trust-region method instead of the stan- dard DRL approach. The authors compared their work with machine learning algorithms only. This work can be enhanced by including comparisons with traditional optimization algorithm baselines.

In [51], the authors focused on a downlink multi-cluster system composed of 𝑁 transmitting antennas at the BS, an IRS with passive elements, and single-antenna mobile users in a IRS-aided multiple-input single-output non orthogonal multiple access (NOMA) network. Their goal was to maximize the sum rate across users, considering data rate, transmission power, and task-specific (decoding order, IRS bounds) constraints. They introduced the IRS-NOMA-aided wireless network, leveraging an IRS to enhance spectrum efficiency. User positions were determined using LSTM, while clustering utilized a K-means-based Gaussian mixture model. They proposed three machine learning based algorithms for predicting user mobility, clustering users, and designing the phase shift matrix. The study emphasized the importance of select- ing the appropriate parameters to enhance the training performance of neural networks. The authors compared results with their variations of their own work. The authors compared their work with machine learning algorithms only. This work can be enhanced by including comparisons with traditional optimization algorithm baselines.

In [133], the authors worked in a cellular vehicle-to-everything net- work environment to maximize the sum rate for both V2V and vehicle- to-network (V2N) communications. They formulated their objective as the maximization of a weighted sum rate of V2V and V2N links, subject to task-specific constraints such as decoding error probability, transmis- sion power limits, and subchannel allocation. The resulting problem was a mixed-integer nonlinear programming (MINLP) problem, which

they addressed using a multi-agent DRL framework augmented with SHAP-based feature selection. Specifically, they first applied SHAP to calculate the importance of each input feature and performed feature selection accordingly. They then trained the DRL model on the reduced feature set to learn optimal resource allocation strategies, specifically, subchannel and power assignment, that maximize the sum rate while reducing training complexity. The authors argued that the complexity and non-convexity of the problem rendered traditional optimization methods impractical. Their approach was compared against random power and subchannel allocation, full power allocation, a centralized DRL method, and their proposed multi-agent DRL strategy. Simula- tions were conducted using eight V2N and eight V2V communication pairs. However, the relatively small simulation size may limit the generalizability of their findings.

In [134], the authors focused on reliability and delay in a V2V environment, aiming to maximize the sum rate of incumbent cellular users. They formulated the problem with communication constraints, covering queue stability, subchannel allocation, QoS, and transmission power. Their proposed solution involved a Dueling double DQN-based agent capable of channel selection, power allocation, and interference management to mitigate outage probability and excessive delay. Addi- tionally, they applied FL to synchronize the DRL models of deployed agents, maintaining a global model. Comparative analysis with greedy and clustering approaches validated the effectiveness of their proposed method. The authors compared results with their variations of their works, greedy approach, and cluster based approach. To enhance their work, the authors could incorporate an optimization benchmark algo- rithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

The studies we have reviewed above illustrate the diverse and innovative approaches to sum rate maximization, ranging from tradi- tional convex optimization to advanced neural network-based solutions and reinforcement learning techniques. While machine learning algo- rithms often provide computational efficiency and scalability, many studies [38,41,51,52] fall short in comparing their approaches with traditional optimization benchmarks, which could better validate their findings. Furthermore, some works rely on limited simulation con- figurations [133], potentially restricting the generalizability of their results. Future research should focus on comprehensive benchmark- ing, hybrid algorithm design, and larger-scale simulations to bridge the gap between theoretical advancements and practical applications in sum rate optimization. These efforts will enhance the robustness and applicability of proposed methods in real-world wireless network scenarios.

    - Objective function variations

Throughput optimization has been extensively studied in various communication scenarios, including mmWave 5G, cognitive radio, UAV-based networks, augmented reality, and IoT environments, using advanced reinforcement learning and optimization techniques. Next we present works we studied from the recent literature.

In [47], the authors used a mmWave 5G environment. They formu- lated the problem of maximizing throughput in a multi-hop architec- ture. They formulated the problem as maximizing flow bits between the integrated access and backhaul network and UEs with different com- munication constraints related to data rate and link. They solved this MINLP problem with a column-generation-based approach by generat- ing optimal candidate action set (i.e., topology). The authors proposed a DRL approach based on LSTMs. To enhance their work, the au- thors could have incorporated an optimization benchmark algorithm (e.g., B&B) without considering perfect channel links to provide a stan- dard reference point for evaluating the performance and effectiveness of the proposed methods.

Motivated by the benefits of DRL, in [53], the authors formulated the problem of maximizing system throughput subject to minimum satisfaction constraints per service in multiservice wireless networks.

They studied the radio resource allocation problem in the context of optimization techniques and heuristic approaches with a single-input, single-output downlink cellular system composed of several sector cells. The objective function was formulated to maximize throughput with constraints related to data rate and QoS. The authors proposed a multi- agent DQL solution where resource blocks acted as agents and the agent’s actions consisted of choosing a UE. The agent’s state was a com- bination of two critical aspects, i.e., standard information to all agents and specific information to a particular agent. The first algorithm defined reward value as a measure of possibilities regarding satisfaction and system throughput for a given assignment. The second proposed algorithm dealt with the limitation of DQL models, i.e., limited mem- ory. The proposed algorithm could achieve better performance than the other benchmark approaches (i.e., reallocation-based assignment for improved spectral efficiency and satisfaction [142], rate maximization under experience constraints [143]) in terms of system throughput and outage rate (ratio of number of events without resource outage vs total simulated events). The study utilized a relatively small simula- tion size (six UEs, four resource blocks only), which may impact the generalizability of the results.

In [86], the authors introduced a DRL-based decentralized resource allocation scheme for cognitive radio and cooperative communication systems. They focused on maximizing system throughput using spectral bands efficiently by formulating an optimization problem to maxi- mize throughput while considering data rate, power, link, and QoS constraints. They employed cooperative DRL, where cognitive radios collaborated to determine the optimal policy. They restricted coopera- tion to radios within the same cell radius to streamline computation and reduce complexity. They proposed two algorithms based on this frame- work to solve the resource allocation problem. The authors in [53,86] compared their work with machine learning algorithm based variations of their proposed work. These works can be enhanced by including comparisons with traditional optimization algorithm baselines.

In [135], the authors investigated resource optimization in a UAV-BS scenario, where ground users relied on UAVs for network coverage. They aimed to minimize resource utilization and design optimal tra- jectories for the system. Their objective was to maximize throughput, considering transmission power, link and distance constraints. They transformed the problem into a partially observable MDP and proposed a multi-agent DRL algorithm. They integrated local critic networks to compute the global joint action value using the QMIX framework [144]. Additionally, they developed a framework for maximizing the overall throughput and calculating UAV paths for each UAV in the system. The study utilized a relatively small simulation size (two UAVs and seven users only), which may impact the generalizability of the results. The authors showed that using DRL, they can achieve the better throughput as compared to traditional deterministic optimization baseline with the added advantage of low run-time complexity of DRL based algorithm.

In [50], the authors proposed a novel machine learning-based framework to optimize resource allocation decisions for augmented re- ality by learning the relationship between computing and networking. They formulated the problem as a max–min optimization problem, aim- ing to maximize performance while minimizing the worst-case effects of parameters, with constraints related to link, QoS, and compute resource utilization (CPU utilization). Their proposed framework learned the worst-case effects by modeling stochastic variations in delays and throughput observed across both the communication infrastructure (e.g., links, routers) and the processing units (e.g., edge or cloud servers) where application data was transmitted and subsequently executed. The framework extracted maximum delay and minimum throughput values for worst-case modeling by employing different seed values for traffic generation, facilitating neural network training. How- ever, this learning approach was not error-free, and ensuring bounds remained challenging, suggesting a conservative design approach for stricter bounds. The study utilized a relatively small simulation size, which might have impacted the generalizability of the results. The

authors compared their work with machine learning algorithm-based variations of their proposed method. This work could have been fur- ther enhanced by including comparisons with traditional optimization algorithm baselines.

In [40], the authors focused on resource allocation and spectrum management in a clustered IoT environment. Initially, they solved the problem centrally using graph coloring techniques for slot scheduling. Later, they proposed a distributed block-based Q-learning algorithm for decentralized slot assignment. Their objective was to maximize the SIR across time slots, subject to link and task-specific constraints (SIR). The authors used Block-Q-learning, controllers operated independently, with rewards assigned to blocks (clusters) rather than individual slots, distinguishing their approach from traditional slot-based methods. The study utilized a relatively small simulation size, which may impact the generalizability of the results. The authors compared their work with machine learning algorithm-based variations of their proposed algorithm only.

The authors in [136] focused on a MEC-based wireless network where they optimized offloading decisions to maximize the data rate for all connected UEs. They formulated the problem as a maximization of a data rate utility function across all users, subject to constraints related to link quality and QoS requirements. They argued that tra- ditional optimization algorithms were computationally infeasible for this problem due to its complexity and dynamic nature. Instead, they employed a DRL-based approach integrated with an AI-driven SDN ap- plication plane to make intelligent offloading decisions aimed at maxi- mizing system throughput. The performance of their proposed method was compared against a deep learning-based baseline [145] and a proximity-based offloading scheme [146]. The results demonstrated that their approach achieved superior offloading decision accuracy and higher data rates compared to the other two methods. However, the relatively small simulation size (thirty UEs and multiple MECs) may have limited the generalizability of the findings.

In [39], the authors addressed the challenge of resource allocation in a wireless network, framing it as a MINLP. They aimed to maximize the minimum data rate for D2D pairs, considering various channel parameters like channel allocation indicator, transmit power, and D2D pair power on the channel. They imposed constraints on the problem related to transmission power, data rate, channel, QoS and system power. To address this problem, they developed a customized B&B algorithm. This algorithm introduced a novel approach to pruning non- optimal nodes by replacing the standard pruning policy with a learned one. They experimented with two pruning policies: one learned through imitation learning and the other being the original policy. By employing learned pruning policies, they improved the performance of the B&B method, resulting in improved optimization. Data imbalance in the dataset can be worked upon to improve the generalizability of results. In [137], the authors worked in a 5G wireless environment with

URLLC traffic, aiming to guarantee QoS while effectively managing system delay in downlink transmission. They formulated the problem as the maximization of successfully delivered packets to UEs, subject to link conditions and QoS constraints. The authors highlighted two main challenges: the frequent arrival and departure of UEs, which causes variability in the number of users requiring resource block al- location, and the inherently exploratory nature of DRL-based methods, which can lead to suboptimal performance in strict URLLC scenarios. To address these challenges, they proposed an actor-critic-based DRL architecture that leverages graph neural networks instead of tradi- tional DNNs to enhance adaptability and ensure more efficient resource allocation. The performance of their method was evaluated against variants of their proposed approach and a standard DDPG-based DRL algorithm. While the results were promising, the generalizability of their findings could be further strengthened by including comparisons with traditional optimization baselines, such as the branch-and-bound (B&B) method.

These studies highlight the potential of reinforcement learning and machine learning approaches to maximize throughput under diverse constraints; however, their generalizability could be enhanced by in- corporating traditional optimization benchmarks and larger simulation environments for broader validation.

    - Objective function tradeoffs

Throughput optimization tradeoffs has been extensively studied in various communication scenarios, including mmWave 5G, cognitive radio, UAV-based networks, augmented reality, and IoT environments, using advanced reinforcement learning and optimization techniques as discussed below.

The UAV technology has recently attracted much attention as a candidate to meet the 6G ubiquitous connectivity demand and boost the resiliency of terrestrial networks. In [138], the authors proposed a unified model-free framework, AdaptSky, for 3D UAV placement and power allocation in a NOMA-based network. The authors formulated a problem of maximizing the data rate and Jain’s fairness index [147] with data rate, transmission power, and distance constraints. They inte- grated a dual network with DRL and demonstrated the significant gain in model generalization over conventional DRL. They demonstrated the effectiveness of AdaptSky for different channel models in both the sub-6 GHz and mmWave spectrum. AdaptSky maximizes users’ sum rate while maintaining high fairness. Simulation results showed that adapt sky outperformed other optimization-based mathematical framework approaches. The authors could provide more comprehensive information about the simulation setup to enhance the generalizability of the work.

In [46], the authors addressed holistic network optimization, focus- ing on a network structured into four layers: real-world, pre-processing, edge, and cloud. Their objective was to maximize the overall system throughput. They formulated the problem as maximizing the average receiving rate on clouds, incorporating constraints related to compute resource utilization and data rate. Initially, they employed virtual queues in the system to establish Lyapunov stability. Building upon these queues, they deployed a DQL-based resource allocation algorithm to optimize resource utilization. Their proposed approach involved a real-time updating of the DRL-based edge forwarding optimization algorithm. The authors provided an appropriate benchmark algorithm comparison and showed that their proposed DRL based algorithm outperforms traditional techniques. The study utilized a relatively small simulation size (ten UEs, ten edge nodes, and ten clouds), which may impact the generalizability of the results.

In [42], the authors aimed to maximize throughput in ultra-dense networks by addressing user clustering and channel allocation. The problem involved maximizing the product of SINR and user asso- ciation with two communication constraints related to subchannels. Recognized as NP-hard and non-convex, the authors employed neural networks to construct a graph, predicting SINR using transmission time intervals and resource block information. This SINR facilitated the construction of a conflict graph, where a modified Min k-cut clustering algorithm [148] divided the wireless network into clusters. They used a dedicated algorithm which allocates subchannels to users in each cluster based on the conflict graph. As proposed in their third algo- rithm, further throughput enhancements were achieved by allocating unallocated channels. The authors compared their work with relevant graph theory-based optimization algorithm baseline, demonstrating the effectiveness of their proposed approach.

In [139], the authors addressed optimization challenges in a hybrid orthogonal multiple access-NOMA wireless environment, focusing on user association, subchannel assignment, power allocation, and multi- ple access technique selection. They aimed to maximize the downlink sum rate under link, transmission power, subchannels, and QoS con- straints, formulated as an MINLP problem. Initially, they explored greedy and meta-heuristic methods but found them computationally expensive. The authors then proposed a DRL-based resource allocation

approach, demonstrating its superiority over heuristic and greedy meth- ods through comparative analysis. The study utilized a relatively small simulation size (one BS and ten users only), which may impact the generalizability of the results.

These studies highlight the potential of reinforcement learning and machine learning approaches to maximize throughput under diverse constraints; however, their generalizability could be enhanced by in- corporating traditional optimization benchmarks and larger simulation environments for broader validation.

Lessons learned: The reviewed works confirm our observations related to the literature. Most works either need to emphasize the importance of simulation size or the use of traditional optimization algorithm baselines to improve the generalizability and generalizabil- ity of research results in resource optimization. Several studies, such as [40,41,53,138], and [43], utilized relatively small simulation en- vironments with fewer than ten BS and UE, which may limit the generalizability of their findings. Similarly, [50,135], and [139] also employed small simulation sizes, potentially impacting the robustness of their results. To enhance the contextual relevance and effective- ness of their methods, studies like [47,51,86], and [38] could benefit from incorporating traditional optimization algorithm baselines. For instance, [44] effectively provided appropriate comparisons with rel- evant benchmarks, enhancing the generalizability of their findings. In contrast, [42,46,52], and [134] could improve their work by in- corporating optimization benchmark algorithms to provide a standard reference point for evaluating performance. In terms of very specific observations, addressing data imbalance in datasets, as noted in [39], can further enhance research outcomes. This section reviewed works with sum rate as objective functions optimized primarily using machine learning, as Table 6 shows. The next section focuses on works with capacity-based objectives.

  - Capacity maximization

The capacity of a network can be defined as a collection of all available resources. These resources can be of a computational and communication nature. While some authors aim to jointly optimize system-wide capacity by integrating both computational and commu- nication resources, others focus on optimizing only one aspect of the system, either computation or communication, depending on the ap- plication context [79,82,83]. Other authors [80,107,114] have taken a more indirect approach and have optimized the resource block, i.e., basically an indirect way to look at capacity.

    - Pure objective function

Maximizing system capacity and computation resource allocation in heterogeneous and cloud-based radio networks has been a central focus of recent studies [79,82], with authors exploring advanced optimiza- tion and machine learning techniques. In this section, we review the recent literature on capacity maximization.

In [82], the authors addressed user association and resource allo- cation challenges in heterogeneous networks to maximize total system capacity. They formulated their optimization problem as maximizing system capacity with respect to link and subchannel constraints as a non-convex coupling optimization task, which they solved using two distinct approaches. The first approach decoupled user association and resource allocation, employing a multi-agent Q-learning algorithm for independent optimization. The second approach treated the problem holistically, utilizing a multi-agent DQL algorithm for simultaneous op- timization. However, the study’s limited simulation setup, comprising four macro BSs, four small BSs, and virtual small cells, raises con- cerns about the generalizability of the results. The authors compared their work with machine learning algorithm based variations of their proposed work. This work can be enhanced by including comparisons with traditional optimization algorithm baselines. In [79], the authors proposed a dynamic resource allocation framework for BSs in CRANs,

Summary of capacity-based publications for machine learning based comprehensive solutions

B&B: branch and bound; BS: base station; MIP: mixed integer programming; QoS: quality of service; SINR: signal-to-interference-plus-noise ratio; UE: user equipment.

| Reference | Objectives | Constraint type | Problem type | Classic approach | Remarks |
| --- | --- | --- | --- | --- | --- |
| [82] | Maximize system | Link, subchannel | Non-convex | – | The study utilized a relatively small |
|  | capacity |  |  |  | simulation size (four macro BSs, four |
|  |  |  |  |  | small BSs, and virtual small cells), |
|  |  |  |  |  | which may impact the generalizability of |
|  |  |  |  |  | the results. |
| [79] | Maximize computation | Compute resource utilization | – | – | The authors compared results with |
|  | capacity | (CPU utilization, task priority) , |  |  | variations of their own works. To |
|  |  | QoS |  |  | enhance the study, an optimization |
|  |  |  |  |  | benchmark algorithm (e.g., B&B) could |
|  |  |  |  |  | be incorporated. This would provide a |
|  |  |  |  |  | standard reference point for evaluating |
|  |  |  |  |  | the performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |
| [83] | Maximize network | QoS, subchannel, task-specific | – | – | The 2D representation of the state may |
|  | capacity and spectral | (SINR) |  |  | become a bottleneck as the size of |
|  | efficiency |  |  |  | action state grows. |
| [80] | Minimize resource | Link, transmission power, QoS | – | – | To enhance the study, an optimization |
|  | block allocation |  |  |  | benchmark algorithm (e.g., B&B) could |
|  |  |  |  |  | be incorporated. This would provide a |
|  |  |  |  |  | standard reference point for evaluating |
|  |  |  |  |  | the performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |
| [107] | Minimize resource | System power, transmission | Non-convex MIP | – | The authors compared results with |
|  | block allocation | power, QoS, task-specific (satellite |  |  | variations of their own work. To |
|  |  | beam bounds), compute resource |  |  | enhance the study, an optimization |
|  |  | utilization, task-specific |  |  | benchmark algorithm (e.g., B&B) could |
|  |  |  |  |  | be incorporated. This would provide a |
|  |  |  |  |  | standard reference point for evaluating |
|  |  |  |  |  | the performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |
| [114] | Maximize bandwidth, | Link, QoS, system power, | Non-convex, nonlinear | – | They compared their work with standard |
|  | user association, cache | compute resource utilization |  |  | optimization algorithms and utilized a |
|  | deployment | (cache bounds) |  |  | reasonable simulation size (one macro |
|  |  |  |  |  | BS, twenty floating access points, and |
|  |  |  |  |  | two hundred floating UEs). |

focusing on maximizing compute resource allocation while considering demand and prioritization factors. The authors formulated an optimiza- tion problem to maximize compute resource allocation with QoS, and compute resource utilization (CPU utilization, task priority) constraints. They used machine learning models, including SVM, LSTM networks, and time delay neural networks to predict computation loads, with SVM and time delay neural networks achieving the highest accuracy. They proposed two approaches for load prediction: pre-filtering for enhanced accuracy and incorporating error margins to account for uncertainty. The authors compared their work with machine learning algorithm-based variations of their proposed work. This work can be enhanced by including comparisons with traditional optimization algorithm baselines.

Authors of [79,82] demonstrated the effectiveness of machine learn- ing and optimization approaches for capacity maximization in het- erogeneous networks and CRANs. However, their robustness could be improved by incorporating large-scale simulations and comparisons with traditional optimization baselines.

    - Objective function variations

Recent advancements in resource management, spectrum allocation, and user association highlight innovative approaches such as genetic al- gorithms, reinforcement learning, and graph theory to address complex optimization challenges in wireless and satellite networks.

In [83], the authors investigated spectrum allocation in cognitive radio networks, aiming to maximize both network capacity and spectral efficiency. They formulated the problem as a multi-objective optimiza- tion task with QoS, subchannel and task-specific (SINR) constraints. The authors utilized the non-dominated sorting genetic algorithm [149]

for prioritizing diverse solutions and proposed a novel reinforcement learning-based approach to dynamically adjust parameters such as mu- tation and crossover rates. The authors developed a 2D representation of the state space using anchor graph hashing to manage the complex- ity of large state–action spaces, though this approach may introduce bottlenecks as the state space grows.

In [80], the authors proposed a resource allocation framework for small cells, focusing on minimizing resource block allocation while meeting link, transmission power, and QoS requirements. They solved the problem using a combination of graph theory and LSTM networks. Ground truth data were generated via greedy graph coloring and sub- sequently used for LSTM training. Comparative analysis demonstrated the superiority of the LSTM-based solution over KNN and DNN-based approaches. To improve their work, the authors recommended the inclusion of a standard optimization benchmark algorithm for a more comprehensive performance evaluation.

In [107], the authors addressed resource management challenges in high-throughput satellite communication network, formulating a multi- dimensional problem to minimize discrepancies between requested and available capacity. Constraints included system power, transmission power, QoS, and task-specific (satellite beam bounds). They decom- posed the problem into beamforming and user domain sub-problems and solved them using clustering with self-supervised learning and a DRL-based optimization algorithm, respectively. Simulation results with one hundred and eighty users and twelve beams showed that the proposed approach outperformed traditional benchmarks such as proportional fair algorithms and supervised learning methods.

These studies underscore the potential of advanced optimization techniques and machine learning methods in enhancing network ca- pacity and resource management; however, their impact could be

Table 8

Summary of algorithmic-parameter-based papers for machine learning based comprehensive solutions B&B: branch and bound; BS: base station.

| Reference | Objectives | Constraint type | Problem type | Traditional approach | Remarks |
| --- | --- | --- | --- | --- | --- |
| [85] | Minimize queue length | Link, data rate | Concave | – | The study utilized a relatively small |
|  |  |  |  |  | simulation size (one BS and ten users |
|  |  |  |  |  | only), which may impact the |
|  |  |  |  |  | generalizability of the results. |
| [87] | Minimize height of bin | Task-specific (bin configuration | – | – | The 2D representation model becomes a |
|  |  | constraints) |  |  | bottleneck for large-scale deployment. |
| [108] | Maximize utility | Link, QoS, distance | – | – | The authors compared the results with |
|  | function of throughput |  |  |  | variation of their own work. To enhance |
|  | and power consumption |  |  |  | the study, they could have incorporated |
|  |  |  |  |  | an optimization benchmark algorithm |
|  |  |  |  |  | (e.g., B&B). This would provide a |
|  |  |  |  |  | standard reference point for evaluating |
|  |  |  |  |  | the performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |

further validated by incorporating standard optimization benchmarks and broader comparative analyses.

    - Objective function tradeoffs

Tradeoffs in resource allocation and user association are critical in fog-based RANs, requiring advanced optimization techniques to address non-convex and nonlinear challenges effectively. Although scarce, we discuss literature available for capacity tradeoffs below.

In [114] addressed the optimization of user association, resource allocation, and cache deployment in fog-based RANs. The problem was formulated with constraints related to link, QoS, system power, and compute resource utilization (cache bounds). They formulated the problem as a non-convex, nonlinear task, which was approximated using convex optimization techniques and transformed it into a global consensus problem. They proposed an alternating direction method of multipliers (ADMM)-based optimization algorithm to solve this prob- lem, demonstrating superior performance compared to the interior point method. The study’s extensive simulation setup, which included one macro BSs, twenty floating access points, and two hundred (UE) devices, strengthened the generalizability of its findings.

# Lessons learned:

The lessons learned from the reviewed works emphasize the need for comparisons with benchmark optimization algorithms and the im- portance of adequate simulation sizes to enhance the validity and generalizability of research outcomes. Studies such as [79,83], and [80] could significantly benefit from comparing their results with bench- mark optimization algorithms. Similarly, increasing the simulation size would improve the robustness and scalability of the proposed solutions in [80,82], where limited bus routes and small numbers of macro BSs, small BSs, and virtual small cells were used. In contrast, the authors of [107] demonstrated the superiority of their proposed approach over traditional benchmarks, while [114] effectively compared their work with standard optimization algorithms using a sizable simulation setup. In the next section, we review works that optimize algorithm-based parameters for resource optimization along with a summary provided in Table 7.

  - Algorithm parameters optimizations

Various parameters can be optimized for resource optimization when using standard algorithms. Instead of utilizing known networking parameters (i.e., time or capacity), some authors have opted to model them as algorithmic parameters and then optimize those algorithmic parameters [85,87,88]. This subsection includes all these efforts.

    - Pure objective function

In [85], the authors proposed a reinforcement learning-based scheduling algorithm designed to achieve optimal throughput network performance. Their custom reward function focused on minimizing the average queue length while ensuring link and data rate constraints. The authors formulated the problem as a MDP, leveraging reinforcement learning techniques to optimize the solution. Their algorithm intro- duced a novel reward function that excluded immediate rewards to reduce learning uncertainty. However, the study employed a limited simulation environment with only one BS and ten users, which can affect the generalizability of the results.

In [87], the authors addressed resource management in the open RAN system by proposing a bin-assignment algorithm. Their objective was to minimize bin height, representing resource usage, while bal- ancing remote unit demands with distributed unit resources, subject to formal constraints related to task-specifics (i.e., bin configuration con- straints). They transformed the problem into a MDP and solved it using a two-dimensional ranked reward-inspired play algorithm, incorporat- ing policy iteration, Monte Carlo tree searches, and a DNN for action prediction. Despite its strengths, the two-dimensional representation model may face scalability issues in large-scale deployments.

In [108], the authors explored resource allocation in multi-UAV environments with dynamic communication scenarios. Their goal was to maximize system utility while considering constraints on link, QoS, and distance. Treating each UAV as an independent agent with partial state information, they employed DRL within a FL framework. This approach enables cooperative decision-making among UAVs, yield- ing superior performance compared to standard DRL-based methods. Simulations with six UAVs and two hundred users validated their framework. The authors compared their work with machine learning algorithm based variations of their proposed work. This work can be enhanced by including comparisons with traditional optimization algorithm baselines.

Lessons learned: The lessons learned from the reviewed works showed the need for appropriate simulation sizes and the inclusion of benchmark optimization algorithms to enhance the reliability and scalability of the proposed solutions. For instance, in [85], the authors used a simulation setup with only one BS and ten UEs, which is insuf- ficient for generalizable results. The authors in [108] could improve their study by incorporating an optimization benchmark algorithm (e.g., B&B), providing a standard reference point for evaluating the effectiveness and performance of their methods. In terms of context- specific comments, the two dimensional representation model in [87] could become a bottleneck in large-scale deployments, limiting its applicability. Various works discussed in this section have optimized algorithmic parameters, with detailed summaries provided in Table 8. The next section summarizes works that do not fit directly into the previously mentioned categories.

  - Less represented and task-specific formulation optimizations

In the literature, some works do not fit into any of the above categories. It is worth noting that either indirectly, these objectives are refined representations of one or more of the categories above, but establishing a link between each parameter and its core is another sub- ject altogether and is beyond the scope of this paper. In this work, we classify these works [37,45,48,49,81,84,95,96,98–102] in this category for ease of understanding.

In their study [95], the authors investigated resource allocation in end-to-end network slicing, considering uncertainties in demand, data rate requests, and CSI. They aimed to maximize the revenue-cost difference in RAN based on SDN and NFV concepts. With constraints including data rate, subchannel, transmission power, delay, QoS, link, and compute resource utilization (virtual machine assignment), they formulated the problem as a MINLP. Recurrent deterministic policy gradient was the primary solution selected by authors due to problem complexity and uncertainties among several DRL algorithms. They compared recurrent deterministic policy gradient with other methods, such as soft actor-critic, DDPG, distributed, and greedy algorithms. They found that soft actor-critic outperforms the others, highlighting its effectiveness in handling uncertainty in network slicing. The authors compared their results with greedy and DRL baselines. To enhance their work, authors could have incorporated an optimization benchmark al- gorithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

In their paper [98], the authors proposed a machine learning-based framework for resource allocation, aiming to enhance video quality amidst the challenges posed by the COVID-19 pandemic. They framed their problem as a multi-class, multi-objective optimization task and focused on increasing the number of video consumers while main- taining QoS standards with subchannel, priority and link constraints. Their proposed framework employed a hierarchical DRL approach. The master reinforcement learning agent determined the videos priority order (i.e., order in which each user should see the videos). They decomposed the objective function into three sub-problems. These sub-problems involved determining the optimal video priority policy, scheduling rules for each video class, and radio spectrum allocation to UEs. They proposed separate training algorithms for training the master and slave DRL agents, facilitating efficient resource allocation while maintaining QoS standards. To enhance their work, the authors could have incorporated an optimization benchmark algorithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

In their paper [96], the authors focused on optimizing resource allocation and user association in a back-scatter vehicular network. They formulated a non-convex combined integer programming prob- lem. They proposed a utility function to calculate the profit obtained from users, incorporating factors such as profit per user, radio and power resource costs, and backhaul network costs during commu- nication. They aimed to maximize this utility function, subject to constraints related to transmission power, link, and QoS. To solve this problem, they divided it into two sub-problems. The first sub-problem involves vehicle user equipment user association and spectrum sharing and addressed it using Q-learning to determine an association matrix and sharing scheme. For the second sub-problem of power allocation, they opted for a DNN-based approach. They trained the DNN to approx- imate weighted minimum mean square error algorithm outputs and deployed it at runtime to solve the problem efficiently. The authors evaluated their results with two different algorithms, i.e., maximum power and fixed allocation. To enhance their work, authors could have incorporated an optimization benchmark algorithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

In [102], the authors proposed a novel energy-efficient resource management approach leveraging a gated graph convolutional network

as a deep surrogate model for rapid QoS estimation. Their proposed framework incorporated three sustainability models namely, thermal, energy, and cooling—to optimize energy efficiency in data centers. By employing the performance-to-power ratio as a heuristic, their proposed approach effectively balances the load on cloud hosts, max- imizing computing power while minimizing energy consumption. The proposed model framed the objective of optimizing energy efficiency, measured by the QoS score, as a multi-objective scheduling prob- lem with task-specific (active/inactive tasks) and compute resource utilization (host CPU limits) constraints. Their approach considered the interplay of energy, thermal, and cooling factors in scheduling decisions. Through experiments conducted on simulated and physical cloud environments using the CloudSim toolkit [150] and the cou- pled simulation and container orchestration framework [151], their proposed framework demonstrated superior performance over exist- ing baselines regarding energy consumption, service level agreement violation, scheduling time, cost, and temperature. The authors made appropriate benchmark comparisons with a large simulation size in this work.

In [101], the authors worked on power allocation and pilot as- signment in a MIMO environment. They formulated the problem as maximizing spectral efficiency and uplink direction for all users. They formulated the problem with different communication constraints re- lated to power and QoS. Then, they proposed a stateless Q-learning- based algorithm for pilot sequence and power allocation. Additionally, they proposed algorithms for pilot sequence and power allocation using the Hungarian and iterative convex methods. The authors have made appropriate benchmark comparisons with relevant simulation sizes in this work. The authors showed that they can achieve same perfor- mance by using machine learning methods as traditional optimization methods with the added advantage of machine learning’s low runtime complexity.

In [100], the authors investigated a MEC scenario where IoT devices could offload tasks to MEC servers via a broker system. They framed the resource management challenge as a double auction game, aiming to maximize the sum of utility functions (resource prices) for both MEC servers and UEs. They characterize the problem by compute resource utilization (CPU utilization, bidding prices) constraints. They proposed an algorithm that combines belief learning and reinforcement learning. Reinforcement learning facilitated agents to learn from their experiences, while belief learning empowered them to select their next strategy based on observations of other agents’ choices. With this algorithm, MEC servers and UEs could dynamically adjust their bidding prices through interactions with the environment. The study utilized a relatively small simulation size (five buyers and three sellers only), which may impact the generalizability of the results.

In [99], the authors explored the optimization of sub-carrier power allocation to maximize channel spectrum efficiency. Their objective was to enhance total spectrum efficiency, subject to three constraints: two power constraints and one link constraint. They operated in a system with imperfect CSI, relying only on delayed CSI. Each D2D pair was treated as a learning agent, and the authors proposed a double DRL approach to solve the problem. They improved their DRL system by integrating experience delay, fixed Q-target, and action selection- evaluation decoupling techniques. Furthermore, they proposed a DQN training algorithm to train deep Q networks in this framework. They presented a comparative analysis against existing algorithms to demon- strate the superior performance of their approach. The authors uti- lized only two sub-carriers and twenty D2D pairs in their simulation environment.

The authors of [37] solved the resource allocation problem in a virtual reality environment where the data uploaded and downloaded were correlated. They optimized the upload and download link traffic for the whole network. The objective function aimed to maximize the successful transmission probability by optimizing resource block alloca- tion in a specific time slot. The problem had different link constraints.

They solved this problem using a Q-learning-based algorithm, using Q- learning instead of using a value table to store success probabilities and utilized an echo state network to generate these probabilities for successful transmissions. The output of the reinforcement learning algorithm was the probability of successful transmission and prediction variation of this probability. To enhance their work, the authors could have incorporated an optimization benchmark algorithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

In [45], the authors introduced an AI-enabled big-data-driven per- sonalization layer in the wireless network architecture. This layer aimed to tailor network performance to suit end-users, departing from traditional optimization for predefined parameters like SINR and throughput. They formulated the problem as a multi-objective opti- mization, simultaneously targeting user satisfaction and minimizing resource block (i.e., spectrum) utilization. The authors formulated two optimization problems with multiple constraints. The first problem focused on maximizing saved resources for users, with constraints related to data rate, link, and system power. The second problem aimed to maximize user satisfaction, with constraints related to system power, QoS, data rate, and allocation. They employed a surrogate model, utilizing a DNN, to predict and enhance user satisfaction behavior in real time to solve the formulated problems. To enhance their work, the authors could have incorporated an optimization benchmark algorithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

In [81], the authors investigated a vehicular fog computing sce- nario where vehicles have computation resources that could be utilized or shared based on availability. They proposed a contractual system rewarding vehicles for resource sharing, enabling them to request resources from roadside units if necessary. Their problem formulation aimed to maximize a utility function incorporating task processing and offloading task variables, subject to constraints related to compute resource utilization (bounds on reward achievable in the system). They proposed a scheme where offloading decisions were made using DNN, while resource allocation was handled through distributed DRL. To manage offloading conflicts, they employed a queuing model based on rewards. To enhance their work, the authors could have incorporated an optimization benchmark algorithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

The authors of [84] considered a multi-server, multi-device en- vironment where they aimed to reduce task offloading overhead by proposing a task offloading strategy. They formulated an offloading overhead minimization problem with constraints related to delay and compute resource utilization (CPU bounds). To solve this problem, they utilized the Monte Carlo-based policy iteration method. In other words, they utilized the double DQL algorithm to solve this problem. Double DQL comprises two identical Q-networks, where the target Q-network is used to calculate the target Q-value. They utilized this approach to generate better results than edge computing and local computing- only algorithms. They compared their results with random and edge computing. To enhance their work, authors could have incorporated an optimization benchmark algorithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

In [48], the authors considered a vehicular environment, empha- sizing task offloading and migration to enhance resource management through collaboration between MEC and cloud servers. They formu- lated the problem as maximizing the acceptance rate within time and resource constraints, involving five distinct constraints related to compute resource utilization (CPU bounds), link, and delay. Their proposed solution leveraged a reinforcement learning-based algorithm for selecting the execution node. If the task’s execution time was within the resources available at the local node, it was executed locally; otherwise, a reinforcement learning-based algorithm determined the

offloading node for that specific task. The authors compared their work with random task assignment (i.e., assign incoming tasks to MECs randomly) and greedy task assignment (i.e., assign incoming tasks to MEC with the most resources) results. In [49], the authors developed a hybrid traffic control scheme to enhance successful packet transmis- sions. They framed the problem as maximizing number of successful devices accessed, constrained by two task specific constraints related to access class barrier factor, and vehicle packet ratios. Their proposed solution employed a DRL algorithm, specifically a proximal policy optimization variant with a clipped surrogate objective instead of a constraint function. This approach aims to mitigate system complexity while effectively addressing the problem. The authors compared results of their proposed approach with variations of their work variations. To enhance these works [48,49], the authors could have incorporated an optimization benchmark algorithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

In [116], the authors focused on resource optimization in an indus- trial IoT setting, and specifically on a task inference scenario involving edge-cloud cooperation. They constructed a utility function comprising delay, error inference, and task queuing, subject to multiple constraints related to link, queue stability, compute resource utilization (CPU bounds, data size, and task accuracy). They converted the problem into a deterministic form using Lyapunov optimization and subsequently ap- plied a DRL algorithm for solving it. They compared variations of their algorithm by maintaining specific fixed components, thus evaluated its efficacy. The study utilized a relatively small simulation size, which may impact the generalizability of the results.

The authors in [152] proposed a resource allocation scheme for UAV-based vehicle-to-everything networks. They formulated an opti- mization problem aimed at maximizing bandwidth efficiency while satisfying constraints on transmission rate, delay, and power consump- tion. To address this NP-hard problem, they employed developed an LSTM-based approach and compared its performance against DDPG and DQN algorithms. However, the study could be strengthened by includ- ing a baseline comparison (e.g., B&B) to provide a more comprehensive performance benchmark.

The authors in [153], the authors investigated zero-touch deter- ministic communication in M2M wireless networks. They formulated an optimization problem to maximize a utility function based on la- tency guarantees, subject to constraints on successful transmission probability, subchannel allocation, device association, bandwidth, and transmission power. They addressed the non-convex nature of the problem was addressed using a MDP-like framework, for which they proposed a solution based on LSTM and random graphs [154]. Their They evaluated their approach was evaluated against DQN and DDPG algorithms. However, the study could benefit from including a baseline comparison (e.g., B&B) to enhance benchmarking.

Lessons learned: Comparison with appropriate benchmark algo- rithms can validate the past proposed works more strongly. In [48, 84,95,102], the authors compare their works with appropriate bench- mark approaches. In [37,49,81,96,98,101], the authors only compare their works with variations of their own algorithms. A traditional baseline algorithm comparison can further demonstrate the impact of the proposed solutions. For [45], the authors used only four UEs in their simulation tests, and the authors of [100,116] also used a small simulation size. In summary, the research papers in this section addressed resource allocation and optimization challenges with unique objective formulations using machine learning-based solutions. Table 9 summarizes all the works we discussed in section above.

# Machine learning as a partial solution

This section reviews the works of authors who have formulated and partially solved an optimization problem using machine learning. The distinction between comprehensive and partial lies in the extent

Table 9

Summary of less represented and task-specific formulations for machine learning based comprehensive solutions

B&B: branch and bound; D2D: device to device; DRL: deep reinforcement learning; MEC: mobile edge computing; MILP: mixed integer linear programming; MINLP: mixed-integer nonlinear programming; QoS: quality of service; RAN: radio access network.

| Reference | Objectives | Constraint type | Problem type | Classic approach | Remarks |
| --- | --- | --- | --- | --- | --- |
| [95] | Maximize difference | Data rate, subchannel, | MINLP | – | The authors compared their results with |
|  | between revenue and | transmission power, link, delay, |  |  | greedy and DRL baseline. To enhance |
|  | RAN cost | QoS, compute resource utilization |  |  | the study, an optimization benchmark |
|  |  | (virtual machine assignment) |  |  | algorithm (e.g., B&B) could have been |
|  |  |  |  |  | incorporated. This would provide a |
|  |  |  |  |  | standard reference point for evaluating |
|  |  |  |  |  | the performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |
| [98] | Maximize number of | Subchannel, priority, link | Multi-objective | – | To enhance the study, the authors could |
|  | customers with respect |  |  |  | have incorporated an optimization |
|  | to QoS |  |  |  | benchmark algorithm (e.g., B&B). This |
|  |  |  |  |  | would provide a standard reference |
|  |  |  |  |  | point for evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |
| [96] | Maximize profit from | QoS, transmission power, link | Non-convex integer | – | To enhance the study, the authors could |
|  | user |  | programming |  | have incorporated an optimization |
|  |  |  |  |  | benchmark algorithm (e.g., B&B). This |
|  |  |  |  |  | would provide a standard reference |
|  |  |  |  |  | point for evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |
| [102] | Maximize QoS score | Task-specific (on active, inactive | – | – | The authors have made appropriate |
|  |  | tasks), computation resource |  |  | benchmark comparisons along with a |
|  |  | utilization (host CPU limits) |  |  | good simulation size in this work. |
| [101] | Maximize spectral | Transmission power, QoS | – | – | The authors have made appropriate |
|  | efficiency and user |  |  |  | benchmark comparisons along with a |
|  | antenna direction |  |  |  | good simulation size in this work. |
| [100] | Maximize utility | Compute resource utilization | – | – | The study utilized a relatively small |
|  | function of buyer and | (CPU utilization, bidding prices) |  |  | simulation size (five buyers and three |
|  | seller nodes |  |  |  | sellers only), which may impact the |
|  |  |  |  |  | generalizability of the results. |
| [99] | Maximize spectrum | Link, system power | – | – | The authors utilized only two |
|  | efficiency |  |  |  | sub-carriers and twenty D2D pairs in |
|  |  |  |  |  | their simulation environment. |
| [37] | Maximize successful | Link | Non-convex | – | To enhance the study, the authors could |
|  | transmission probability |  |  |  | have incorporated an optimization |
|  |  |  |  |  | benchmark algorithm (e.g., B&B). This |
|  |  |  |  |  | would provide a standard reference |
|  |  |  |  |  | point for evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |
| [45] | Maximize user | Data rate, link, system power | – | – | To enhance the study, the authors could |
|  | satisfaction and saved |  |  |  | have incorporated an optimization |
|  | resources |  |  |  | benchmark algorithm (e.g., B&B). This |
|  |  |  |  |  | would provide a standard reference |
|  |  |  |  |  | point for evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |
| [81] | Maximize utility | Compute resource utilization | – | – | An optimization benchmark algorithm |
|  | function (reward based | (bounds on rewards available) |  |  | (e.g., B&B) could have been incorporated |
|  | on processing own and |  |  |  | to enhance the study. This would |
|  | other vehicles’ tasks) |  |  |  | provide a standard reference point for |
|  |  |  |  |  | evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |
| [84] | Minimize node | Delay, link, compute resource | – | Iterative Monte Carlo | The authors compared their results with |
|  | overhead | utilization (CPU utilization at |  |  | random and edge computing. To |
|  |  | MEC) |  |  | enhance the study, they could have |
|  |  |  |  |  | incorporated an optimization benchmark |
|  |  |  |  |  | algorithm (e.g., B&B). This would |
|  |  |  |  |  | provide a standard reference point for |
|  |  |  |  |  | evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |

(continued on next page)

of machine learning involvement. Traditional optimization algorithms contribute more to the solution than the machine learning component in these partial solutions. The formulated problem is typically divided into multiple sub-problems, and one or more (but not all) sub-problems are addressed using machine learning. The remaining sub-problems are solved using traditional optimization approaches. Fig. 5 shows the various classifications in the partial solutions’ category.

  - Energy minimization

Energy is a critical component in any future wireless network system. This subsection reviews energy-related based papers [55,104, 105,112] that proposed partial solutions in their formulations using machine learning.

Table 9 (continued).

| Reference | Objectives | Constraint type | Problem type | Classic approach | Remarks |
| --- | --- | --- | --- | --- | --- |
| [48] | Maximize task | Link, delay, compute resource | MILP | – | The authors compared with random and |
|  | acceptance rate at MEC | utilization (CPU bounds) |  |  | greedy results. To enhance the study, an |
|  |  |  |  |  | optimization benchmark algorithm (e.g., |
|  |  |  |  |  | B&B) could be incorporated. This would |
|  |  |  |  |  | provide a standard reference point for |
|  |  |  |  |  | evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |
| [49] | Maximize number of | Task-specific (Access class barrier | – | – | The authors compared their results with |
|  | successful devices | factor, device transmission ratio) |  |  | variations of their own work. To |
|  | accessed |  |  |  | enhance the study, they could have |
|  |  |  |  |  | incorporated an optimization benchmark |
|  |  |  |  |  | algorithm (e.g., B&B). This would |
|  |  |  |  |  | provide a standard reference point for |
|  |  |  |  |  | evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |
| [116] | Maximize bandwidth | data rate, delay, and power | – | – | The authors compared their results with |
|  | efficiency |  |  |  | variations of their own work. To |
|  |  |  |  |  | enhance the study, they could have |
|  |  |  |  |  | incorporated an optimization benchmark |
|  |  |  |  |  | algorithm (e.g., B&B). This would |
|  |  |  |  |  | provide a standard reference point for |
|  |  |  |  |  | evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |
| [152] | Maximize number of | Task-specific (Access class barrier | Non-convex | – | The authors compared their results with |
|  | successful devices | factor, device transmission ratio) |  |  | DQN and DDPG based algorithms. To |
|  | accessed |  |  |  | enhance the study, they could have |
|  |  |  |  |  | incorporated an optimization benchmark |
|  |  |  |  |  | algorithm (e.g., B&B). This would |
|  |  |  |  |  | provide a standard reference point for |
|  |  |  |  |  | evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |
| [153] | Maximize utility | Successful transmission | Non-convex | – | The authors compared their results with |
|  | function of latency | probability, subchannel allocation, |  |  | DQN and DDPG based algorithms. To |
|  | guarantees | device association, bandwidth, |  |  | enhance the study, they could have |
|  |  | and transmission power |  |  | incorporated an optimization benchmark |
|  |  |  |  |  | algorithm (e.g., B&B). This would |
|  |  |  |  |  | provide a standard reference point for |
|  |  |  |  |  | evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |

/ig. 5. Machine learning as a partial solution.

    - Pure objective function

In [104], the authors proposed a deep learning-based framework to solve distributed non-convex optimization problems in wireless re- source management, aiming to minimize system power consumption under constraints related to link, data rate, capacity, and queue sta- bility. Instead of Lyapunov optimization, they applied a primal–dual- based constrained training strategy, transforming the problem into a tractable unsupervised learning task. Two solution strategies were

proposed. The first was a centralized deep learning approach where a single DNN, trained with global CSIs, produced optimal power control solutions. The second was a distributed implementation designed for networks with limited backhaul capacity. In this approach, each node was equipped with two DNN units, a quantizer and an optimizer, en- abling local decision-making through low-bit binary message exchange using a stochastic binarization technique. This allowed each node to independently infer optimal strategies with limited coordination. They demonstrated the approach on power control problems in cognitive multiple access channels and interference channels. The distributed framework learned efficient decentralized behavior without full knowl- edge of global states. Their results showed that both centralized and distributed approaches achieved near-optimal performance compared to traditional optimization baselines, validating the potential of deep learning in constrained wireless resource management.

The authors of [36] focused on minimizing energy consumption in a NOMA-enabled relay-assisted FL environment. They aimed to minimize energy consumption in local device computations and the transmission of local model parameters to the server. They formulated the problem with constraints related to QoS, transmit power and link, making it NP- hard. They divided the problem into two sub-problems. They utilized a graph theory-based algorithm to calculate a joint FL graph for the resource scheduling and power allocation sub-problem. They derived a closed-form solution for the second sub-problem of computation frequency allocation based on the solution to the first sub-problem. Both solutions were executed iteratively to achieve a joint solution. They proposed a graph pruning approach to reduce the complexity of solving the resource scheduling and power allocation problem, thereby

Table 10

Summary of energy-based related publications on machine learning based partial solutions

B&B: branch and bound; BS: base station; DRL: deep reinforcement learning; MIP: mixed integer programming; NP: nondeterministic polynomial time; QoS: quality of service; UE: user equipment.

| Reference | Objectives | Constraint type | Problem type | Classic approach | Remarks |
| --- | --- | --- | --- | --- | --- |
| [104] | Minimize power | Transmission power, delay, other, | MIP | Lyapunov | The authors performed an appropriate |
|  | utilization | computation task scheduling, |  | optimization | comparison with a traditional |
|  |  | other computation |  |  | optimization solution. |
| [36] | Minimize energy | QoS, transmit power, link | NP-hard | – | They only utilize DRL variations to |
|  | consumption |  |  |  | compare with their work. An |
|  |  |  |  |  | optimization algorithm baseline can be |
|  |  |  |  |  | added. |
| [55] | Maximize energy | System power, link, QoS | – | Lagrange | A suitable comparison with an optimal |
|  | efficiency |  |  | decomposition | baseline solution (i.e., gradient iteration) |
|  |  |  |  | method | is provided, which uses a simulation |
|  |  |  |  |  | setup consisting of three BSs and sixty |
|  |  |  |  |  | users. Although this setup may be seen |
|  |  |  |  |  | as limited in scale, it effectively |
|  |  |  |  |  | demonstrates the feasibility and relative |
|  |  |  |  |  | performance of the proposed method in |
|  |  |  |  |  | a controlled environment. |
| [155] | Minimize energy cost | Latency, compute resource | – | Lagrange | The study utilized a relatively small |
|  |  | utilization (CPU bounds at UE |  | decomposition | simulation size (one access point and |
|  |  | and server) |  | method, closed form | ten UEs), which may impact the |
|  |  |  |  |  | generalizability of the results. |
| [105] | Maximize system power | Transmission power, QoS, link | Duel-objective | Nash equilibrium | The study utilized a relatively small |
|  | and maximize |  |  |  | simulation size, which may impact the |
|  | throughput |  |  |  | generalizability of the results. |
| [112] | Maximize energy and | System power, link, compute | Non-convex | Bubble sort | The study utilized a relatively small |
|  | time | resource utilization (collaborative |  |  | simulation size (twelve users, four |
|  |  | task processing), capacity, system |  |  | high-altitude balloons only), which may |
|  |  | power |  |  | impact the generalizability of the results. |
| [156] | Maximize latency and | Delay, compute resource | NP-hard | – | They compared their results with three |
|  | energy consumption | utilization (CPU utilization), |  |  | DRL-based algorithms i.e., advantage |
|  |  | distance, link, |  |  | actor-critic [157], Double Dueling Deep |
|  |  |  |  |  | Q-Network [158], and soft actor-critic |
|  |  |  |  |  | [159]. A complete traditional |
|  |  |  |  |  | optimization algorithm, e.g., B&B, can |
|  |  |  |  |  | be added to increase the generalizability |
|  |  |  |  |  | of their work. |

improving their algorithm’s performance. Comparative analysis against fixed and variations of their algorithms showed superior performance of their proposed solution. This work can be enhanced by including comparisons with traditional optimization algorithm baselines.

    - Objective function variations

The authors of [55] aimed to optimize power allocation, user as- sociation, and subchannel. They defined an optimization problem to maximize energy efficiency, establishing different constraints related to system power, link, and QoS. To solve the user association problem, they proposed an algorithm based on the Lagrange dual decomposition method. For power and subchannel allocation, they employed machine learning techniques. Specifically, they trained two separate DNN for each case semi-supervised. These DNN were used to learn the mapping for optimal power and subchannel allocation. In the case of the sub- channel network, the input consisted of gain and channeling strategies, while the output was the optimal subchannel for the user. A suitable comparison with an optimal baseline solution (i.e., gradient iteration) is provided, which uses a simulation setup consisting of three BSs and sixty users. Although this setup may be seen as limited in scale, it effectively demonstrates the feasibility and relative performance of the proposed method in a controlled environment.

The authors of [155] aimed to optimize system energy cost in a MEC environment. They formulated an objective function to minimize the energy cost of the system, considering both access points and UEs. The problem was subject to constraints related to latency and compute resource allocation (CPU bounds for UE and servers). The authors decomposed the problem into two sub-problems. They proposed a DRL- based network for the first sub-problem of wireless channel allocation

and offloading. They utilized a closed-form solution and an advanced optimization algorithm for the second sub-problem of optimal local frequency. They compared the performance of their proposed solution with binary offloading, local computing, and full offloading schemes. The study utilized a relatively small simulation size (one access point and ten UEs), which may impact the generalizability of the results.

    - Objective function tradeoffs

In [105], the authors solved a dual-optimization problem to en- hance energy efficiency and throughput. They formulated an objective function that minimized total power consumption while maximizing the SINR with constraints related to power, SINR, and interference. Considering a system with imperfect CSI that included both delay and estimation error, they developed a game theory model. This model defined the players, strategy space, and utility. Next, they converted the problem into a non-cooperative game optimization problem, and the authors theoretically proved the existence of a Nash equilibrium for their model. Assuming the precoding and interference suppression matrix aligned with the perfect channel state, they used an RNN-based algorithm to predict the utility function, focusing on predicting the in- terference suppression matrix. Leveraging these RNN predictions, they designed an iterative algorithm to optimize power. The study utilized a relatively small simulation size (four receiving, eight transmitting antennas and thirty UEs), which may impact the generalizability of the results.

The authors of [112] aimed to minimize computation time and energy consumption in MEC high altitude baloon (HAB) network. They formulated an optimization problem to optimize the weighted sum of

energy and time consumption for each user. The objective function involved optimizing user association, service sequence, and task allo- cation by minimizing energy and time. The problem had constraints for system power, link, and compute resource utilization (collaborative task processing). Given the non-convex nature of the formulated prob- lem, they divided it into three sub-problems. For the user association sub-problem, they employed a FL-based SVM algorithm, where SVMs were deployed on the UE, and the FL approach computed and updated local gradients. This ensured task processing and model training at each HAB. They used a sorting algorithm (bubble sort) for the service sequence sub-problem. Finally, they utilized plain linear programming for the task allocation sub-problem to generate a solution. The study utilized a relatively small simulation size (twelve users, four HABs only), which may impact the generalizability of the results.

The authors in [156] worked on a low earth orbit satellite (LEOS)- assisted open RAN network architecture designed for remote regions such as deserts and oceans. Their objective was to provide high- bandwidth, low-latency connectivity while considering the power and computational limitations of both IoT devices and LEOSs. They aimed to minimize overall system latency and energy consumption, taking into account constraints such as delay, CPU utilization, link quality, and communication distance. To address this challenge, they decom- posed the problem into three subproblems: offloading decision, path selection, and resource allocation. They solved the offloading decision using a PPO-based DRL algorithm implemented with an actor-critic architecture [160]. For path selection, they applied Dijkstra’s algo- rithm [161], and used the Karush–Kuhn–Tucker (KKT) conditions to solve the resource allocation problem for each time slot. This hybrid approach combined the adaptability of learning-based methods with the mathematical rigor of traditional optimization techniques to im- prove overall system performance. They compared their results with three DRL-based algorithms i.e., advantage actor-critic [157], Double Dueling Deep Q-Network [158], and soft actor-critic [159]. A complete traditional optimization algorithm, e.g., B&B, can be added to increase the generalizability of their work.

Lessons learned: In this section, we reviewed more works that have made appropriate comparisons with required benchmarks and appropriate simulation sizes. In [55,104], the authors performed ap- propriate benchmark comparisons and [55] have used an appropriate simulation size. On the other hand, increasing the simulation size would improve the robustness of the results in [105,112,155]. The following section discusses papers with the data rate-based objective functions. Table 10 summarizes all the works discussed in the section above.

  - Sum rate maximization

Similar to the works on comprehensive solutions, the objective function can be modeled as sum rate [56,59–61], data rate [57,58], and a few other variations [54,62] by the authors in their solutions. This subsection reviews these works where they partially solved these problems using machine learning.

    - Pure objective function

In [56], the authors focused on optimizing the UAV position, beam- forming, and UE-UAV association in UAV-assisted cellular networks. They formulated a MINLP problem to maximize the achievable rate for UEs by optimizing the UAV position, associations, power allocations, and transmit beamforming. The problem was subject to power, data rate, link, and distance constraints. To solve this problem, they pro- posed a novel difference of convex algorithm (DCA)-based Q-Learning algorithm. The authors developed an iterative algorithm for DCA to determine transmit power, beamforming, and UAV association. Subse- quently, they employed Q-learning to calculate rewards based on the DCA algorithm’s outcomes, facilitating the construction of an optimal policy through iterative iterations. The study utilized a relatively small

simulation size (ten UAVs and fifteen UEs), which may impact the generalizability of the results.

In [59], the authors focused on resource allocation using DRL in a small cell cluster environment where complete CSI was unavail- able. Instead, they utilized partial CSI and previous sum rate data to optimize the current sum rate. They formulated the problem that aims to maximize the achievable sum rate while considering power, allocation, and link constraints. To solve the formulated problem, the authors decomposed it into per-resource-block resource allocation and power control problem, selecting the discretization level with a brute force resource allocation algorithm and experimenting with two differ- ent power levels. Composing the problem per resource block ensures statistical similarity in resource allocation problems over time. They employed DRL to predict the next sum rate, utilizing an additional tar- get network to estimate the true Q-value. Additionally, they integrated prioritized experience replay and custom loss functions into their DRL framework. They compared their results with other approaches, such as the maximum SINR ratio and the multi-arm bandit problem, demon- strating the superiority of their approach. They also did conducted an appropriate comparison with a traditional optimization benchmark.

In [60], the authors addressed a scenario involving RIS-assisted single uplink communication, where the spectrum is shared between D2D links and cellular links. They aimed to maximize the sum rate by combining power and discrete phase shifts, involving communi- cation constraints related to power, link, and QoS. Formulated as a mixed integer non-convex optimization problem, they split it into two sub-problems: power allocation and phase shift. They proposed an algorithm for power allocation using the Lagrangian method and iterative gradient descent. Next, they employed a local search method to handle the phase shift problem. Finally, they devised an algorithm to optimize the sum rate by integrating solutions from the aforementioned sub-problems. They also compared maximum transmission power and random phase shift with their proposed approach and achieved max- imum sum rate under uncertain channel conditions. To enhance their work, the authors could have incorporated a traditional optimization benchmark to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

In [61], the authors focused on the power allocation and hybrid beamforming in a high-speed railway environment to maximize the achievable sum rate. They imposed three communication constraints related to power and task-specific analog beamforming. To address this problem, they decomposed it into two sub-problems. The first sub- problem focused on beamforming and was split into two components: calculating analog beamforming for BSs and mobile relays and then de- riving a digital beamforming matrix based on these calculations. They address the second sub-problem, involving power allocation, using a multi-agent DDPG approach. They defined this environment’s agents, states, and reward functions and proposed a multi-agent DDPG-based power allocation algorithm. Additionally, they proposed a secondary algorithm to supervise the overall implementation. The authors made a performance comparison for random power allocation, maximal power allocation, DQN, and multi-agent DDPG with their proposed approach. They showed that their proposed fully digital approach achieves the best spectrum efficiency. To enhance their work, the authors could have incorporated an optimization benchmark algorithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

In [57], the authors optimized power allocation and optimal chan- nel allocation. They formulated the optimal power allocation problem to maximize the data rate in a convex function. The objective function was constrained by the devices’ power and NOMA constraints. They then converted this power allocation convex optimization problem into Lagrangian form and derived a closed-form solution using Lagrange multipliers. Subsequently, they addressed the second problem of chan- nel allocation via DQL. They arranged the devices in descending order based on their priority and then trained a DQL model to predict the

Table 11

Summary of data rate-based papers for machine learning based partial solutions

ADMM: alternating direction method of multipliers; B&B: branch and bound; CSI: channel state information; DCA: difference of convex algorithm; DDPG: deep deterministic policy gradient; DQN: deep Q network; MIP: mixed integer programming; MINLP: mixed-integer nonlinear programming; QoS: quality of service.

| Reference | Objectives | Constraints type | Problem type | Classic approach | Remarks |
| --- | --- | --- | --- | --- | --- |
| [56] | Maximize sum rate | Data rate, distance, link, system | Non-convex | DCA | The study utilized a relatively small |
|  |  | power |  |  | simulation size, which may impact the |
|  |  |  |  |  | generalizability of the results. |
| [59] | Maximize sum rate | Transmission power, link | MIP | Brute force | An appropriate comparison with a |
|  |  |  |  | algorithm | traditional optimization benchmark is |
|  |  |  |  |  | made. |
| [60] | Maximize sum rate | Transmission power, link, QoS | Mixed integer | Local search | A comparison with maximum |
|  |  |  | non-convex | algorithm | transmission power and random phase |
|  |  |  |  |  | shift is made. To enhance the study, an |
|  |  |  |  |  | optimization benchmark algorithm (e.g., |
|  |  |  |  |  | B&B) could have been incorporated. This |
|  |  |  |  |  | would provide a standard reference |
|  |  |  |  |  | point for evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |
| [61] | Maximize the | Transmission power, task-specific | Non-convex | Beamforming | Comparison with random power |
|  | achievable sum rate | (analog beamforming) |  | algorithm | allocation, maximal power allocation, |
|  |  |  |  |  | DQN, and multi-agent DDPG. An |
|  |  |  |  |  | optimization benchmark algorithm (e.g., |
|  |  |  |  |  | B&B) could be incorporated to enhance |
|  |  |  |  |  | the study. This would provide a |
|  |  |  |  |  | standard reference point for evaluating |
|  |  |  |  |  | the performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |
| [57] | Maximize data rate | Data rate, system power | Convex | Lagrange multiplier | The authors considered perfect CSI for |
|  |  |  |  |  | their work which is not a realistic |
|  |  |  |  |  | assumption. |
| [58] | Maximize data rate | Data rate, distance, link, system | MINLP | Linear programming | An appropriate comparison with a |
|  |  | power |  | solver | traditional optimization benchmark is |
|  |  |  |  |  | made. |
| [62] | Maximize data | Distance, delay, compute resource | – | Simulated annealing | The authors compared the results with |
|  | transmission rate | utilization (data storage) |  | algorithm | variation of their own work. To enhance |
|  |  |  |  |  | the study, an optimization benchmark |
|  |  |  |  |  | algorithm (e.g., B&B) could be |
|  |  |  |  |  | incorporated. This would provide a |
|  |  |  |  |  | standard reference point for evaluating |
|  |  |  |  |  | the performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |
| [54] | Maximize throughput | Link, QoS | Convex mixed | ADMM | To enhance the study, an optimization |
|  | to average delay ratio |  | integer |  | benchmark algorithm (e.g., B&B) could |
|  |  |  |  |  | be incorporated. This would provide a |
|  |  |  |  |  | standard reference point for evaluating |
|  |  |  |  |  | the performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |

probabilities for the channel-device pair. They replaced the Q table with a DNN comprising an auto-encoder and LSTM. The authors consid- ered CSI to be perfect for their work, which is impractical in real-world applications.

In [58], the authors introduced DeepMux, a deep learning-based approach designed to address channel sounding (channel sounding is a technique that evaluates a radio environment for wireless com- munication [162]) and resource allocation challenges. Although this work discusses resource-sounding solutions, its primary focus is deep learning-based resource allocation. They framed the problem as max- imizing the data rate at each station, considering constraints related to link, transmission power, distance, and data rate. This problem was initially formulated as MINLP, which was then converted into mixed integer linear programming (MILP) using classic linearization techniques due to bounded SINR values. However, solving the problem with MOSEK (Matlab Toolbox) [163] was time-consuming. To mitigate this, they used a DNN to handle binary variables, predicting them to transform the problem into linear optimization. Thus, any linear solver could address it. The DNN receives SINR values as input and generates a resource unit assignment as output. Training data for the DNN was obtained from MOSEK’s output. They compared their results

with traditional 802.11 baselines of 802.11-T0G4, 802.11-T1G4, and 802.11-T1G16 [164] regarding error vector magnitude and throughput. Additionally, they validated their solution using a wireless test bed rather than a simulation.

    - Objective function variations

In [62], the authors worked on an opportunistic network, concen- trating on user location prediction and resource allocation within the MEC network. They framed the problem as maximizing transmitted data traffic, subject to delay, distance, and compute resource utilization (data storage) constraints. This was modeled as a linear program- ming problem. They divided the problem into location prediction and caching tasks. They used LSTM to forecast the mobile UE’s location for user position prediction. In terms of cache allocation, they proposed two distinct algorithms. The first was a simulated-annealing-based multi-content offloading algorithm, known for its rapid response to changes, suitable for local optimization. Additionally, they introduced a genetic algorithm-based multi-content offload algorithm. The authors compared the performance results of their proposed approach with variations of their own work. To enhance their work, the authors could have incorporated an optimization benchmark algorithm (e.g. B&B) to

provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

    - Objective function tradeoff

In [54], the authors aimed to minimize delay and improve through- put in a V2V environment. Their framework comprises three steps. Initially, they converted the resource allocation scheme to allocate resources to various slices into an MDP problem and solved it using re- inforcement learning. Subsequently, they aggregated the D2D resources into a shared D2D resource pool. Finally, they transformed this physical resource allocation problem into a convex MIP problem to minimize average delay with constraints related to link and QoS. They then proposed an ADMM-based algorithm to address this resource allocation problem. To enhance their work, the authors could have incorporated an optimization benchmark algorithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

Lessons learned: In [58], the authors have compared their work with relevant 802.11 baselines (e.g., 802.11-T0G4, 802.11-T1G4, and 802.11-T1G16 [164]), while in [60,61], they compared their pro- posed algorithm with maximum transmission power and random phase shift. In [54,60–62] an optimization algorithm baseline can be added to improve the work. In contrast, [59] utilized appropriate bench- mark baseline. In [56], the authors considered ten UAVs and sixteen users only. They could increase the simulation size to further demon- strate the scalability and generalizability of their results. In terms of context-specific observations, the authors of [57] proposed their solution considering a perfect CSI environment which is almost im- possible in real-life scenarios. This section reviewed works with data rate as objective functions optimized using traditional optimization approaches and machine learning. Table 11 summarizes all the works discussed in the section above. In the next section, we focus on works with other formulation objectives.

  - Less represented and task-specific formulations optimizations

This section covers works that either do not align with the previ- ously defined categories [117,165,167], or fall into niche, task-specific areas with limited representation in existing literature [118,166,168].

In [165], the authors developed a distributed traffic processing framework, proposing a solution to handle traffic with elastic resource partitioning in heterogeneous networks. They aimed to minimize traffic handling time while ensuring QoS for all users. The problem was formu- lated using constraints related to transmission power, link, and compute resource utilization (CPU utilization). Given the non-convex nature of the problem, they split it into two sub-problems. The first sub-problem, targeting ultra-low latency service, was solved using a dual evolution algorithm to approximate the optimal solver for power allocation. They employed a DRL-based algorithm incorporating QoS profiling for the second sub-problem, which involved resource partitioning in a dynamic environment. Finally, they proposed a third algorithm to integrate solutions from the previous algorithms, achieving complete system functionality. They performed an appropriate comparison with an optimal traditional algorithm baseline.

In [166], the authors focused on UAV-aided D2D networks, aiming to simultaneously optimize relay deployment, channel allocation, and relay assignment. They framed the optimization problem as maximizing relay capacity while imposing two link constraints. The authors decom- posed the problem into two sub-problems. First, they optimized channel allocation and relay assignment while keeping relay deployment fixed, employing a DRL-based algorithm for a solution. Second, they fixed the channel allocation and relay assignment and optimized relay de- ployment using an online learning-based algorithm. To solve the entire problem collectively, they proposed a third iterative algorithm that called both sub-problem algorithms iteratively. The authors compared their work against an exhaustive search and convex algorithm baseline.

In [167], the authors addressed unsupervised user clustering and sub-carrier assignment. They initially utilized K-means clustering to group users and then formulated an optimization problem for assigning users to each carrier. The objective was to minimize the total exposure of users to uplink at a given time with communication constraints related to QoS, link, and power. As the problem was non-convex, they proposed an algorithm for user clustering using K-means clus- tering [171] with the elbow method. Each cluster was then assigned to a specific sub-carrier. Additionally, they introduced a secondary algorithm for power optimization, employing an iterative water-filling method to optimize the power level for each user. The study utilized a relatively small simulation size (fifteen UEs with one hundred and twenty eight subcarriers), which may impact the generalizability of the results.

In [117], the authors addressed resource allocation and scheduling in a SDN environment by deploying AI agents across different layers of SDN to enhance performance. These agents operated at multiple levels: user device level for analyzing and predicting user request behavior, SDN controller for collecting and analyzing service request data, and base controller for resource scheduling and allocation. The problem was formulated to minimize the user’s degree, representing the ratio of allocated resources to the resources demanded with constraints related to QoS, and compute resource utilization (storage at server). They proposed a resource allocation algorithm resembling a swarm optimization problem, with the AI agent composition possibly including DNN models. They assumed the communication between the BS and the agent to be perfect which is not realistic as the AI agents can make wrong decisions if there is imperfect information.

In [118], the authors targeted vehicular edge networks, seeking to develop an intelligent offloading system. They framed the problem as maximizing QoE for users, subject to constraints related to data rate, link and compute resource utilization (data storage at MEC). They divided the problem into two sub-problems. First, they quantified QoE for tasks across vehicles using a two-sided matching algorithm to maximize total utility. They addressed resource allocation in the second sub-problem using a DRL-based algorithm, enhanced with dropout regularization and DDQNs. Additionally, a parent algorithm integrated solutions from the sub-problems to solve the overall problem. The study utilized a relatively small simulation size (ten vehicles and five roadside units), which may impact the generalizability of the results.

In [168], the authors addressed the joint optimization problem of resource allocation and edge server placement, recognizing its NP- hard nature. They decomposed it into two sub-problems for resolution. They integrated Monte Carlo simulation, genetic algorithm, and neural network techniques to solve this challenge. Their objective was to min- imize the distance between BSs and their respective edge servers while considering resource allocation and server placement with constraints concerning distance and compute resource utilization (CPU bounds). First, they addressed the server resource allocation using Monte Carlo simulation and trained a neural network to replicate simulation out- comes. Second, they employed a genetic algorithm for server location determination, using the trained DNN as the fitness function to expedite the search process. The authors compared their results with K-means, using random and top-first approaches. An optimization benchmark algorithm (e.g. B&B) could have been incorporated to enhance the study. This would provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

In [169], the authors addressed a scenario where multiple users with multiple tasks required computational resources. They determined that the available system resources were sufficient to handle the cur- rent tasks. In cases where resources were abundant, they utilized an offloading indicator and cost for offloaded processing to make optimal decisions. Their objective function aimed to minimize the cost for overhead computation, while considering compute resource utilization (CPU limits at UE and MEC) constraints. Due to the NP-hard and knap- sack nature of the formulated problem, they proposed an offloading

Summary of other-formulations-based papers for machine learning based partial solutions

B&B: branch and bound; BS: base station; KKT: Karush–Kuhn–Tucker; MEC: mobile edge computing; MINLP: mixed-integer nonlinear programming; NP: nondeterministic polynomial time; QoS: quality of service; UE: user equipment.

| Reference | Objectives | Constraints type | Problem type | Classical approach | Remarks |
| --- | --- | --- | --- | --- | --- |
| [165] | Minimize the cell’s time | Transmission power, link, | Non-convex | Dual evolution | An appropriate comparison with an |
|  | to handle traffic | compute resource utilization (CPU |  | (Lagrange-based) | optimal traditional algorithm baseline is |
|  |  | utilization) |  | algorithm | done. |
| [166] | Maximize relay | Link | – | Online | The authors compared their approach |
|  | capacity |  |  | learning-based | against an exhaustive search and convex |
|  |  |  |  | algorithm | algorithm baseline. |
| [167] | Maximize total uplink | Transmission power, capacity, | Non-convex | Iterative waterfall | The study utilized a relatively small |
|  | exposure | QoS |  | method | simulation size, which may impact the |
|  |  |  |  |  | generalizability of the results. |
| [117] | Minimize regret degree | QoS, compute resource utilization | – | Particle swarm | Communication between BS and agent is |
|  |  | (storage at server) |  | optimization | as assumed to be perfect, and the AI |
|  |  |  |  |  | agents can make wrong decisions if |
|  |  |  |  |  | there is imperfect information. |
| [118] | Maximize QoS for users | Data rate, link, compute resource | – | Matching based | The study utilized a relatively small |
|  |  | utilization (data storage at MEC) |  | scheme | simulation size (ten vehicles, five |
|  |  |  |  |  | roadside units), which may impact the |
|  |  |  |  |  | generalizability of the results. |
| [168] | Minimize conveying | Distance, compute resource | NP-hard | Monte Carlo | The authors compared their results with |
|  | distance | utilization (CPU bounds) |  |  | K-means, random and top-first |
|  |  |  |  |  | approaches. To enhance the study, an |
|  |  |  |  |  | optimization benchmark algorithm (e.g., |
|  |  |  |  |  | B&B) could have been incorporated. This |
|  |  |  |  |  | would provide a standard reference |
|  |  |  |  |  | point for evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |
| [169] | Minimize cost for | Compute resource utilization | NP-hard | Genetic algorithm | The study utilized a relatively small |
|  | overhead computation | (CPU limits at UE and MEC) |  |  | simulation size (ten users, twenty tasks), |
|  |  |  |  |  | which may impact the generalizability of |
|  |  |  |  |  | the results. |
| [170] | Minimize system delay | QoS, link, transmission power, | MINLP | Lagrangian function | They simulate their network with twenty |
|  |  | compute resource allocation (CPU |  | and KKT conditions | five UEs, one edge server, and eight |
|  |  | utilization), link, task-specific |  |  | channels, which can impact the |
|  |  | (time scheduling) |  |  | generalizability of their proposed work. |

algorithm based on K-clustering and genetic algorithms to mitigate overhead computations and reduce energy consumption due to these overhead computations. Additionally, they introduced a simplified al- gorithm using multi-knapsack for the same problem. To demonstrate the superiority of their algorithms, they compared their results with genetic algorithms and particle swarm optimization. The study utilized a relatively small simulation size (ten users, twenty tasks), which may impact the generalizability of the results.

In [170], the authors worked on a cognitive IoT and 5G-based wireless network with the goal of minimizing system delay while ensuring blockchain security. They formulated their objective as the joint weighted minimization of three components: D2D transmission queuing delay, blockchain authentication computing delay, and edge computing queuing delay. The problem was constrained by QoS re- quirements, transmission power limits, compute resource availability (CPU utilization), link conditions, and task-specific constraints such as time scheduling. To address the complexity of the formulation, the problem was decomposed into two subproblems. The first subproblem, involving time scheduling, channel allocation, and power control, was solved using a DQN-based reinforcement learning algorithm. For the second subproblem – optimal compute resource allocation on the edge server – the authors constructed a Lagrangian function and applied the dual decomposition method. They then used the Karush-Kuhn-Tucker (KKT) conditions on the transformed problem to derive the optimal allocation strategy. Their system was simulated with twenty five UEs, a single edge server, and eight communication channels. While the results demonstrated effectiveness, the relatively small simulation scale may limit the generalizability of their approach.

Lessons learned: In [166], the authors compared their work with an exhaustive search and a baseline convex algorithm. In [168], the

authors compared their approach with K-means, random, and top- first algorithms but did not cover an optimization algorithm baseline compared with [165] where appropriate benchmark comparisons are given. The simulation size used in [118,167,169] is relatively small, which may impact the generalizability of the results. In term of context- specific observations, in [117], the authors considered the information flow between the user and BS to be perfect. If this flow is disturbed by an environmental factor (often the case), it can lead to incorrect results for the AI agent. In summary, the works in this section address resource allocation and optimization challenges with unique objective formulations using a combination of machine learning and traditional solutions. Table 12 summarizes all the works discussed in the section above.

# Optimization within machine learning environment

This category covers works where the researchers took a machine- learning environment in wireless networks and tried to optimize some aspects of this machine-learning environment using traditional opti- mization approaches. The most common use cases are based on FL environments where the authors work to optimize the system’s perfor- mance by optimizing FL parameters (e.g., local and global iterations). Fig. 6 shows the various classifications in the optimizations in machine learning environment-only category.

  - Energy minimization

This subsection covers machine learning-based studies that incorpo- rate power and energy considerations in their objective functions. The selected works focus on system-wide energy consumption [109,172] and related energy-efficiency formulations [173].

Summary of energy-based publications for machine learning environment-only solutions

ADMM: alternating direction method of multipliers; B&B: branch and bound; FL: federated learning.

| Reference | Objectives | Constraint type | Problem type | Classical approach | Remarks |
| --- | --- | --- | --- | --- | --- |
| [109] | Minimize energy and | Link, task-specific (FL training) | Non-convex | Lagrange multiplier | The authors performed an appropriate |
|  | delay |  |  | method | comparison with an optimal traditional |
|  |  |  |  |  | baseline algorithm. |
| [172] | Minimize energy | Data rate, delay, task-specific (FL | Non-convex | Lyapunov | The authors provided theoretical and |
|  | consumption | accuracy), compute resource |  | optimization | numerical results with relevant |
|  |  | utilization (CPU bounds) |  |  | comparisons. |
| [173] | Minimize total energy | Data rate, link, compute resource | – | ADMM | The authors compared their work with |
|  | and task computation | utilization (CPU frequency |  |  | ADMM, greedy, and optimal baselines. |
|  | time | bounds) |  |  |  |
| [174] | Minimize the energy | Transmission power, delay, | – | – | The authors compared results with |
|  | consumption for one | compute resource utilization (CPU |  |  | variations of their own work. To |
|  | iteration | time, CPU frequency, data |  |  | enhance the study, an optimization |
|  |  | storage) |  |  | benchmark algorithm (e.g. B&B) could |
|  |  |  |  |  | have been incorporated. This would |
|  |  |  |  |  | provide a standard reference point for |
|  |  |  |  |  | evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |

/ig. 6. Optimization in machine learning environment.

    - Objective function tradeoffs

In [109], the authors introduced a novel hierarchical federated edge learning framework and formulated a computation and communication resource allocation problem within this framework. They aimed to minimize the overall energy and delay for each global iteration. They imposed link and task-specific (FL training) constraints. The formu- lated problem was non-convex, so they decomposed the scheduling algorithm into two sub-problems. The first sub-problem focused on resource allocation in a single-edge server, which they addressed using the Lagrange multiplier method. For the second sub-problem involving edge association for multiple edge servers, they proposed an algorithm that constantly adjusts the edge association strategy by changing the training group formation for edges based on preference to minimize overhead. They performed an appropriate comparison with an optimal traditional baseline algorithm to demonstrate the effectiveness of their approach.

In [172], the authors focused on resource allocation for machine

learning tasks at the edge in wireless networks. They introduced an edge machine learning framework comprising various algorithms to fa- cilitate resource allocation. They formulated the problem as minimizing average long-term optimization task to balance energy consumption, learning accuracy, and latency. The objective was to minimize the expected energy consumption value, subject to data rate, delay, task- specific (FL accuracy), and compute resource utilization (CPU bounds) constraints. To solve this problem, the authors proposed a set of Lya- punov optimization-based algorithms. The authors provided theoretical

and numerical results with relevant comparisons, demonstrating the impact of their proposed approach.

In [173], the authors addressed the service placement and resource allocation problem in multi-user MEC scenarios, focusing on optimiz- ing resource allocation and AI service placement. The latter involves determining whether placing the AI model at the edge improves perfor- mance. They formulated this as a MINLP problem, aiming to optimize the total energy consumption and task computation time across all users with respect to data rate, link, and compute resource utilization (CPU frequency bounds) constraints. Initially, they proposed separate solutions for resource allocation and service placement: an ellipsoid method for resource allocation and a greedy search algorithm for service placement. They introduced a joint allocation scheme using the ADMM. Utilizing a bisection search algorithm, this approach achieved a linear time complexity of 𝐾1, offering improved efficiency. The authors

conducted performance comparisons of their proposed approach with

ADMM, greedy, and optimal traditional optimization baselines.

In [174], the authors addressed energy consumption minimization and energy harvesting maximization in a FL setting with heterogeneous devices. They formulated this as an optimization problem aiming to minimize energy consumption for training one iteration, subject to constraints related to subchannel and transmission power constraints, data size, and task scheduling. Using the Lagrangian dual method, the authors decoupled the variables and proposed solutions for processor frequency and dataset size allocation. Additionally, they optimized transmission power using an iterative algorithm and employed a closed- form solution for time splitting. They conducted a comparative analysis of their proposed approaches with variations of their algorithms. To enhance their work, authors could have incorporated an optimiza- tion benchmark algorithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

Lessons learned: We observed the generic issue of providing an appropriate benchmark in this section. In [109,172,173], the authors provided an appropriate benchmark comparison while [174] could have incorporated an optimization benchmark algorithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods. Table 13 summarizes all the works discussed in the section above.

  - Latency minimization

Time is an important metric in optimization problems. We cov- ered those works here that formulate time-based objective functions (latency, delay, time) in a machine-learning environment. In this sub- section, we consider delay [76,77], followed by latency [78] and time (other representations) [67] papers.

Summary of latency-based papers for machine learning environment-only solutions

B&B: branch and bound; FL: federated learning; IoT: Internet of things; MINLP: mixed-integer nonlinear programming; QoS: quality of service; UAV: unmanned aerial vehicle; UE: user equipment.

| Reference | Objectives | Constraint type | Problem type | Classical approach | Remarks |
| --- | --- | --- | --- | --- | --- |
| [76] | Minimize delay and | Link | Integer | Convex toolkit | The authors compared results of their |
|  | global error rate |  | programming |  | proposed approach with results of |
|  |  |  |  |  | variations of their own work. To |
|  |  |  |  |  | enhance the study, an optimization |
|  |  |  |  |  | benchmark algorithm (e.g., B&B) could |
|  |  |  |  |  | have been incorporated. This would |
|  |  |  |  |  | provide a standard reference point for |
|  |  |  |  |  | evaluating the performance and |
|  |  |  |  |  | effectiveness of the proposed methods. |
| [77] | Minimize service | Link, task-specific (FL accuracy, | MINLP | B&B | The study utilized a relatively small |
|  | latency | UAV velocity, UAV position), |  |  | simulation size (six IoT devices and one |
|  |  | compute specific constraints (UAV |  |  | UAV), which may impact the |
|  |  | CPU bounds, UE CPU bounds. ) |  |  | generalizability of the results. |
| [78] | Minimize maximum | Link, compute resource utilization | – | Iterative alternating | The study utilized a relatively small |
|  | latency for all UEs | (CPU bound) |  | algorithm | simulation size (four UEs and one |
|  |  |  |  |  | server), which may affect the |
|  |  |  |  |  | generalizability of the results. |
| [67] | Minimize machine | Link, transmission power, QoS, | – | – | The authors compared their work |
|  | learning model | compute resource utilization (CPU |  |  | against gradient-based benchmarks. |
|  | execution time and | bounds) |  |  |  |
|  | model training loss |  |  |  |  |

    - Pure objective function

In [76], the authors addressed optimization in a FL environment tailored for cognitive IoT applications in smart industries. They aimed to enhance system performance by minimizing computation delay and model error rate, treating the problem as an integer linear program- ming problem. Their objective function focused on minimizing the overall cost, which comprises computation delay and global error rate with multiple constraints related to network links. Demonstrating the convexity of the problem, they utilized standard convex optimization methods, such as in CVXPY [175], for its solution. They also addressed resource block allocation, relaxing constraints to render the problem convex, and employed standard convex toolkit methods to solve it. The authors compared results of their proposed approach with varia- tions of their own works. To enhance their work, the authors could have incorporated an optimization benchmark algorithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

In their paper, [77], the authors addressed optimizing an air-ground

integrated wireless network with a focus on minimizing service latency. Their problem formulation focused on model decision, computation resource allocation, and UAV trajectory control with constraints related to compute resource utilization, power, and communication latency. Given the non-convex nature of the formulated problem MINLP, they decomposed it into three sub-problems and converted each into a con- vex sub-problem. Then, they proposed an iterative algorithm to opti- mize each sub-problem using traditional optimization methods like B&B or the Lagrange duality method. Notably, machine learning was solely involved in DNN model selection, where each vehicle had different AI models, and the selection process utilized traditional optimization techniques. The study utilized a relatively small simulation size (six IoT devices and one UAV), which may impact the generalizability of the results.

In their paper [78], the authors addressed the issue of DNN par- titioning and offloading to resource-constrained edge devices. They focused on a DNN model requiring significant computational power for training. To facilitate computation on edge devices, they partitioned the model into smaller chunks for distribution to various edge devices. The authors formulated their resource allocation problem to minimize the maximum latency of all UEs, employing a min–max approach with link and compute resource utilization (CPU bound) constraints. addressed it as a multi-choice knapsack problem (NP-hard). They pro- posed an iterative alternating algorithm that iteratively optimized by

reallocating computation resources with minimal utility. Furthermore, they enhanced their algorithm by introducing a variable time step to improve efficiency. The study utilized a relatively small simulation size (four UEs and one server), which may impact the generalizability of the results.

    - Objective function tradeoffs

In [67], the authors solved the challenges of deploying UAVs as BSs in a FL environment, where the UAVs had limited battery capacity, and the attached devices varied in terms of battery and computation power. They developed an asynchronous FL framework, allowing networks to be trained locally without transmitting the entire dataset to the UAV BS. Local updates were computed asynchronously on high-computation devices. The authors further formulated the problem of UAV placement, device selection, and resource management, aiming to minimize model execution time and learning accuracy loss over time, considering con- straints related to link, transmission power, QoS, and compute resource utilization (CPU bounds). The formulated problem was non-convex and NP-hard. To address these challenges, the authors proposed an asynchronous FL framework utilizing asynchronous advantage actor– critic DRL. They proposed two algorithms based on reinforcement learning: the first algorithm for UAV placement, decision selection, and resource management, and the second for asynchronous FL. The authors compared their work against the gradient-based benchmarks to demonstrate the efficacy of their proposed approach.

Lessons learned: In [67], authors have compared their work against the standard adopting gradient-based benchmark, which is missing in [76]. In [76], authors have compared their work with variations of their own proposed algorithm. In [77,78], simulation size can be increased (six IoT devices plus one UAV and four UEs plus one server) to improve results.

In this section, we studied recent available literature on time- based objectives for resource optimization within machine learning environments. The upcoming section discusses works with machine learning arguments-based objectives for resource optimization. Table 14 summarizes all the works discussed in the above section.

  - Parameter optimization in machine learning based methods

This subsection reviews recent works of authors who focused on the machine learning parameters as objective functions. These broadly

Table 15

Parameter optimization in machine learning based methods for machine learning environment-only solutions

B&B: branch and bound; FL: federated learning; KLD: kullback–leibler divergence; QoS: quality of service; UAV: unmanned aerial vehicle; UE: user equipment.

| Reference | Objectives | Constraint type | Problem type | Classical approach | Remarks |
| --- | --- | --- | --- | --- | --- |
| [89] | Minimize classification | Data rate, system power | Non-smooth and | Majorization- | The authors performed an appropriate |
|  | error |  | non-convex | minimization | comparison with an optimal traditional |
|  |  |  |  | framework | baseline. |
| [93] | Minimize training loss | Link, compute resource utilization | Nonlinear and | Iterative algorithm | The authors compared the results of |
|  | and training cost | (data size constraint), system | non-convex |  | their proposed work with variations of |
|  |  | power |  |  | their own work. An optimization |
|  |  |  |  |  | benchmark algorithm (e.g., B&B) could |
|  |  |  |  |  | have been incorporated to enhance the |
|  |  |  |  |  | study. This would provide a standard |
|  |  |  |  |  | reference point for evaluating the |
|  |  |  |  |  | performance and effectiveness of the |
|  |  |  |  |  | proposed methods. |
| [75] | Minimize training time | Link, delay, system power, | Non-convex | Traditional | An appropriate comparison with an |
|  | and energy | computation task scheduling (CPU |  | optimization | optimal baseline solution (standard FL) |
|  | consumption | frequency) |  | algorithms | is done using a simulation setup |
|  |  |  |  |  | comprising a hundred UEs. While this |
|  |  |  |  |  | setup may be considered limited in |
|  |  |  |  |  | scale, it demonstrates the feasibility and |
|  |  |  |  |  | relative performance of the proposed |
|  |  |  |  |  | method in a controlled environment. |
| [90] | Minimize UAV’s overall | Link, task-specific (FL iterations | Convex | Successive convex | The authors compared their work with a |
|  | training energy | for convergence, UAV CPU |  | optimization | traditional optimization (i.e., B&B) |
|  | consumption | bounds, QoS |  |  | benchmark. |
| [94] | Maximize FL delay | Link, transmission power, QoS, | – | Proposed algorithm | The authors compared their work with a |
|  | imbalance between UEs | compute resource utilization |  |  | traditional optimization (i.e., B&B) |
|  |  |  |  |  | benchmark. |
| [92] | Minimize probability of | Data rate, task-specific (helper | Non-convex | Learning-centric | The authors compared their work with a |
|  | selection of FL helper | selection probability) |  | power allocation | traditional optimization (i.e., B&B) |
|  |  |  |  |  | benchmark. |
| [91] | Maximize learning | Link, compute resource utilization | Non-convex | Fractional | The study utilized a relatively small |
|  | efficiency | (FL round data bounds, batch size |  | optimization method | simulation size (with twelve UEs), which |
|  |  | bounds) |  |  | may impact the generalizability of the |
|  |  |  |  |  | results. |
| [97] | Minimize KLD edge | Link, system power | Non-convex | Kullback-Leibler | The authors compared their work with |
|  | nodes) |  |  | divergence KLD | state-of-the-art benchmarks and |
|  |  |  |  |  | demonstrated its superiority. |

include loss [89,93], training time [75,90], and cost [94] among oth- ers [91,92].

In [89], the authors addressed the challenge of processing machine- type communication data volumes on edge devices constrained by lim- ited power allocation typically optimized for communication through- put rather than learning throughput. Traditional algorithms like water- filling and min–max fairness proved inefficient in this context. The authors aimed to minimize classification error while considering total power, data rate, and sample collection. They developed a learning- centric power allocation algorithm variant, formulating it as a non- convex, non-smooth optimization problem. Their solution leveraged a majorization-minimization framework, constructing upper bounds on the empirical classification error model and replacing the problem with surrogate problems. They framed their objective as minimizing the maximum classification error with power budget and data rate constraints. The authors derived the optimal solution through gradient descent or brute force search, further refining parameters for gen- eralization error using convolutional neural networks and SVM. The authors compared their results with an optimal traditional baseline demonstrating the efficacy of approach proposed.

In [93], the authors worked on FL, employing classical optimization techniques to optimize FL accuracy and training trade-offs in MEC networks. They formulated the problem as a minimization task, aiming to minimize the combination of training loss and model training cost in the FL framework. Their formulation encompassed three constraints related to energy consumption, number of users, and dataset size. Given the nonlinear and non-convex nature of the problem, they ini- tially decomposed it into three independent sub-problems: accuracy

loss, communication cost, and joint optimization. Subsequently, they devised an iterative algorithm (non-machine learning) to solve each derived sub-problem iteratively. The authors compared the results with variations of their own work. To enhance their work, the authors could have incorporated an optimization benchmark algorithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods.

In [75], the authors proposed an innovative FL algorithm designed to manage heterogeneous datasets across diverse UEs in wireless net- works. They assumed that the loss function at each UE was smooth and non-convex. By employing a time-sharing multi-access protocol for communication standards, their algorithm achieves the tradeoff between convergence time and energy consumption across UEs with varying power capacities. Their optimization problem aimed to min- imize training time and energy consumption under constraints such as a time-sharing uplink transmission limit, computing time, and local accuracy. To solve this wireless domain problem, they divided it into three sub-problems, with the first two converted into convex prob- lems, leaving the third as non-convex. They solved these sub-problems using traditional optimization algorithms to achieve the overarching objective. A major limitation of their approach was its assumption that local processing must be completed before initiating communication between UEs for global training. The authors considered many UEs in their simulations and compared their work with optimal solutions.

In [90], the authors minimized the energy consumption of a UAV swarm by optimizing various parameters in a FL environment. Their optimization targets included convergence threshold, local and global iterations, bandwidth, and computation resources. The overarching

objective function aimed to optimize the overall energy consumption, integrating constraints related to local convergence thresholds, local iteration counts, computation resource allocations, bandwidth, and bandwidth and convergence thresholds. They approached this com- plex problem by segmenting it into three sub-problems, addressing them directly and via a novel iterative algorithm through successive convex optimization. Furthermore, they investigated fairness in joint optimization, introducing a new variable in the objective function and addressing this revised problem with a new iterative algorithm proposal. The authors compared their work with a standard benchmark to demonstrate the superiority of their proposed approach.

The authors of [94] explored the Internet of vehicles character- ized by multiple highly mobile vehicles. They initially developed an algorithm to determine the participation of specific vehicles in FL. Subsequently, they focused on the optimization problem of resource allocation to reduce the cost of FL for participating vehicles. They consider this problem as the minimization of FL costs (comprising of participation in the current round and waiting time for the current round to conclude). They formulated the problem subject to link, QoS, and transmission power constraints. Their proposed solution involved an algorithm for decision-making regarding participation in FL, which is input into a multi-agent DRL algorithm responsible for training and resource allocation. They developed an algorithm outlining the allo- cation procedure for multi-agent DDPG based joint resource allocation, encapsulating their comprehensive approach to FL optimization in IoVs. The authors compared their work with a standard benchmark to show the impact of their proposed approach.

In [92], the authors addressed the formulation of a resource allo- cation and scheduling problem in wireless networks. Their problem formulation aimed at minimizing a combination of the probability of selecting a helper as the FL gradient uploader, the channel assignment indicator, and the number of bits allocated by each helper for subchan- nels. They comprehensively addressed the communication dynamics with data rate, and task-specific (helper selection probabilities) con- straints. Transforming the problem into a MINLP via the transformation method, they managed the complexities inherent in their formulation. The equivalent problem was then decomposed into two sub-problems. First, they addressed the resource allocation sub-problem using the B&B method, and then with their proposed algorithm providing a sub- optimal solution near to the optimal solution with reduce complexity. Second, they employed the penalty convex-concave procedure (PCCP) technique [176] for helper scheduling, leveraging the interior point method to find efficient solutions. This multi-faceted approach enables them to address the intricacies of resource allocation and scheduling in their FL framework. The authors compared their work with a standard benchmark (i.e., B&B) to demonstrate the efficacy of their proposed approach.

In [91], the authors operated in a wireless FL environment and their goal was to address the training time acceleration problem. They introduced a novel metric, ‘‘learning efficiency’’, defined as the ratio of global loss delay to end-to-end efficiency. The objective function was formulated to maximize this learning efficiency. Their problem in- cluded constraints related to link and compute resource utilization (FL round data bounds, batch size bounds). They subsequently divided the problem into two separate sub-problems. The first sub-problem aimed to solve the local gradient calculation, uploading, and formulation, framed as a min–max optimization problem. They then transformed this problem into a convex one and resolved it using fractional op- timization. They proposed a two-dimensional search algorithm as a solution to this sub-problem. For the second subproblem, they solved the issue of downloading the global gradient and updated the local model by applying Karush-Kuhn–Tucker conditions. The study utilized a relatively small simulation size (with twelve UEs), which may impact the generalizability of the results.

In [97], the authors operated in a hierarchical FL environment with IoT heterogeneous systems. They proposed resource allocation

and user assignment while considering the user data distributions and wireless communication characteristics. They utilized the kullback– leibler divergence (KLD) to address the data imbalance problem. The objective here was to optimally allocate end users to edge nodes while minimizing the KLD. They formulated the problem with different constraints related to link and system power. The formulated problem is an integer programming NP-hard problem. The authors initially transformed this problem into a linear programming problem. They then determined the linear programming solution and optimized the bandwidth allocation for available end users, ensuring KLD minimiza- tion in the process. They compared their work with state-of-the-art benchmarks and demonstrated its superiority.

Lessons learned: The authors in [89,90,92,94,97] compared their work with the standard benchmark. In [93], the authors compared their results with variations of their own work. To enhance their work, the authors could have incorporated an optimization benchmark algorithm (e.g., B&B) to provide a standard reference point for evaluating the performance and effectiveness of the proposed methods. In [75], the authors compared their work with an optimal baseline and used a rea- sonably large simulation size (one hundred UEs). In contrast, in [91], the authors used a small simulation size with twelve UEs only. Table 15 summarizes all the works discussed in the section above.

# Challenges and future research directions

As machine learning becomes increasingly integrated into wireless networks, several critical research challenges must be addressed to en- sure reliable, scalable, and efficient deployment. In this section, we first highlight key limitations in the existing literature, followed by chal- lenges related to machine learning models and datasets. We emphasize the need for effective data handling, adaptive learning mechanisms, and improved model interpretability in dynamic wireless environments. The section concludes with suggestions recommendations for promising open research directions.

  - Limitations of existing literature

We have thoroughly analyzed works in the literature that cover in the context of the utilization of machine learning in wireless net- works. We categorized organized the existing approaches into differ- ent categories, found identified emerging trends, and highlighted key shortcomings. These common shortcomings can affect the validity, scalability, and practical impact of the proposed solutions. Instead of listing these issues, we provide a discussion of each along with its potential implications.:

- Simulation Size and Settings: Many studies rely on relatively small simulation scenarios, often with limited numbers of users, devices, or environmental variability [77,78,91,105,112, 155,169]. While this simplifies implementation, it significantly reduces the generalizability of the results and may fail to reflect the performance of the proposed models in large-scale, real-world deployments.

- Perfect CSI: A considerable number of works [28,32,35,57,59,99] assume perfect CSI, which is rarely achievable in real-world wireless networks due to estimation errors, feedback delays, and user mobility. Such assumptions can lead to overly optimistic per- formance evaluations and overlook the robustness of the proposed approaches under imperfect or partial CSI conditions.

- Comparison against standard benchmark: Some studies [54, 60–62,76,168,174] compare machine learning-based methods only against other machine learning algorithms (e.g., DDPG, DQN) without including classical optimization techniques (e.g., convex solvers, heuristic-based methods, B&B). This limits the ability to assess whether ML truly offers superior performance or if similar outcomes could be achieved through simpler, more interpretable methods.

- Simulation parameters: The choice of simulation parameters (e.g., learning rates, data rates, path loss models) is often not jus- tified or aligned with standardized wireless system models [29,30, 65,74,113,130]. This makes it difficult to compare results across different studies or replicate the experimental settings for further validation. A lot of works have these parameters not mentioned at all, which limits their reproducibility by a third party.

- Energy Consumption in ML Model deployment: The de- ployment of machine learning models, particularly deep learn- ing architectures, in energy-constrained wireless environments (e.g., IoT devices, UAVs, edge nodes) is often overlooked [29,31, 32,106]. The energy consumed during training and inference can be significant and should be factored into the evaluation, espe- cially when promoting models for real-time or mobile use cases.

  - Challenges related to machine learning and deep learning models

In future wireless networks, labeled data availability remains an active issue. In many cases, large amounts of data are available, but processing and adequately labeling the data requires a lot of work. One good example is AI applications in the healthcare domain using future wireless networks. Even if a small percentage of samples are mislabeled, it can create complications in the patient diagnosis. Deploying machine learning models on UE also becomes an issue, as pushing frequent updates means initializing and retraining the whole model again. Ad- ditionally, privacy concerns about the sharing of sensitive user data for centralized training present significant challenges, highlighting the need for increased emphasis on privacy-preserving techniques such as federated learning and differential privacy.

Machine learning models are typically trained to perform a single task. Only the most advanced models, such as reinforcement learn- ing, can handle multiple tasks, like playing several different games. However, when the input data distribution changes, for example, due to shifts in data sources or environmental conditions, the model’s output must adapt accordingly. This makes it challenging for machine learning models to generalize to unseen data or adapt to new input scenarios. Scalability is another concern because, as the computational and memory requirements for retraining or adapting models to for new conditions increase significantly in large-scale wireless networks with thousands of many devices.

In the dynamic environment of constantly changing data streams, the frequency of model updates is a crucial aspect that requires further research. The utilization of pre-trained models from existing domains in new domains is a growing trend, but in the context of resource optimization in future wireless networks, it is still in its early stages and requires more attention from authors. These areas are mature enough for further research and could significantly enhance model performance in future wireless networks. Additionally, cross-domain transfer presents challenges such as data heterogeneity, model compat- ibility, and the potential for transferring domain-specific biases, all of which must be addressed to ensure effective and reliable deployments.

  - Challenges related to dataset

The performance of a machine learning model heavily depends on the dataset. The dataset’s quality dictates how well we can perform on a specific task using machine learning. If there is a deliberate misclassification, it can lead to the model performing very poorly. To avoid this, dataset collection and labeling gain paramount importance. To have this process clean and secure, we need to have some secu- rity parameters in place to verify the correction and sanctity of the dataset. We can also use synthetic datasets in place of real datasets, but generating suitable synthetic datasets remains a challenge. Moreover, the use of synthetic data raises concerns about how well these models generalize in real-world environments, especially when the synthetic

data does not capture the complete diversity and noise of wireless network conditions.

In future wireless networks, we will have various types of sources (BSs, UEs, UAVs) generating data. All of these sources generate data at different rates and in different formats. Converting them into a uniform format and then matching them in the same time stamps is a significant challenge for researchers, network configurators and programmers. In future wireless networks, most situations studied in the literature do not have a prior dataset. Most authors resort to simulations to generate a dataset that works. This situation is a major concern because the simulation mimics but is not an original dataset. Therefore, the collection of real datasets for future wireless networks becomes very important. The lack of standardized, publicly available real-world datasets also restricts reproducibility across studies and makes it difficult to benchmark different machine learning approaches under uniform conditions.

Machine learning components are increasingly proposed in the lit-

erature to replace traditional optimization components. These compo- nents do the work much faster than traditional components but lack transparency. It means that a slight change in the input can trigger many different responses from the machine learning output. Tracking the change that triggered this change is hard. Very recently, XAI models are being utilized to overcome this challenge. Still, choosing machine learning based approaches over traditional optimization approach a difficult choice, and further research is needed to overcome this prob- lem. Additionally, integrating XAI with resource-constrained wireless environments remains an open challenge, as most XAI methods add computational overhead and may not scale well in latency-sensitive applications.

  - Open research directions

In this survey, we have conducted a comprehensive study on the use of machine learning in wireless networks. Nevertheless, there remains considerable potential for future research to enhance our understanding of this field using this work as a building blockfoundation. One good start is exploring and developing hybrid approaches that integrate machine learning with traditional optimization techniques, an area that has not yet been thoroughly investigated. Furthermore, establishing standard benchmarks, such as publicly accessible datasets or reference models, would will create a common foundation for evaluating perfor- mance across various wireless scenarios and enhance reproducibility in future studies. Lastly, cross-layer optimization, which includes joint optimization across different domains (e.g., air, ground, and sea net- works), represents is another important research direction for future exploration. Energy consumption associated with deploying and train- ing machine learning models, particularly deep learning models, is rarely addressed in the existing literature. There is a significant need for further research to evaluate the energy impact of deploying these models in wireless environments, especially on resource-constrained devices.

# Conclusion

Future wireless networks have the potential to bring disruptive changes in our lives. Shifting offices online to the wide adoption of online education is only possible with the advancements of these fast and high-speed networks (future wireless networks). The variety of de- vices that constitute these networks is enormous. These devices include traditional BSs, UAVs, HABs, mobile relay nodes, various kinds of UEs, and much more. Resource management optimization, and scheduling becomes crucial among other aspects of these networks. In summary, these future wireless networks will face challenges in resource alloca- tion and intelligent decisions on the fly for optimization and scheduling in certain situations. Most traditional optimization methods are un- suitable in these scenarios as there is a trade-off for either time or

precision. Unlike these traditional methods, deep learning methods do not require this trade-off and can produce a workable output quickly and accurately. These models derive their decisions from the context and input information. They also improve by updating themselves via retraining previous information with these updated data batches.

Our survey provides a structured taxonomy of machine learning techniques utilization in wireless communication. We, critically ana- lyzing analyze their assumptions and limitations while also identifying underexplored areas such as energy-aware ML and hybrid optimization models (further highlighted in the open research direction section). From a practical viewpoint, our survey acts as an aid for system design- ers and engineers by highlighting the use, advantages, and drawbacks of different machine learning-based techniques in practical wireless scenarios, especially in resource-constrained contexts like IoT and UAV networks.

The survey underscores the advantages of these technologies over traditional methods in terms of scalability, adaptability, and efficiency by exploring a range of approaches that include comprehensive machine-learning to machine learning environment specific solutions. The synthesis of existing literature indicates that, while machine learn- ing has unveiled new capabilities such as dynamic adaptability, pre- dictive modeling, and context-aware decision-making, significant gaps and challenges remain. These include the necessity for robust bench- marks, larger-scale simulations, and better integration with diverse network environments. Additionally, the focus on sustainability, en- ergy efficiency, and real-time responsiveness highlights the pressing need for ongoing innovation in algorithm design and deployment strategies, making contributions in this field more crucial than ever. Future research should address these shortcomings by promoting in- terdisciplinary collaborations, refining machine learning models to improve their interpretability and fairness, and incorporating emerging paradigms like FL and generative AI. By pursuing these directions, the field can ensure that machine learning remains pivotal in the efficient, intelligent, and adaptive resource management needed for next-generation wireless networks.

# CRediT authorship contribution statement

Mudassar Liaq: Writing – original draft, Methodology, Investiga- tion, Formal analysis, Conceptualization. Sana Sharif: Writing – review & editing, Methodology, Investigation. Sherali Zeadally: Writing – re- view & editing, Validation, Supervision. Waleed Ejaz: Writing – review & editing, Validation, Supervision, Resources, Funding acquisition.

# Declaration of competing interest

The authors declare that they have no known competing finan- cial interests or personal relationships that could have appeared to influence the work reported in this paper.

# Acknowledgments

We thank the anonymous reviewers for their valuable comments which helped us improve the content, organization, and presentation of this paper. This research work was supported by Natural Sciences and Engineering Research Council of Canada (NSERC).

# Data availability

No data was used for the research described in the article.

# References

- U.S. Shanthamallu, A. Spanias, C. Tepedelenlioglu, M. Stanley, A brief survey of machine learning methods and their sensor and IoT applications, in: 2017 8th International Conference on Information, Intelligence, Systems & Applications, IISA, IEEE, 2017, pp. 1–8.

- J. Kaur, M.A. Khan, M. Iftikhar, M. Imran, Q.E.U. Haq, Machine learning techniques for 5G and beyond, IEEE Access 9 (2021) 23472–23488.

- J. Jagannath, N. Polosky, A. Jagannath, F. Restuccia, T. Melodia, Machine learn- ing for wireless communications in the Internet of Things: A comprehensive survey, Ad Hoc Netw. 93 (2019) 101913.

- S. Sritharan, H. Weligampola, H. Gacanin, A study on deep learning for latency constraint applications in beyond 5G wireless systems, IEEE Access 8 (2020) 218037–218061.

- K.B. Letaief, Y. Shi, J. Lu, J. Lu, Edge artificial intelligence for 6G: Vision, enabling technologies, and applications, IEEE J. Sel. Areas Commun. 40 (1) (2022) 5–36.

- M. Lin, Y. Zhao, Artificial intelligence-empowered resource management for future wireless communications: A survey, China Commun. 17 (3) (2020) 58–77.

- X. Xu, H. Li, W. Xu, Z. Liu, L. Yao, F. Dai, Artificial intelligence for edge service optimization in internet of vehicles: A survey, Tsinghua Sci. Technol. 27 (2) (2022) 270–287.

- F. Tang, B. Mao, Y. Kawamoto, N. Kato, Survey on machine learning for intelligent end-to-end communication toward 6G: From network access, routing to traffic control and streaming adaption, IEEE Commun. Surv. Tutor. 23 (3) (2021) 1578–1598.

- F. Hussain, S.A. Hassan, R. Hussain, E. Hossain, Machine learning for resource management in cellular and IoT networks: Potentials, current solutions, and open challenges, IEEE Commun. Surv. Tutor. 22 (2) (2020) 1251–1275.

- A. Mughees, M. Tahir, M.A. Sheikh, A. Ahad, Towards energy efficient 5G networks using machine learning: Taxonomy, research challenges, and future research directions, IEEE Access 8 (2020) 187498–187522.

- Y. Sun, M. Peng, Y. Zhou, Y. Huang, S. Mao, Application of machine learning in wireless networks: Key techniques and open issues, IEEE Commun. Surv. Tutor. 21 (4) (2019) 3072–3108.

- A. Salh, L. Audah, N.S.M. Shah, A. Alhammadi, Q. Abdullah, Y.H. Kim, S.A. Al-Gailani, S.A. Hamzah, B.A.F. Esmail, A.A. Almohammedi, A survey on deep learning for ultra-reliable and low-latency communications challenges on 6G wireless systems, IEEE Access 9 (2021) 55098–55131.

- R. Masroor, M. Naeem, W. Ejaz, Resource management in UAV-assisted wireless networks: An optimization perspective, Ad Hoc Netw. 121 (2021) 102596.

- S. Zhang, D. Zhu, Towards artificial intelligence enabled 6G: State of the art, challenges, and opportunities, Comput. Netw. 183 (2020) 107556.

- H. Dahrouj, R. Alghamdi, H. Alwazani, S. Bahanshal, A.A. Ahmad, A. Faisal, R. Shalabi, R. Alhadrami, A. Subasi, M.T. Al-Nory, et al., An overview of machine learning-based techniques for solving optimization problems in communications and signal processing, IEEE Access 9 (2021) 74908–74938.

- Z. Shah, M. Naeem, U. Javed, W. Ejaz, M. Altaf, A compendium of radio resource management in UAV-assisted next generation computing paradigms, Ad Hoc Netw. 131 (2022) 102844.

- S. Taimoor, L. Ferdouse, W. Ejaz, Holistic resource management in UAV-assisted wireless networks: An optimization perspective, J. Netw. Comput. Appl. (2022) 103439.

- H. Sun, Y. Liu, A. Al-Tahmeesschi, A. Nag, M. Soleimanpour-Moghadam, B. Canberk, H. Arslan, H. Ahmadi, Advancing 6G: Survey for explainable AI on communications and network slicing, IEEE Open J. Commun. Soc. (2025).

- D. Alsadie, A comprehensive review of AI techniques for resource management in fog computing: Trends, challenges and future directions, IEEE Access (2024).

- S. Wang, M.A. Qureshi, L. Miralles-Pechuán, T. Huynh-The, T.R. Gadekallu,

M. Liyanage, Explainable AI for 6G use cases: Technical aspects and research challenges, IEEE Open J. Commun. Soc. (2024).

- S.S. Sefati, A.U. Haq, R. Craciunescu, S. Halunga, A. Mihovska, O. Fratu, et al., A comprehensive survey on resource management in 6G network based on Internet of Things, IEEE Access (2024).

- N. Alhussien, T.A. Gulliver, Toward AI-enabled green 6G networks: A resource management perspective, IEEE Access (2024).

- W. Ejaz, S.K. Sharma, S. Saadat, M. Naeem, A. Anpalagan, N.A. Chughtai, A comprehensive survey on resource allocation for CRAN in 5G and beyond networks, J. Netw. Comput. Appl. 160 (2020) 102638.

- K. Petersen, S. Vakkalanka, L. Kuzniarz, Guidelines for conducting systematic mapping studies in software engineering: An update, Inf. Softw. Technol. 64 (2015) 1–18.

- S. Keele, et al., Guidelines for performing systematic literature reviews in software engineering, 2007.

- M. Petticrew, H. Roberts, Systematic Reviews in the Social Sciences: A Practical Guide, John Wiley & Sons, 2008.

- H. Xiang, M. Peng, Y. Sun, S. Yan, Mode selection and resource allocation in sliced fog radio access networks: A reinforcement learning approach, IEEE Trans. Veh. Technol. 69 (4) (2020) 4271–4284.

- G. Qian, Y. Zhou, C. He, X. Li, D. Feng, Power allocation scheme based on support vector machine forDAS and CAS, Phys. Commun. 38 (2020) 100941.

- Z. Wang, T. Lv, Z. Chang, Computation offloading and resource allocation based on distributed deep learning and software defined mobile edge computing, Comput. Netw. 205 (2022) 108732.

- H. Amiriara, M.R. Zahabi, V. Meghdadi, Power-location optimization for coop- erative nomadic relay systems using machine learning approach, IEEE Access 9 (2021) 74246–74257.

- M. Pourghasemian, M.R. Abedi, S.S. Hosseini, N. Mokari, M.R. Javan, E.A. Jorswieck, AI-based mobility-aware energy efficient resource allocation and trajectory design for NFV enabled aerial networks, IEEE Trans. Green Commun. Netw. (2022).

- Y. Liu, C. He, X. Li, C. Zhang, C. Tian, Power allocation schemes based on machine learning for distributed antenna systems, IEEE Access 7 (2019) 20577–20584.

- W. Zhang, Z. Zhang, S. Zeadally, H.-C. Chao, V.C. Leung, MASM: A multiple- algorithm service model for energy-delay optimization in edge artificial intelligence, IEEE Trans. Ind. Inform. 15 (7) (2019) 4216–4224.

- J. Zhang, Y. Shen, Y. Wang, X. Zhang, J. Wang, Dual-timescale resource allocation for collaborative service caching and computation fffloading in IoT systems, IEEE Trans. Ind. Inform. 19 (2) (2023) 1735–1746.

- N. Sharma, K. Kumar, Energy efficient clustering and resource allocation strategy for ultra-dense networks: A machine learning framework, IEEE Trans. Netw. Serv. Manag. (2023).

- M.S. Al-Abiad, M.Z. Hassan, M.J. Hossain, Energy-efficient resource allocation for federated learning in NOMA-enabled and relay-assisted internet of things networks, IEEE Internet Things J. 9 (24) (2022) 24736–24753.

- M. Chen, W. Saad, C. Yin, M. Debbah, Data correlation-aware resource management in wireless virtual reality (VR): An echo state transfer learning approach, IEEE Trans. Commun. 67 (6) (2019) 4267–4280.

- A.A. Khan, R.S. Adve, Centralized and distributed deep reinforcement learning methods for downlink sum-rate optimization, IEEE Trans. Wirel. Commun. 19

(12) (2020) 8410–8426.

- M. Lee, G. Yu, G.Y. Li, Learning to branch: Accelerating resource allocation in wireless networks, IEEE Trans. Veh. Technol. 69 (1) (2020) 958–970.

- F. Hussain, R. Hussain, A. Anpalagan, A. Benslimane, A new block-based reinforcement learning approach for distributed resource allocation in clustered Iot networks, IEEE Trans. Veh. Technol. 69 (3) (2020) 2891–2904.

- Y.-H. Xu, X. Liu, W. Zhou, G. Yu, Generative adversarial LSTM networks learning for resource allocation in UAV-served M2M communications, IEEE Wirel. Commun. Lett. 10 (7) (2021) 1601–1605.

- J. Cao, T. Peng, X. Liu, W. Dong, R. Duan, Y. Yuan, W. Wang, S. Cui, Resource allocation for ultradense networks with machine-learning-based interference graph construction, IEEE Internet Things J. 7 (3) (2020) 2137–2151.

- G. Jia, Z. Yang, H.-K. Lam, J. Shi, M. Shikh-Bahaei, Channel assignment in uplink wireless communication using machine learning approach, IEEE Commun. Lett. 24 (4) (2020) 787–791.

- Q.-V. Pham, T. Huynh-The, M. Alazab, J. Zhao, W.-J. Hwang, Sum-rate max- imization for UAV-assisted visible light communications using NOMA: Swarm intelligence meets machine learning, IEEE Internet Things J. 7 (10) (2020) 10375–10387.

- R. Alkurd, I.Y. Abualhaol, H. Yanikomeroglu, Personalized resource alloca- tion in wireless networks: An AI-enabled and big data-driven multi-objective optimization, IEEE Access 8 (2020) 144592–144609.

- C. Zhang, M. Dong, K. Ota, Employ AI to improve AI services: Q-learning based holistic traffic control for distributed co-inference in deep learning, IEEE Trans. Serv. Comput. 15 (2) (2022) 627–639.

- B. Zhang, F. Devoti, I. Filippini, D. De Donno, Resource allocation in mmWave 5G IAB networks: a reinforcement learning approach based on column generation, Comput. Netw. 196 (2022) 108248.

- E. Karimi, Y. Chen, B. Akbari, Task offloading in vehicular edge computing networks via deep reinforcement learning, Comput. Commun. 189 (2022) 193–204.

- H. Zhang, M. Jiang, X. Liu, K. Long, V.C. Leung, AI-aided traffic control scheme for M2M communications in the internet of vehicles, in: ICC 2022-IEEE International Conference on Communications, IEEE, 2022, pp. 5053–5057.

- D. Roy, A.S. Rao, T. Alpcan, G. Das, M. Palaniswami, Achieving AI-enabled robust end-to-end quality of experience over backhaul radio access networks, IEEE Trans. Cogn. Commun. Netw. (2022).

- X. Gao, Y. Liu, X. Liu, L. Song, Machine learning empowered resource allocation in IRS aided MISO-NOMA networks, IEEE Trans. Wirel. Commun. 21 (5) (2022) 3478–3492.

- Y. Shen, Y. Shi, J. Zhang, K.B. Letaief, Graph neural networks for scalable radio resource management: Architecture design and theoretical analysis, IEEE J. Sel. Areas Commun. 39 (1) (2021) 101–115.

- J.V. Saraiva, I.M. Braga, Jr., V.F. Monteiro, F.R.M. Lima, T.F. Maciel, W.C. Freitas, Jr., F.R.P. Cavalcanti, Deep reinforcement learning for QoS-constrained resource allocation in multiservice networks, J. Commun. Inf. Syst. 35 (1) (2020) 66–76.

- G. Sun, G.O. Boateng, D. Ayepah-Mensah, G. Liu, J. Wei, Autonomous resource slicing for virtualized vehicular networks with D2D communications based on deep reinforcement learning, IEEE Syst. J. 14 (4) (2020) 4694–4705.

- H. Zhang, H. Zhang, K. Long, G.K. Karagiannidis, Deep learning based radio resource management in NOMA networks: User association, subchannel and power allocation, IEEE Trans. Netw. Sci. Eng. 7 (4) (2020) 2406–2415.

- P. Luong, F. Gagnon, L.-N. Tran, F. Labeau, Deep reinforcement learning-based resource allocation in cooperative UAV-assisted wireless networks, IEEE Trans. Wirel. Commun. 20 (11) (2021) 7610–7625.

- S. Rezwan, W. Choi, Priority-based joint resource allocation with deep q-learning for heterogeneous NOMA systems, IEEE Access 9 (2021) 41468–41481.

- P.K. Sangdeh, H. Zeng, Deepmux: Deep-learning-based channel sounding and resource allocation for IEEE 802.11 ax, IEEE J. Sel. Areas Commun. 39 (8) (2021) 2333–2346.

- J. Jang, H.J. Yang, Deep reinforcement learning-based resource allocation and power control in small cells with limited information exchange, IEEE Trans. Veh. Technol. 69 (11) (2020) 13768–13783.

- Y. Chen, B. Ai, H. Zhang, Y. Niu, L. Song, Z. Han, H.V. Poor, Reconfigurable intelligent surface assisted device-to-device communications, IEEE Trans. Wirel. Commun. 20 (5) (2021) 2792–2804.

- J. Xu, B. Ai, Experience-driven power allocation using multi-agent deep rein- forcement learning for millimeter-wave high-speed railway systems, IEEE Trans. Intell. Transp. Syst. (2022).

- W.-C. Chien, S.-Y. Huang, C.-F. Lai, H.-C. Chao, M.S. Hossain, G. Muhammad, Multiple contents offloading mechanism in AI-enabled opportunistic networks, Comput. Commun. 155 (2020) 93–103.

- S. Wang, M. Chen, X. Liu, C. Yin, S. Cui, H.V. Poor, A machine learning approach for task and resource allocation in mobile-edge computing-based networks, IEEE Internet Things J. 8 (3) (2020) 1358–1372.

- Q. Fan, J. Bai, H. Zhang, Y. Yi, L. Liu, Delay-aware resource allocation in fog-assisted IoT networks through reinforcement learning, IEEE Internet Things

J. 9 (7) (2022) 5189–5199.

- M.S. Allahham, A.A. Abdellatif, N. Mhaisen, A. Mohamed, A. Erbad, M. Guizani, Multi-agent reinforcement learning for network selection and resource allocation in heterogeneous multi-RAT betworks, IEEE Trans. Cogn. Commun. Netw. (2022).

- Y.-C. Wu, T.Q. Dinh, Y. Fu, C. Lin, T.Q. Quek, A hybrid DQN and optimization approach for strategy and resource allocation in MEC networks, IEEE Trans. Wirel. Commun. 20 (7) (2021) 4282–4295.

- H. Yang, J. Zhao, Z. Xiong, K.-Y. Lam, S. Sun, L. Xiao, Privacy-preserving federated learning for UAV-enabled networks: Learning-based joint scheduling and resource management, IEEE J. Sel. Areas Commun. 39 (10) (2021) 3144–3159.

- S. Guo, K. Zhang, B. Gong, W. He, X. Qiu, A delay-sensitive resource allocation algorithm for container cluster in edge computing environment, Comput. Commun. 170 (2021) 144–150.

- A. Chowdhury, S.A. Raut, H.S. Narman, DA-DRLS: Drift adaptive deep rein- forcement learning based scheduling for IoT resource management, J. Netw. Comput. Appl. 138 (2019) 51–65.

- Z. Tong, X. Deng, J. Mei, B. Liu, K. Li, Response time and energy consumption co-offloading with SLRTA algorithm in cloud–edge collaborative computing, Future Gener. Comput. Syst. 129 (2022) 64–76.

- B. Sellami, A. Hakiri, S.B. Yahia, P. Berthou, Energy-aware task scheduling and offloading using deep reinforcement learning in SDN-enabled iot network, Comput. Netw. 210 (2022) 108957.

- B. Gu, M. Alazab, Z. Lin, X. Zhang, J. Huang, AI-enabled task offloading for improving quality of computational experience in ultra dense networks, ACM Trans. Internet Technol. (TOIT) 22 (3) (2022) 1–17.

- H.M. Hariz, S. Sheikhzadeh, N. Mokari, M.R. Javan, B. Abbasi-Arand, E.A. Jorswieck, AI-based radio resource management and trajectory design for PD- NOMA communication in IRS-UAV assisted networks, 2021, arXiv preprint arXiv:2111.03869.

- H. Zhou, M. Erol-Kantarci, V. Poor, Learning from peers: Deep transfer rein- forcement learning for joint radio and cache resource allocation in 5G RAN slicing, IEEE Trans. Cogn. Commun. Netw. (2022).

- C.T. Dinh, N.H. Tran, M.N. Nguyen, C.S. Hong, W. Bao, A.Y. Zomaya, V. Gramoli, Federated learning over wireless networks: Convergence analysis and resource allocation, IEEE/ACM Trans. Netw. 29 (1) (2020) 398–409.

- L.U. Khan, M. Alsenwi, I. Yaqoob, M. Imran, Z. Han, C.S. Hong, Resource optimized federated learning-enabled cognitive internet of things for smart industries, IEEE Access 8 (2020) 168854–168864.

- C. Deng, X. Fang, X. Wang, UAV-enabled mobile edge computing for AI appli- cations: Joint model decision, resource allocation and trajectory optimization, IEEE Internet Things J. (2022).

- X. Tang, X. Chen, L. Zeng, S. Yu, L. Chen, Joint multiuser DNN partitioning and computational resource allocation for collaborative edge intelligence, IEEE Internet Things J. 8 (12) (2021) 9511–9522.

- R. Guerra-Gomez, S. Ruiz-Boque, M. Garcia-Lozano, J.O. Bonafe, Machine learning adaptive computational capacity prediction for dynamic resource management in C-RAN, IEEE Access 8 (2020) 89130–89142.

- S. Zafar, S. Jangsher, A. Al-Dweik, Resource allocation using deep learning in mobile small cell networks, IEEE Trans. Green Commun. Netw. (2022).

- J. Zhao, M. Kong, Q. Li, X. Sun, Contract-based computing resource manage- ment via deep reinforcement learning in vehicular fog computing, IEEE Access 8 (2019) 3319–3329.

- Z. Cheng, N. Chen, B. Liu, Z. Gao, L. Huang, X. Du, M. Guizani, Joint user association and resource allocation in HetNets based on user mobility prediction, Comput. Netw. 177 (2020) 107312.

- A. Kaur, K. Kumar, A reinforcement learning based evolutionary multi-objective optimization algorithm for spectrum allocation in cognitive radio networks, Phys. Commun. 43 (2020) 101196.

- J. Fang, M. Zhang, Z. Ye, J. Shi, J. Wei, Smart collaborative optimizations strategy for mobile edge computing based on deep reinforcement learning, Comput. Electr. Eng. 96 (2021) 107539.

- J. Bae, J. Lee, S. Chong, Learning to schedule network resources throughput and delay optimally using Q+-learning, IEEE/ACM Trans. Netw. 29 (2) (2021) 750–763.

- A. Kaur, K. Kumar, Intelligent spectrum management based on reinforcement learning schemes in cooperative cognitive radio networks, Phys. Commun. 43 (2020) 101226.

- X. Wang, J.D. Thomas, R.J. Piechocki, S. Kapoor, R. Santos-Rodríguez, A. Parekh, Self-play learning strategies for resource assignment in Open-RAN networks, Comput. Netw. 206 (2022) 108682.

- J. Bae, J. Lee, S. Chong, Learning to schedule network resources throughput and delay optimally using Q+-learning, IEEE/ACM Trans. Netw. 29 (2) (2021) 750–763.

- S. Wang, Y.-C. Wu, M. Xia, R. Wang, H.V. Poor, Machine intelligence at the edge with learning centric power allocation, IEEE Trans. Wirel. Commun. 19

(11) (2020) 7293–7308.

- Y. Shen, Y. Qu, C. Dong, F. Zhou, Q. Wu, Joint training and resource allocation optimization for federated learning in UAV swarm, IEEE Internet Things J. (2023).

- J. Ren, G. Yu, G. Ding, Accelerating DNN training in wireless federated edge learning systems, IEEE J. Sel. Areas Commun. 39 (1) (2021) 219–232.

- W. Wen, Z. Chen, H.H. Yang, W. Xia, T.Q. Quek, Joint scheduling and resource allocation for hierarchical federated edge learning, IEEE Trans. Wirel. Commun. (2022).

- C. Feng, Z. Zhao, Y. Wang, T.Q. Quek, M. Peng, On the design of federated learning in the mobile edge computing systems, IEEE Trans. Commun. 69 (9) (2021) 5902–5916.

- G. Wang, F. Xu, H. Zhang, C. Zhao, Joint resource management for mobility supported federated learning in internet of vehicles, Future Gener. Comput. Syst. 129 (2022) 199–211.

- A. Gharehgoli, A. Nouruzi, N. Mokari, P. Azmi, M.R. Javan, E.A. Jorswieck, AI-based resource allocation in end-to-end network slicing under demand and CSI uncertainties, IEEE Trans. Netw. Serv. Manag. 20 (3) (2023) 3630–3651.

- W.U. Khan, T.N. Nguyen, F. Jameel, M.A. Jamshed, H. Pervaiz, M.A. Javed,

R. Jäntti, Learning-based resource allocation for backscatter-aided vehicular networks, IEEE Trans. Intell. Transp. Syst. (2022).

- A.A. Abdellatif, N. Mhaisen, A. Mohamed, A. Erbad, M. Guizani, Z. Dawy,

W. Nasreddine, Communication-efficient hierarchical federated learning for Iot heterogeneous systems with imbalanced data, Future Gener. Comput. Syst. 128 (2022) 406–419.

- I.-S. Comşa, A. Molnar, I. Tal, P. Bergamin, G.-M. Muntean, C.H. Muntean,

R. Trestian, A machine learning resource allocation solution to improve video quality in remote education, IEEE Trans. Broadcast. 67 (3) (2021) 664–684.

- B. Gu, X. Zhang, Z. Lin, M. Alazab, Deep multiagent reinforcement-learning- based resource allocation for internet of controllable things, IEEE Internet Things J. 8 (5) (2021) 3066–3074.

- Q. Li, H. Yao, T. Mai, C. Jiang, Y. Zhang, Reinforcement-learning-and belief-learning-based double auction mechanism for edge computing resource allocation, IEEE Internet Things J. 7 (7) (2020) 5976–5985.

- H.L. dos Santos, J.C. Marinello, C.M. Panazio, T. Abrão, Machine learning-aided pilot and power allocation in multi-cellular massive MIMO networks, Phys. Commun. 52 (2022) 101646.

- S. Tuli, S.S. Gill, M. Xu, P. Garraghan, R. Bahsoon, S. Dustdar, R. Sakellariou,

O. Rana, R. Buyya, G. Casale, et al., HUNTER: AI based holistic resource management for sustainable cloud computing, J. Syst. Softw. 184 (2022) 111124.

- S. Wang, M. Chen, X. Liu, C. Yin, S. Cui, H.V. Poor, A machine learning approach for task and resource allocation in mobile-edge computing-based networks, IEEE Internet Things J. 8 (3) (2021) 1358–1372.

- H. Lee, S.H. Lee, T.Q. Quek, Deep learning for distributed optimization: Applications to wireless resource management, IEEE J. Sel. Areas Commun. 37 (10) (2019) 2251–2266.

- J. Sheng, Z. Tang, C. Wu, B. Ai, Y. Wang, Game theory-based multi-objective optimization interference alignment algorithm for HSR 5G heterogeneous ultra-dense network, IEEE Trans. Veh. Technol. 69 (11) (2020) 13371–13382.

- P. Yu, M. Yang, A. Xiong, Y. Ding, W. Li, X. Qiu, L. Meng, M. Kadoch, M. Cheriet, Intelligent-driven green resource allocation for industrial internet of things in 5g heterogeneous networks, IEEE Trans. Ind. Inform. 18 (1) (2022) 520–530.

- D. Zhao, H. Qin, N. Xin, B. Song, Flexible resource management in high- throughput satellite communication systems: A two-stage machine learning framework, IEEE Trans. Commun. (2023).

- Z. Dai, Y. Zhang, W. Zhang, X. Luo, Z. He, A multi-agent collaborative environment learning method for UAV deployment and resource allocation, IEEE Trans. Signal Inf. Process. Netw. 8 (2022) 120–130.

- S. Luo, X. Chen, Q. Wu, Z. Zhou, S. Yu, HFEL: Joint edge association and resource allocation for cost-efficient hierarchical federated edge learning, IEEE Trans. Wirel. Commun. 19 (10) (2020) 6535–6548.

- T. Liu, T. Zhang, J. Loo, Y. Wang, Deep reinforcement learning-based resource allocation for UAV-enabled federated edge learning, J. Commun. Inf. Netw. 8

(1) (2023) 1–12.

- L.P. Qian, H. Zhang, Q. Wang, Y. Wu, B. Lin, Joint multi-domain resource allocation and trajectory optimization in UAV-assisted maritime IoT networks, IEEE Internet Things J. 10 (1) (2023) 539–552.

- S. Wang, M. Chen, C. Yin, W. Saad, C.S. Hong, S. Cui, H.V. Poor, Federated learning for task and resource allocation in wireless high-altitude balloon networks, IEEE Internet Things J. 8 (24) (2021) 17460–17475.

- C. Fang, H. Xu, Y. Yang, Z. Hu, S. Tu, K. Ota, Z. Yang, M. Dong, Z. Han, F.R.

Yu, et al., Deep-reinforcement-learning-based resource allocation for content distribution in fog radio access networks, IEEE Internet Things J. 9 (18) (2022) 16874–16883.

- X. Liu, H. Zhang, K. Long, A. Nallanathan, V.C. Leung, Energy efficient user association, resource allocation and caching deployment in fog radio access networks, IEEE Trans. Veh. Technol. 71 (2) (2022) 1846–1856.

- W. Fan, Z. Chen, Z. Hao, F. Wu, Y. Liu, Joint task offloading and resource allocation for quality-aware rdge-assisted machine learning task inference, IEEE Trans. Veh. Technol. (2023).

- W. Fan, Z. Chen, Z. Hao, Y. Su, F. Wu, B. Tang, Y. Liu, DNN deployment, task offloading, and resource allocation for joint task inference in IIoT, IEEE Trans. Ind. Inform. 19 (2) (2023) 1634–1646.

- Y. Cao, R. Wang, M. Chen, A. Barnawi, AI agent in software-defined net- work: Agent-based network service prediction and wireless resource scheduling optimization, IEEE Internet Things J. 7 (7) (2020) 5816–5826.

- Z. Ning, P. Dong, X. Wang, J.J. Rodrigues, F. Xia, Deep reinforcement learning for vehicular edge computing: An intelligent offloading system, ACM Trans. Intell. Syst. Technol. (TIST) 10 (6) (2019) 1–24.

- V.N. Vapnik, Statistical Learning Theory, Wiley, 1998.

- B. Everitt, Miscellaneous clustering methods, in: Cluster Analysis, Wiley, 2011.

- D. Svozil, V. Kvasnicka, J. Pospichal, Introduction to multi-layer feed-forward neural networks, Chemometr. Intell. Lab. Syst. 39 (1) (1997) 43–62.

- S. Ruder, An overview of gradient descent optimization algorithms, 2016, arXiv preprint arXiv:1609.04747.

- D.E. Rumelhart, G.E. Hinton, R.J. Williams, Learning representations by back-propagating errors, Nature 323 (6088) (1986) 533–536.

- R.S. Sutton, A.G. Barto, Reinforcement Learning: An Introduction, MIT Press, 2018.

- V. Mnih, K. Kavukcuoglu, D. Silver, A.A. Rusu, J. Veness, M.G. Bellemare, A. Graves, M. Riedmiller, A.K. Fidjeland, G. Ostrovski, et al., Human-level control through deep reinforcement learning, Nature 518 (7540) (2015) 529–533.

- T. Li, A.K. Sahu, A. Talwalkar, V. Smith, Federated learning: Challenges, methods, and future directions, IEEE Signal Process. Mag. 37 (3) (2020) 50–60.

- Y.-H. Xu, Q.-M. Sun, W. Zhou, G. Yu, Resource allocation for UAV-aided en- ergy harvesting-powered D2D communications: A reinforcement learning-based scheme, Ad Hoc Netw. 136 (2022) 102973.

- B. Dai, J. Niu, T. Ren, M. Atiquzzaman, Toward mobility-aware computation offloading and resource allocation in End–edge–cloud orchestrated computing, IEEE Internet Things J. 9 (19) (2022) 19450–19462.

- H. Wang, H. Zhang, X. Liu, K. Long, A. Nallanathan, Joint UAV placement optimization, resource allocation, and computation offloading for THz band: A DRL approach, IEEE Trans. Wirel. Commun. (2023).

- S. Mhatre, F. Adelantado, K. Ramantas, C. Verikoukis, Intelligent QoS-aware slice resource allocation with user association parameterization for beyond 5G O-RAN-based architecture using DRL, IEEE Trans. Veh. Technol. (2024).

- Z. Li, M. Chen, K. Wang, C. Pan, N. Huang, Y. Hu, Parallel deep reinforcement learning based online user association optimization in heterogeneous networks, in: 2020 IEEE International Conference on Communications Workshops, ICC Workshops, IEEE, 2020, pp. 1–6.

- M. Fortunato, M.G. Azar, B. Piot, J. Menick, M. Hessel, I. Osband, A. Graves, V. Mnih, R. Munos, D. Hassabis, O. Pietquin, C. Blundell, S. Legg, Noisy networks for exploration, in: International Conference on Learning Representations, 2018, [Online]. Available: https://openreview.net/forum?id=rywHCPkAW.

- N. Khan, A. Abdallah, A. Celik, A.M. Eltawil, S. Coleri, Explainable AI-aided feature selection and model reduction for DRL-based v2x resource allocation, IEEE Trans. Commun. (2025) 1–1.

- X. Li, L. Lu, W. Ni, A. Jamalipour, D. Zhang, H. Du, Federated multi- agent deep reinforcement learning for resource allocation of vehicle-to-vehicle communications, IEEE Trans. Veh. Technol. 71 (8) (2022) 8810–8824.

- S. Yin, F.R. Yu, Resource allocation and trajectory design in UAV-aided cellular networks based on multiagent reinforcement learning, IEEE Internet Things J. 9 (4) (2022) 2933–2943.

- S.D.A. Shah, M.A. Gregory, F. Bouhafs, F. Den Hartog, Artificial intelligence- defined wireless networking for computational offloading and resource allo- cation in edge computing networks, IEEE Open J. Commun. Soc. 5 (2024) 2039–2057.

- W. Jing, J. Wang, J. Ren, Z. Lu, H. Shao, X. Wen, Radio resource allocation optimization for delay-sensitive services based on graph neural networks and offline dataset, IEEE Trans. Veh. Technol. (2024).

- A. Benfaid, N. Adem, B. Khalfi, Adaptsky: a DRL based resource allocation framework in NOMA-UAV networks, in: 2021 IEEE Global Communications Conference, GLOBECOM, IEEE, 2021, pp. 01–07.

- C. Chaieb, F. Abdelkefi, W. Ajib, Deep reinforcement learning for resource allocation in multi-band and hybrid OMA-NOMA wireless networks, IEEE Trans. Commun. 71 (1) (2023) 187–198.

- A. Abbasi, B. Firouzi, P. Sendur, On the application of harris hawks optimization (HHO) algorithm to the design of microchannel heat sinks, Eng. Comput. 37 (2021) 1409–1428.

- J. Schulman, P. Moritz, S. Levine, M.I. Jordan, P. Abbeel, High-dimensional continuous control using generalized advantage estimation, in: Y. Bengio, Y. LeCun (Eds.), 4th International Conference on Learning Representations, ICLR 2016, San Juan, Puerto Rico, May 2-4, 2016, Conference Track Proceedings, 2016, [Online]. Available: http://arxiv.org/abs/1506.02438.

- F.R.M. Lima, T.F. Maciel, W.C. Freitas, F.R.P. Cavalcanti, Resource assignment for rate maximization with QoS guarantees in multiservice wireless systems, IEEE Trans. Veh. Technol. 61 (3) (2012) 1318–1332.

- D.A. Sousa, V.F. Monteiro, T.F. Maciel, F.R.M. Lima, F.R.P. Cavalcanti, Resource management for rate maximization with QoE provisioning in wireless networks,

J. Commun. Inf. Syst. 31 (1) (2016).

- T. Rashid, M. Samvelyan, C.S. De Witt, G. Farquhar, J. Foerster, S. Whiteson, Monotonic value function factorisation for deep multi-agent reinforcement learning, J. Mach. Learn. Res. 21 (178) (2020) 1–51.

- L. Huang, S. Bi, Y.-J.A. Zhang, Deep reinforcement learning for online compu- tation offloading in wireless powered mobile-edge computing networks, IEEE Trans. Mob. Comput. 19 (11) (2019) 2581–2593.

- S.D.A. Shah, M.A. Gregory, S. Li, R.D.R. Fontes, SDN enhanced multi-access edge computing (MEC) for E2E mobility and QoS management, IEEE Access 8 (2020) 77459–77469.

- R.K. Jain, D.-M.W. Chiu, W.R. Hawe, et al., A quantitative measure of fairness and discrimination, East. Res. Lab. Digit. Equip. Corp. Hudson, MA 21 (1) (1984).

- D.R. Karger, C. Stein, A new approach to the minimum cut problem, J. ACM 43 (4) (1996) 601–640.

- T.C. Bora, V.C. Mariani, L. dos Santos Coelho, Multi-objective optimization of the environmental-economic dispatch with reinforcement learning based on non-dominated sorting genetic algorithm, Appl. Therm. Eng. 146 (2019) 688–700.

- R.N. Calheiros, R. Ranjan, A. Beloglazov, C.A. De Rose, R. Buyya, CloudSim: a toolkit for modeling and simulation of cloud computing environments and evaluation of resource provisioning algorithms, Softw.: Pr. Exp. 41 (1) (2011) 23–50.

- S. Tuli, S.R. Poojara, S.N. Srirama, G. Casale, N.R. Jennings, COSCO: Container orchestration using co-simulation and gradient based optimization for fog computing environments, IEEE Trans. Parallel Distrib. Syst. 33 (1) (2021) 101–116.

- Y.-H. Xu, J.-H. Li, W. Zhou, C. Chen, Learning-empowered resource allocation for air slicing in UAV-assisted cellular V2X communications, IEEE Syst. J. 17

(1) (2022) 1008–1011.

- Y.-H. Xu, W. Zhou, Y.-G. Zhang, G. Yu, Stochastic game for resource manage- ment in cellular zero-touch deterministic industrial M2M networks, IEEE Wirel. Commun. Lett. 11 (12) (2022) 2635–2639.

- M. Drobyshevskiy, D. Turdakov, Random graph modeling: A survey of the concepts, ACM Comput. Surv. 52 (6) (2019) 1–36.

- X. Zhang, X. Zhang, W. Yang, Joint offloading and resource allocation using deep reinforcement learning in mobile edge computing, IEEE Trans. Netw. Sci. Eng. 9 (5) (2022) 3454–3466.

- Q. Wang, S. Chen, C. Yang, Z. Li, Y. Wang, T. Chen, Minimizing energy and latency in leos-assisted open ran architecture towards AI of things, IEEE Internet Things J. (2025).

- J. Lu, J. Yang, S. Li, Y. Li, W. Jiang, J. Dai, J. Hu, A2C-DRL: Dynamic scheduling for stochastic edge-cloud environments using A2C and deep reinforcement learning, IEEE Internet Things J. (2024).

- N. Zhao, Y.-C. Liang, D. Niyato, Y. Pei, M. Wu, Y. Jiang, Deep reinforcement learning for user association and resource allocation in heterogeneous cellular networks, IEEE Trans. Wirel. Commun. 18 (11) (2019) 5141–5152.

- F. Fu, Y. Kang, Z. Zhang, F.R. Yu, T. Wu, Soft actor–critic DRL for live transcod- ing and streaming in vehicular fog-computing-enabled IoV, IEEE Internet Things

J. 8 (3) (2020) 1308–1321.

- J. Schulman, F. Wolski, P. Dhariwal, A. Radford, O. Klimov, Proximal policy optimization algorithms, 2017, arXiv preprint arXiv:1707.06347.

- E.W. Dijkstra, A note on two problems in connexion with graphs, in: Edsger Wybe Dijkstra: His Life, Work, and Legacy, 2022, pp. 287–290.

- R.S. Thomä, D. Hampicke, A. Richter, G. Sommerkorn, U. Trautwein, MIMO vector channel sounder measurement for smart antenna system evaluation, Eur. Trans. Telecommun. 12 (5) (2001) 427–438.

- M. ApS, Mosek optimization toolbox for matlab, User’s Guid. Ref. Man. Version 4 (1) (2019) 116.

- X. Ma, Q. Gao, J. Wang, V. Marojevic, J.H. Reed, Dynamic sounding for multi- user MIMO in wireless LANs, IEEE Trans. Consum. Electron. 63 (2) (2017) 135–144.

- L. Yu, Z. Li, Y. Zhong, Z. Ji, J. Liu, When QoE meets learning: A distributed traffic-processing framework for elastic resource provisioning in HetNets, Comput. Netw. 167 (2020) 106904.

- X. Zhong, Y. Guo, N. Li, Y. Chen, Joint optimization of relay deployment, chan- nel allocation, and relay assignment for UAVs-aided D2D networks, IEEE/ACM Trans. Netw. 28 (2) (2020) 804–817.

- M.A. Jamshed, F. Heliot, T.W. Brown, Unsupervised learning based emission- aware uplink resource allocation scheme for non-orthogonal multiple access systems, IEEE Trans. Veh. Technol. 70 (8) (2022) 7681–7691.

- M. Shao, J. Liu, Q. Yang, G. Simon, A learning based framework for MEC server planning with uncertain BSs demands, IEEE Access 8 (2020) 198832–198844.

- H. Tang, H. Wu, Y. Zhao, R. Li, Joint computation offloading and resource allocation under task-overflowed situations in mobile-edge computing, IEEE Trans. Netw. Serv. Manag. 19 (2) (2022) 1539–1553.

- B. Huang, J. Feng, X. Jin, Sensing-communication-computing integrated re- source allocation for AI-empowered trustworthy IoT, IEEE Trans. Consum. Electron. (2025).

- J. MacQueen, Some methods for classification and analysis of multivariate observations, in: Proceedings of the Fifth Berkeley Symposium on Mathematical Statistics and Probability, Volume 1: Statistics, vol. 5, University of California Press, 1967, pp. 281–298.

- M. Merluzzi, P. Di Lorenzo, S. Barbarossa, Wireless edge machine learning: Resource allocation and trade-offs, IEEE Access 9 (2021) 45377–45398.

- Z. Lin, S. Bi, Y.-J.A. Zhang, Optimizing AI service placement and resource allocation in mobile edge intelligence systems, IEEE Trans. Wirel. Commun. 20 (11) (2021) 7257–7271.

- J. Feng, W. Zhang, Q. Pei, J. Wu, X. Lin, Heterogeneous computation and resource allocation for wireless powered federated edge learning systems, IEEE Trans. Commun. 70 (5) (2022) 3220–3233.

- S. Diamond, S. Boyd, CVXPY: A python-embedded modeling language for convex optimization, J. Mach. Learn. Res. 17 (83) (2016) 1–5.

- D. Ismailova, W.-S. Lu, Penalty convex-concave procedure for source localiza- tion problem, in: 2016 IEEE Canadian Conference on Electrical and Computer Engineering, CCECE, IEEE, 2016, pp. 1–4.